# MACOS

# Modeling and Analysis for Controlled Optical Systems

#### Contact

For information on MACOS contact:

David Redding dcr@huey.jpl.nasa.gov

Jet Propulsion Laboratory 306-438 4800 Oak GroveDrive

Pasadena CA 91109

#### Acknowledgment

The MACOS program was developed at the Jet Propulsion Laboratory, California Institute of Technology.

MACOS software and documentation are copyright 1992-2026 by the California Institute of Technology, Jet Propulsion Laboratory.

U.S. Government sponsorship of parts of MACOS under NASA Contract NAS7-918 is acknowledged.

MACOS graphical functions are supplied by PGPLOT, an astronomical plotting package. PGPLOT is copyrighted by the California Institute of Technology. PGPLOT is available free of charge from CalTech by anonymous ftp to deimos.caltech.edu, or contact:

Tim Pearson tjp@deimos.caltech.edu Astronomy Department, 105-24 California Institute of Technology Pasadena, CA 91125.

#### Installation

A separate installation manual is provided with the distribution tape.

## Table of Contents

SECTION 1	Introduction	11

  - Modeling and Analysis for Controlled Optical Systems  11

  - Overview of this Manual  12

  - New Features of Version 2.8  12

SECTION 2	Technical Overview	13

  - Geometry and Coordinate Systems  13

    - Global Coordinates  13

    - Local coordinates  15

  - Geometric Optics: Tracing Rays  16

  - Physical Optics: Diffraction  17

    - Single Plane Diffraction  21

    - Conventional Multi-plane Diffraction 22

    - Multi-plane Diffraction in MACOS  22

  - Polarized Light  23

SECTION 3	User Interface	25

  - Starting and Stopping MACOS  25

  - Help  26

  - Entering Commands  28

  - Graphics Display  29

  - Off-Line Graphics Display  30

  - PostScript and GIF Graphics Output  31

  - MACOS Files  31

    - .In-File  31

    - .Out-File  32

    - Glass Tables  32

    - MACOS Data Directory  32

    - Data Export  32

  - Using Macros  33

    - Journal Files  33

    - Executing Macros  33

  - Memory Requirements  34

SECTION 4	Describing Optical Systems	35

  - Specifying the Light Source  35

    - Source Position and Orientation  36

    - Refractive Index and Extinction Coefficient at the Source  36

    - Wavelength  37

    - Flux  37

    - Aperture  37

    - Ray Grids  38

  - Example Light Source  39

  - Specifying Optics  41

  - Element Types  43

    - Reflectors  43

    - Refractors  44

    - Glass Tables  46

    - Lens Arrays  47

    - Reflective Gratings  49

    - Transmissive Gratings  51

    - Reflective Holographic Optical Elements (HOEs)  51

    - Reference Surfaces or Dummy Elements  53

    - Focal Planes  55

    - Return Surfaces  56

    - Hex and Pie Segmented Mirrors  57

    - Flower Segmented Mirrors  61

    - Reflective Non-Sequential Surfaces  62

    - Refractive Non-Sequential Surfaces  64

  - Element Surface Types  64

    - Flat Surfaces  64

    - Conic Surfaces  64

    - Generalized (10th-Order) Aspheric Surfaces  65

    - Zernike Surfaces  66

    - Monomial Surfaces  70

    - Toric Surfaces  70

    - Anamorphic Surfaces  70

    - User-Defined Surfaces: Deformable Mirrors  71

    - Interpolated Surfaces Using Regularly Gridded Data  72

    - Interpolated Surfaces Using X-Y-Z Data  72

  - Obscurations and Apertures  73

  - Specifying Coatings  78

  - Example Telescope  78

  - Editing .In-Files  83

  - Commands  84

    - OLD and LOAD  84

    - MODIFY  85

    - SAVE  85

    - SUMMARIZE  85

    - STATUS 86

    - RESET  86

    - SHOW  86

  - Summary of Prescription Variables  86

SECTION 5	Ray-Trace Analysis	91

  - Background  91

  - Ray-Trace Commands  92

    - DRAW  92

    - MAP  93

    - Undefined Rays  94

    - RAY  95

    - OPD  97

    - SPOT  99

    - OBSCURATION  99

  - Beam Set-Up Commands  101

    - STOP  101

    - CENTER  102

    - COORD  102

    - FFP  103

    - PFP  104

    - FEX  105

    - PERTURB  107

    - PREAD  108

    - ATMOS  108

    - POLARIZATION  109

    - NOPOLARIZATION  109

    - SINT  109

SECTION 6	Diffraction Analysis	111

  - Single- versus Multi-plane Diffraction  111

  - Setting up Diffraction Propagations  113

    - Choosing Propagator Type  114

    - Far-Field  115

    - Near-Field  116

  - Commands  118

    - ORS  118

    - SRS  118

    - REGRID  119

    - SCALAR  119

    - VECTOR  119

    - Propagation commands  119

    - BEAM  119

| SECTION 7 |  |  | Beam Propagation and Image Simulation | 121 |
| --- | --- | --- | --- | --- |
|  | 7.1 |  | Background 121 |  |
|  | 7.2 |  | Beam Propagation Commands 122 7.2.1 INTENSITY 122 |  |

    - PIXILATE  124

    - STRETCH  125

    - WINDOW  126

    - COMPOSE  126

    - ADD  127

    - Propagating a Perturbed Beam  127

    - GBLUR  130

    - DAD  131

    - GAIN  131

    - AMPLITUDE and PHASE  131

    - REAL and IMAGINARY  133

  - Plot Type Commands  134

| 7.3.1 | GRAY | 134 |
| --- | --- | --- |
| 7.3.2 | WIRE | 134 |
| 7.3.3 | SLICE | 134 |

    - CONTOUR  135

    - COLUMN and ROW  135

    - TEXT  136

    - BINARY  137

    - MATLAB  137

    - FITS  137

| SECTION 8 |  |  | Differential Ray-Tracing and Linear Optical Models | 139 |
| --- | --- | --- | --- | --- |
|  | 8.1 |  | Background 139 |  |
|  |  |  | - Linear Ray States 140 - Linear Models 141 |  |
|  | 8.2 |  | Commands for Building Models 143 |  |
|  |  |  | - BUILD 143 - DMBUILD 144 |  |
|  | 8.3 |  | Commands for Changing Models 144 |  |

    - LPERTURB  144

    - LPREAD  144

    - LRESET  145

  - Commands for Exercisizing Models  145

    - PARTIALS  145

    - LOPD  147

    - LSPOT  147

    - LINTENSITY  147

    - LPIXILATE  147

  - Commands for Exporting Models  148

|  |  |  | Contents |  |
| --- | --- | --- | --- | --- |
| SECTION 9 |  |  | Subroutine MACOS and Nonlinear Optical Models | 149 |
|  | 9.1 |  | Background 149 |  |
|  | 9.2 |  | Call Line Variables 150 |  |

    - Dummy Command Variables 150

    - Dummy Output Variables  150

    - Common Blocks and Include Files  151

  - Example  154

  - Commands  154

    - File and Data Manipulation  155

    - Ray-Trace Analysis  156

    - Diffraction Analysis  158

    - Wavefront and Image Plots  159

    - Plot Types  161

    - Linear Models  162

| REFERENCES |  |  | 165 |
| --- | --- | --- | --- |
| APPENDIX A |  |  | Examples and Demonstrations 167 |
|  | A.1 |  | Cassegrain Telescope Examples 167 |
|  |  |  | - Cassegrain.jou 167 - Cassegrain.in 167 - CassWithExitPupil.jou 169 - CassWithExitPupil.in 169 |
|  | A.2 |  | Diffractive Elements Examples 172 |
|  |  |  | - GratingExample.jou 172 - GratingExample.in 172 - HOEExample.jou 173 - HOEExample.in 174 |
|  | A.3 |  | Non-Sequential Surface Examples 175 |
|  |  |  | - CornerCube.jou 175 - CornerCube.in 175 - Luneberg.jou 177 - Luneberg.in 177 |
|  | A.4 |  | Adaptive Optics Example 182 |
|  |  |  | - AOExample.jou 182 - AOExample.in 183 - AOmirror.test 190 |
|  | A.5 |  | Segmented Telescope Example 192 |
|  |  |  | - SegDemo.jou 192 - SegDemo.in 193 |
|  | A.6 |  | Near-Field Propagation Example 197 |
|  |  |  | - coroExample.jou 197 - coroExample.in 198 |
|  | A.7 |  | Image Simulation Example 203 |
|  |  |  | - ImageDemo.jou 203 - ImageDemo.in 203 |
|  | A.8 |  | S-MACOS Example Files 207 |
|  |  |  | - smacosvars.cmn 207 - smacosExample.f 208 |

## SECTION 1: Introduction

This section introduces the features of MACOS and gives an overview of the manual. New features introduced with this version of the code are summarized.

### Modeling and Analysis for Controlled Optical Systems

The Modeling and Analysis for Controlled Optical Systems software provides tools for modeling and analyzing optical systems. It consists of a stand-alone application pro-gram called MACOS and a Fortran subroutine package called SMACOS. SMACOS provides the full functionality of the MACOS application in a form that can be directly integrated with other codes.

MACOS is an optical analysis and model generation program, as opposed to an optical design program. It is focused on generating and exercising fast, accurate, and detailed models combining ray-trace, differential ray-trace, and diffraction techniques. It does not include extensive internal optimization functions or other features to support the optical designer.

MACOS does provide unique modeling capability for system-level design and analysis tasks, however. Capabilities include modeling optics on dynamic structures, deformable optics, and controlled optics. These models are easily integrated with standard struc-tures, controls, and user written analysis tools to create an end-to-end system model.

Using MACOS linear optical models (or SMACOS subroutines), integrated models can be exercised in end-to-end dynamic simulations, Monte Carlo analyses, covariance analyses, or optimization studies to determine system-level performance.

MACOS (and SMACOS) provide computationally efficient general ray-trace capabili-ties. These support standard analyses of optical performance such as ray-traces, spot-diagrams, and wavefront (OPD) plots. They also support MACOS diffraction calcula-tions.

MACOS has a differential ray-trace capability which generates linear matrix models of optical systems. These models are accurate for systems that undergo only small pertur-bations and are essential for tasks such as covariance analysis and control design. Mod-els can be exported to linear analysis programs such as Matlab where they can be exercised alone or in combination with structural, thermal, and control models.

For diffraction calculations, MACOS uses Fresnel propagators driven by exact phases computed by its ray-trace engine. Simulated images (point-spread functions) can be generated using single- or multi-plane diffraction. MACOS supports simulation of mul-tiple source and wavelength images. Digitization (pixilation) can be set to model detec-tor resolution limits.

### Overview of this Manual

This manual provides two services to the user:

    - A technical orientation outlining MACOS ray-trace, differential ray-trace, and dif-fraction modeling capabilities.

    - Detailed description of MACOS commands including reference material and numer-ous examples.

Section 2 of this manual briefly summarizes the technical areas covered by MACOS: geometric optics and physical optics. The purpose is to establish basic definitions and to introduce certain MACOS features. More detailed technical discussion is provided at the beginning of each subsequent section and in the references. Section 3 discusses the basic procedures of starting MACOS, getting help, entering commands, and using files. Sections 4 through 9 proceed in the sequence that a typical MACOS session might:

    - Entering the optical prescription from a drawing of the system (Section 4)

    - Ray-tracing the system (Section 5)

    - Setting up the model for diffraction analysis (Section 6)

    - Simulating images (Section 7)

    - Building and exporting linear models (Section 8)

    - Generating and using SMACOS models (Section 9)

Several example problems are worked in the manual to illustrate specific features of the code. Appendix A contains the prescriptions and journal files for the examples. Appen-dix B contains selected technical papers.

### New Features of Version 4.00

MACOS 4.00 (April 2026) is a feature release built on the Fortran 90 / SMACOS infrastructure introduced in v3.2.  It adds a new general-purpose composite surface (FreeForm), modernizes the graphics layer, expands the SMACOS / GMI MATLAB interface, and adds new utility commands plus a separate segmented-mirror prescription generator.

A summary of changes since MACOS 3.2:

FreeForm composite surface (SrfType = 14). Single surface type combining a conic base with a Cartesian monomial sag (Mon), a free-form Zernike sag (FF), and an interpolated grid sag, all on independent coordinate frames.  See §4.5.11.

Per-element Zernike inputs.  Sparse "modes + coefs" form MonZernCoef / MonZernModes and FFZernCoef / FFZernModes is accepted directly in .in files; conversion to monomial form happens internally at trace time.  See §4.5.4.

Per-ray status tracking.  Every ray records why it failed (Miss / Obscured / Bracket / MaxIter) and at which element.  The end-of-trace WARN summary breaks down failures by category and throttles bracket / iter messages after the first 20.

Graphics modernization.  Built-in giza graphics backend replacing legacy PGPLOT (still selectable at build time).  New IMGMODE command toggles between astronomical (negative, large->dark) and conventional (positive, large->light) display polarity.  Colorbar wedge labels reflect the active STRETCH (LIN / LOG10 / SQRT).  See §7.x.

SEGRAYTRACE command.  Trace a single ray through the geometric center of a chosen segment element; print ray state at every downstream surface.  Useful for verifying segmented-mirror prescriptions.  See §5.2.

SegMirMaker external tool.  Standalone utility, in MACOS_resources/segmirmaker/, that generates segmented-mirror .presc and Hx.m files.  Replaces SMPGe (1992) and adds support for FreeForm parents.  See §4.4.11.

GMI MATLAB interface upgrades.  New 14th MEX argument pmonzern for FreeForm MonZernCoef perturbations (parallel to pzern).  New param.monzernSrf element list.  Nominal-state save/restore extended to cover FreeForm coordinate frames so per-element rigid-body perturbations no longer leak across GMI calls.  See §9.

Graceful Ctrl-C.  SIGINT installs a clean-exit handler at start-up; pressing Ctrl-C at the MACOS> prompt prints 'MACOS: interrupted' and exits cleanly, replacing the Intel-runtime 'forrtl: severe (69)' traceback.

HELP command rebuilt.  All ~140 dispatched commands are now listed (vs. ~75 in v3.2), grouped into 15 categories with one-line effect descriptions and prerequisite tags ([Rx] / [BLD] / [DIFF]).  See §3.2 and Appendix C.

### 1.3 (historical) Changes since MACOS 2.8

MACOS 3.2 Beta Version (3.2b) is a Fortran 90 version of MACOS, developed using MACOS 2.86 as the baseline. From a user’s point of view, the most important improve-ment in MACOS 3.2b is its ability to support runtime specification of the MACOS model size in both interactive mode and in subroutine MACOS (SMACOS). The usage of this feature is described in Section 3.3 for the interactive mode and in Section 9.3 for SMACOS. Taking advantage of Fortran 90 compiler’s strong type-checking, a number of type mismatches between calling and called subroutines in MACOS have been fixed. MACOS 3.2b has been tested on several optical system simulations for space telescope wavefront sensing and control.

MACOS 3.2b retains all improvements that MACOS 2.8 made over earlier versions of MACOS. For users familiar with MACOS before version 2.8, version 2.8 brings some improvements and some changes. New features include additional surface types, addi-tional element types, a new segmentation option, and glass tables. Full backward com-patibility with existing MACOS prescriptions and journal files has been preserved.

MACOS 2.8 made some changes in its interface to SMACOS functions. These include a changed SMACOS argument list; also some important arguments and internal arrays, such as OPMmat, are now REAL*8 rather than REAL*4 variables. In additionto these changes, a user program calling SMACOS in MACOS 3.2b needs to include relevant Fortran 90 modules instead of Fortran 77 common block files as a way of communicat-ing with SMACOS. Programs calling SMACOS should be examined to ensure compati-bility with Version 3.2.

A summary of changes since MACOS 2.8:

    - Toroid surface type was added

### New Features of Version 3.2

- Interpolated data surfaces using general X-Y-Z data are once again supported

- Interpolated data surfaces using square X-Y grids have been added

- Influence function-based surfaces are provided as a “user-defined” surface

- Transmissive and reflective straight-ruled gratings were added

- “Flower” segmentation was added

- A gaussian blur function was added for simulating image jitter and other noise effects

- SMACOS call line arguments were changed

- OPMmat and RaySpot variables were changed to double precision

## SECTION 2: Technical Overview

This section provides a technical orientation to Modeling and Analysis for Controlled Optical Systems, providing brief reviews of geometric and physical optics. Though in the form of a tutorial, it is intended mainly as a summary of definitions and approach.

### Geometry and Coordinate Systems

Optical systems are composed of elements — mirrors, lenses, detectors, gratings, holo-graphic optical elements, and others — that have optical properties. These optical ele-ments are distributed in 3-space, which is to say, they are placed at different locations. Where they are located is of critical importance. Telescopes, cameras, optical systems in general depend on precise element positioning to achieve good optical performance.

Equally important is element orientation—where each element is pointed. Angles of reflection or refraction of a light beam depend on the orientation in 3-space of each sur-face in the system.

#### Global Coordinates

MACOS requires the user to specify the locations and orientations of elements using vectors in a single global coordinate system, as opposed to the usual approach of speci-fying each optical element relative to the preceding element. A single user-defined coor-dinate system is often more convenient when preparing models to be integrated with structural models: the structural coordinates can be used for the optical model as well. It facilitates checking results, especially for decentered and tilted systems. A global coor-dinate system also eliminates unnecessary calculations from the ray-trace. All MACOS calculations are actually done in the user-specified global coordinate system (Ref. Red-ding and Breckenridege). Specifying the system in those coordinates eliminates coordi-nate transformations as well as ray calculations at dummy surfaces. The result is significantly faster execution times.

To illustrate the choice of a global coordinate system, consider the simple 2-mirror Cas-segrain telescope drawn in Figure 1.

In this optical system there are three optical elements: the primary mirror (PM); the sec-ondary mirror (SM); and a focal plane (FP). Two possible global coordinate system are

      - The structural coordinate frame (Frame A), which would be shared by structural models, simplifying integration of optics and structures models.

      - A frame centered at an optical element, such as the primary mirror, and aligned with the optical axis of the system (Frame B). This frame is more natural for optical lay-out. It can also be convenient when comparing results with optical design programs.

The location of each element is defined in MACOS using a 3-vector called VptElt (for “vertex point”). For the primary mirror of our example, VptElt is drawn from the ori-gin of the global coordinate system to the vertex of the element, which is shown as the small circle. VptElt is indexed by 1=x, 2=y, 3=z coordinate values. For planes and

Frame B

Secondary Mirror

Detector

Primary Mirror

Frame A

1.0

5.0

z

x

2.5

FIGURE 1 Coordinate systems for the Cassegrain telescope example.

spheres, which have no unique vertex, VptElt can be any point on the surface. For the two example frames, VptElt takes values as listed in Tables 1 and 2

TABLE 1. Primary mirror location specification

| Location | Frame A | Frame B |
| --- | --- | --- |
| VptElt(1) | 2.5 | 0.0 |
| VptElt(2) | 0.0 | 0.0 |
| VptElt(3) | 5.0 | 0.0 |

TABLE 2. Secondary mirror location specification

| Location | Frame A | Frame B |
| --- | --- | --- |
| VptElt(1) | 2.5 | 0.0 |
| VptElt(2) | 0.0 | 0.0 |
| VptElt(3) | 1.0 | -4.0 |

Comparing the intervertex spacings of the two mirrors (the difference of the PM and SM VptElt), you can see that they are the same in either frame. It does not matter what frame is used, provided the relative location of each element is correct.

Ellipsoidal mirror	Paraboloidal mirror

FIGURE 2  Principal axis directions

The orientation of each element is specified by a unit-vector (a 3-vector of length 1) called PsiElt. For conicoid elements PsiElt is the principal axis direction of the ele-ment surface: the direction of a line drawn from the vertex of the element towards the nearest geometric focus. Figure 2 shows the principal axis for elliptical and paraboloi-

### Geometry and Coordinate Systems

dal mirrors. The principal axes for spheres or flats, which have no unique vertex, are defined from VptElt.

For the primary and secondary mirrors in our example, the principal axes are pointed in the negative z direction (Table 3). Since Frame A and Frame B have the same orienta-tion, (they differ only in translation) the values of PsiElt are the same in both coordi-nate frames.

TABLE 3. Mirror orientations

| Orientation | Primary Mirror | Secondary Mirror |
| --- | --- | --- |
| PsiElt(1) | 0.0 | 0.0 |
| PsiElt(2) | 0.0 | 0.0 |
| PsiElt(3) | -1.0 | -1.0 |

Detailed discussion of how elements are specified is provided in Section 4.

#### Local coordinates

MACOS allows the user to explicitly specify three types of local coordinates: element coordinates, output coordinates, and surface coordinates. Also important, but not explicitly set at each surface, are beam coordinates.

Element coordinates allow the user to simulate optical actuators, such as steering mir-rors or translation stages, by defining coordinates local to the controlled element and aligned with the actuator axes. Actions of the steering mirror are rotations about, or translations along, that axis, and are easily implemented via the MACOS PERturb command (see Section 5.3.7). These element coordinates can also be imbedded into a linear model of the system using the MACOS BUIld and EXPort commands (see Sec-tions 8.2.1 and 3.8.5). This enables creation of optical models directly in terms of actua-tor actions.

Output coordinates are used when calculating certain graphical outputs, such as spot diagrams. They help define sensing axes for simulating detectors (e.g., rows and col-umns of CCDs). When embedded in a linear model, they define the optical outputs of that model by selecting and transforming the optical states into combinations specified by the user.

Element coordinates are centered at a rotation point, specified as a 3-vector RptElt, which is the (0,0,0) point of the element coordinate system expressed in global coordi-nates. All rotations imparted to the element through the PERturb function are about RptElt.

The element coordinate axes are 6-vectors which describe rotational and translational perturbation to the element. Each 6-vector consists of two 3-vectors designating the rotation and translation, respectively. Usually an element will be actuated in rotation or translation only, but the option exists to use coupled rotation/translation coordinates.

Surface coordinates are required to define propertires of certain surface types, such as Zernike surfaces, and are described in Section 4.

Beam coordinates define the orientation of the ray and diffraction grids at particular ele-ments. They are initially defined at the source, and propagated to each subsequent optic by the ray tracing process. In general they are not the same at each element (due to tilt-

ing, rotating and inverting properties of the optical system). Beam coordinates can be computed at any surface using the MACOS COORD command.

### Geometric Optics: Tracing Rays

Geometric optics capture the particle nature of light. In geometric optics, light beams are represented as bundles of rays, which are the trajectories of individual photons. Rays are generally composed of numerous straight-line segments, with direction changes at the reflective or refractive surfaces of optical elements (curved segments occur in regions of continuously changing refractive index). Ray-tracing is the process of propa-gating a bundle of rays through an entire optical system.

Ray-tracing is an extremely valuable tool for determining the performance of optical systems. For example, characteristics of a well-designed imaging system include an extremely small spot diagram at the detector focal plane. Spot diagrams are plots of the pierce points of a ray bundle on a surface, and a small spot indicates a tight focus. An example is provided by the Cassegrain telescope of Figure 3. Two spot diagrams are shown in Figure 4. The unaberrated telescope indeed has a focus at a single spot (diam-eter is approximately 10-10 in arbitrary units). Misaligning the primary mirror causes the spot to spread as shown in Figure 4 to approximately 2x10-8.

Spot diagram at entrance pupil

Cross-section of ray-trace

FIGURE 3  Ray trace of a Cassegrain telescope

As another example, interferometer systems can be evaluated by calculating the flatness of two interfering wavefronts at a pupil of a system. The wavefront of an extended beam is accurately represented by an Optical Path Difference (OPD) map at a particular sur-face, such as the beam-splitter where the interference is to take place.

The predictions of geometric optics are exact only in cases where the wavelength of the light is infinitely small. Of course, no such system exists. Geometric optics do provide extremely good predictions in a wide range of cases, where wavelengths are small com-pared to the dimensions of the optical system. Even in systems with long wavelengths

Physical Optics: Diffraction

and large diffraction effects, geometric optics are useful for determining the best design conditions.

One thing that ray-tracing cannot do is predict the intensity pattern of a focused spot.

Focal plane spot diagram (unaberrated system)

Focal plane spot diagram

(primary mirror tilted 1e-4 radians)

FIGURE 4  Spot diagrams for Cassegrain.in.

Near-focus is a region where diffraction effects dominate, as a perfectly-focused spot is itself infinitely small. For this case as well as others, physical optics approaches must be employed. Physical optics capture the wave nature of light and can accurately account for diffraction around obstructions, or in tightly focused beams, or at diffraction grat-ings. The MACOS approach to physical optics is discussed in the Section 6.

The exit pupil is a conjugate (an image) of the aperture stop of the system. For the Cas-segrain telescope example, the entrance pupil is located at the primary mirror. A point source at the primary produces a virtual image behind the secondary mirror (see Figure 5). The exit pupil is a spherical “reference” surface at the virtual image point, with cen-ter of curvature on the focal plane. This surface coincides with the nominal spherical wavefront converging on the geometric focus.The OPD distribution at the exit pupil determines the image quality. Perfect optical systems have zero OPD; the wavefront leaving the optical system is spherical. In an aberrated optical system the OPD is non-zero as shown in Figure 6.

### Physical Optics: Diffraction

Physical optics capture the wave nature of light. In physical optics, Huygen’s Principle states that each point within a beam of light radiates a spherical wave of light. A wave-front is the envelope of all the waves emitted at a particular instant of time. Waves of

Exit pupil at virtual image

Point source on stop

Secondary Mirror

Primary Mirror

Focal Plane

FIGURE 5  Exit pupil for Cassegrain.in

Unaberrated (defocused)	Aberrated OPD (coma)

FIGURE 6  OPD at the exit pupil

light propagate through an optical train by bouncing off of mirrors or bending through lenses, much as predicted by geometric optics. Unlike geometric optics, however, phys-ical optics predicts that light can leak back into areas of shadow, or areas outside of the edge of the geometric beam. This process is diffraction and is important when model-ling images, the propagation of beams over long distances, and other phenomena.

The physics underlying diffraction and the free-space propagation of light are captured in Maxwell’s equations. Physical optics theories have simplified these equations, enabling practical solutions to optical problems. Scalar diffraction theory assumes time invariance and ignores polarization effects (Ref. Goodman). Fresnel propagation theory further assumes that the beam is paraxial, or a small beam divergence/convergence. This allows the expression of the propagation of light from one planar surface to another pla-nar surface in terms of the paraxial wave equation.

Geometric optics is the solution of Maxwell’s equations in the limit of an infinitesimally small wavelength. This is a good approximation for many practical optics problems.

Physical Optics: Diffraction

Diffraction effects start to dominate the propagation of a beam when the beam size approaches its wavelength (i.e., near focus), or when the dimensions of a surface approaches the beam wavelength (i.e., diffraction grating grooves), or when the beam is projected over distances exceeding its Rayleigh length (Ref. Goodman, Siegman).

The paraxial wave equation can be solved using two-dimensional Fourier transforma-tions for free-space propagation between planes. MACOS implements these using com-putationally efficient fast Fourier transforms (FFTs). Care must be taken when using Fourier optics because of the approximations inherent in the Fresnel assumptions. Using FFTs brings additional concerns of sampling density, aliasing and wrap-around. Violat-ing the Fresnel conditions, or sampling incorrectly, can affect the accuracy of computed results. MACOS provides several features to help you set problems up correctly.

In a physical optics computer model, the light beam is evaluated on reference surfaces. These are “dummy” surfaces located in the beam and aligned to the beam. They have no optical effect and are essentially transparent screens on which the beam is sampled.

The light on the reference surface is represented as a complex matrix, each element of which is the complex amplitude of the scalar field at a particular location. These loca-tions are distributed as a regular rectangular grid of points across each reference surface (Fig. 7). Light is propagated between these reference surfaces using algorithms based (in the case of Fresnel diffraction) on the solutions to the paraxial wave equation.

The complex amplitudes can be interpreted as having magnitude and phase. Viewed geometrically, the phases describe the OPD (see Fig. 8). If a reference surface is not aligned with a particular wavefront, the phases will vary “rapidly” across the reference surface grid. On the other hand, if the reference surface is nearly aligned with the wave-front, the phases vary only slowly. The magnitude describes the amplitude of the elec-tromagnetic field across the exit pupil.

First reference surface

Second reference surface

FIGURE 7  Sampling grid for a collimated beam

Sampling refers to the density of samples per phase crossing. The FFT algorithm works quite well when there are many (at least 4 to 8) sampling points per phase crossing.

When there are fewer, the ability of the FFT to capture higher spatial frequency effects is diminished. This difficulty can become acute when trying to sample diverging or con-verging waves using planar reference surfaces. Fortunately, this is not necessary. A coordinate transformation (Ref. Sziklas and Siegman) allows the paraxial wave equation to be applied to converging or diverging beams, by removing the spherical component of phase from the light. This is equivalent to sampling the light on a spherical reference surface.

Sampling on a sphere takes out the need to sample the spherical part of the wavefront.

Sampling on a plane must be higher to capture the spherical part of the wavefront.

FIGURE 8  Sampling a converging beam

Using a spherical reference surface in a converging or diverging beam allows the sam-pling to be determined by the aberrations in the beam, which are the phase distortions, or the deviations from the nominal spherical wave. This vastly reduces the sampling requirement for most optics problems.

Whether planar or spherical reference surfaces are to be used to sample a particular light beam depends on the curvature of the wavefronts that make up that beam, and also on the width of the beam. MACOS has functions that allow the geometric determination of the curvature. These functions fit pairs of reference surfaces into the beam at specified points, setting up near-field propagations.

The width of the beam grid relative to the amplitude grid is set to avoid aliasing. Alias-ing is an artifact of the FFT algorithm, which transforms amplitudes distributed in space to frequencies distributed in the spatial frequency domain. Because the FFT is a periodic calculation, the frequencies computed for the amplitudes are replicated at higher fre-quencies. If the beam is too wide compared to the extent of the amplitude matrix, some of the replicated “beams” can leak back into the main beam. This effect is aliasing.

The main symptom of aliasing in a diffraction computation is a “noisy looking” beam intensity pattern. Aliasing causes spikes to appear in the beam. These are usually most pronounced in regions of high intensity. Aliasing can be minimized by padding the amplitude matrix so that the gridpoints that coincide with the beam occupy about half of the width, or about a quarter of the area of the grid (Figure 9). The padding has the effect of restricting aliasing to very high-frequency effects which are usually of quite low amplitude. Additional padding can be specified by using smaller values of the nGridPts parameter, which governs the relative size of the ray and diffraction grid.

MACOS does not currently provide shaped windowing features.

The width of the beam in regions far from focus is determined geometrically. When the beam is small compared to the wavelength, the width is determined largely by diffrac-tion considerations. Near focus, diffraction effects dominate. MACOS is set up to auto-matically require adequate padding in diffraction calculations for imaging systems.

Nonetheless, the user is urged to be aware of the rules for constructing diffraction mod-els, as discussed in Section 6. Good discussions of these issues are available (Ref.

Lawrence).

The shape and quality of the point-spread function (PSF) are a good indicator of the per-formance of the optical system. The PSF is the image or intensity pattern due to a point

Physical Optics: Diffraction

L

pad

beam

1/4 L 1/2 L  1/4 L

FIGURE 9  Padding the amplitude matrix.

source. Without aberrations, geometrical optics predicts infinitely small images: a delta function of infinite amplitude and zero width. Alas due to diffraction from finite sized apertures of the optical system, the PSF is never a delta function. If the aberrations are small the imaging performance of a system is not degraded, but the performance is dif-fraction limited. As the aberrations in the optical system are increased, the PSF degrades further: its peak value decreases, the side-lobes grow, and the pattern spread out.

#### Single Plane Diffraction

The simplest and most common PSF calculation uses a ray trace to compute the OPD in the exit pupil of the optical system followed by a single far-field diffraction propagation to the focal plane. Figure 10 shows the PSFs computed by MACOS for the Cassegrain telescope calculated in this fashion (see Figure 6 for the OPD maps).

)

Unaberrated (defocused)	Aberrated OPD (coma)

FIGURE 10 Point spread functions for Cassegrain.in

#### Conventional Multi-plane Diffraction

Conventional physical optics modelling of multi-plane diffraction phenomena unfolds the optical beam train into an equivalent set of thin lenses arranged in linear fashion along the optical axis.2,4 The effect of the lenses is modeled using the thin-phase approximation. This approach assumes that:

      - The beam is paraxial, so that phase changes occur as a quadratic function of distance from the center of the array.

      - The grid does not change shape or dimension across the lens; the beam out of the lens is the same size as the beam into the lens.

      - The lens has no thickness, so all phase changes occur in a plane.

MACOS does not make these approximations. MACOS does not unfold the system nor use the thin-phase approximation.

#### Multi-plane Diffraction in MACOS

MACOS multi-plane diffraction models are set up by placing reference surfaces imme-diately before and after the optical elements of a system, or at the start and end of free-space propagations. These reference surfaces are optimized in pairs using MACOS ORS, SRS and FEX functions, discussed in Sections 5 and 6.

MACOS diffraction calculations use four basic propagators:

- Geometric propagator.

- Fresnel near-field sphere-to-sphere propagator for converging or diverging beams.

- Fresnel/Fraunhofer far-field sphere-to-plane propagator.

- Fresnel near-field plane-to-plane propagator for collimated beams.

The Fresnel propagators are used for the free-space propagations in much the same way as in standard codes. They determine the amplitude on the downstream reference sur-face based on the amplitude on the upstream reference surface. These propagators are described in more detail in Section 6.

MACOS uses the geometric propagator to determine phase changes across optical ele-ments. The geometric propagator uses rays as phase probes aligned with particular amplitude matrix grid points. The phases are determined directly from the exact optical path lengths calculated for the rays from the first reference surface, through the optical elements, to the next reference surface. If necessary due to aberrations through the opti-cal elements, the rays can be regridded to assure alignment with the diffraction grid points. Compared to thin-phase, the MACOS approach

  - Does not assume that the beam is paraxial through the optical element (though an assumption of paraxial beams is built into the Fresnel free-space propagators).

  - Does not assume that the beam out of the lens is the same size as the beam into the lens.

  - Does not assume all phase changes occur in a plane.

  - More accurately accounts for geometric effects, such as field-dependant aberrations.

The MACOS geometric propagator does not compute magnitude changes unless polar-ization ray-trace is being used.

Polarized Light

### Polarized Light

MACOS polarized light summary: wht polarized light is; representation of polarized TE fields using rays; polarization rt; interfaces; coatings; vector diffraction.

## SECTION 3: User Interface

We begin this section by discussing the mechanics of interacting with the MACOS application. Basic file-oriented functions are discussed, as are various graphics options. Memory requirements are discussed. The commands covered in this section are:

Help EXPort JOUrnal EXEcute

### Starting and Stopping MACOS

The particular command used to start the MACOS application program depends—of course—on which machine you are using. If your machine is a…

- Macintosh: open the folder containing the MACOS application and double-click on the MACOS 1.0 icon

- Unix: type the command MACOS with appropriate path names to point to the MACOS executable file

On all machines, the response is the same. A window appears with the MACOS com-mand dialog:

Modeling and Analysis for Controlled Optical Systems MACOS Version 2.8, May 31, 1999

This run is limited to 70 surfaces, 69 segments,	4226

rays (	441 for sensitivity calcs). Up to 3 diffraction planes are stored. Up to 10 interpolated surfaces of up to	255 data points are supported.

Maximum pixel array size is	128 by	128. Diffraction grid size is	128 by	128.

Glass table /home/comp/macosData/macos.glass read, with 202 glasses included. For a list of commands, type H

MACOS>

MACOS>q

The MACOS> prompt indicates readiness of the MACOS application to take commands. To exit MACOS, type Quit or BYE to end a session. The response you get depends on whether a PGPLOT graphics window is open (Unix machines running XWindows only). If so, MACOS will ask you to hit return before it goes away. On Sun machines a warning that “non-standard” floating point results may have been produced will be printed. This can be ignored.

Type <RETURN> for next page:

### Help

Type HELP (or ?) at the MACOS> prompt to see the list of all commands.  The output is paginated -- hit return to advance to the next screen.  Commands are grouped into 15 categories:

Session & files: QUit, END, HELP, REset, STatus, SUMmarize, ELTS, EXEcute, JOUrnal, MREset, !, PWD, CD, RX/LS/LL, VI/EMAcs

Prescription I/O: NEW, OLD, FID, SAVe, EXPort, SHOw, MODify

Source & wavelength: CHIefray, WLENS, SWL, MULtispec, NFIlt/RFIlt/SFIlt, ATMosphere, SETC, SAOpt

Ray tracing: RAY, SEGraytrace, PRAy/RRAy/TPR, MAP, COOrd, ACOor, EFL, BWK, FEXit, FSR, CENter, STOp, CENTRoid, FFP, PFP, FDP, SPCenter

Behavior toggles: LNEg/NOLNeg, POLarized/NOPol, OBS, REGrid/NOREGrid, ORS, SRS, NONe

Surface data: SINt, UDSinit

Perturbation: PERturb, GPERturb, PREad

Linear model: BUild, DMBuild, PARtials, LPErturb, LPRead, LREset, LSPot, LOPd, LPIxilate

Diffraction: PROpagate, BEAm, VECtor, SCAlar, COMpose, ADD, DAdd, CADD, NOIse, SEEd, STRetch

Outputs: OPD, SPOt, ZABerr, ZCOef, LOS, METcalc, INTensity, PIXel, AMPlitude, PHAse/PH, REAl, LOG, MTF/CMTF, IMG

Plot style: GRAy, WIRe, SLIce, COLumn, CONtour, IMGmode, CIR, GIR, PGP

File output: TEXt, BINary, FITs/WFIts, MAT, GETMatlabmatrix

Window & post-processing: PLOcate, NOPLOC, PWIn, SZCo, GBS, BLUr/GBLur, GAIn, ODRaw, PGD, ROW

System optimization: AVAR, MVAR, DVAR, VARS, AFOV, DFOV, FOVS, CALib, SFOV

Misc / debug: SRAy, SRT, SRTrace, DOPD, DOPL, ZRM, JWST_v3d / Vis3d

Casing convention.  Commands are shown as <UPPER prefix><lower tail>, e.g. RAYtrace, SUMmarize, PERturb.  The uppercase part is the minimum-match abbreviation tested by the dispatcher -- typing just RAY, SUM, or PER (case-insensitive) is sufficient.

Prerequisite tags. Lines in the HELP output may end in:

[Rx] -- needs a loaded prescription (run OLD or NEW first)

[BLD] -- needs a built linear model (run BUild or DMBuild)

[DIFF] -- needs a propagated wavefront (run PROpagate first)

Commands without a tag work in any state.  A complete reference of all commands appears in Appendix C.

### MACOS Model Size Specification

The first thing the user will be asked to do after starting MACOS is to specify a model size. The model size determines MACOS computational accuray, such as the number of rays traced and image grid resolution, and obviously also has an impact on the overall computer simulation time. MACOS currently supports the following model sizes: 128, 256, 512, 1024 and 2048 mainly because the fast Fourier transorm routine is optimized for power-of-2 arrays.

The MACOS model size can be changed at any time during a MACOS session. Use the mreset command to set a new model size, as shown in the following example to set a model size to 256:

MACOS> mreset 256

or

MACOS> mreset

Enter new size (128, 256, 512, 1024, 2048): [128]: 256

Modeling and Analysis for Controlled Optical Systems MACOS Version 3.2, February, 2003

This run is limited to 70 surfaces, 69 segments, 16385

rays (	500 for sensitivity calcs). Up to 3 diffraction planes are stored. Up to 10 interpolated surfaces of up to	255 data points are supported.

Maximum pixel array size is	256 by	256. Diffraction grid size is	256 by	256.

In previous versions of MACOS, separate model parameter files are used for different model sizes. For example, the model parameter file param256.cmn is used for the model size 256. In MACOS 3.2b, a single model parameter file called macos_param.txt is used for all model sizes. macos_param.txt contains parameter sets for all supported model sizes. To run MACOS, either macos_param.txt or a link to it must exist in the MACOS execution directory.

### Entering Commands

MACOS>old

MACOS commands can be abreviated using the first letters of the command. The nec-cessary letters are denoted in capital letters. However, they do not need to be entered in capitals as MACOS is case insensitive.

MACOS prompts the user for the next input. For example, as shown below, to enter an old file, the user types OLD. MACOS then prompts for the filename.

Enter file name: lens_array

Input file lens_array.in being loaded. Default used for SegXgrid

Optical Train Summary:

Tracing rays past	4 elements.

Aperture grid type = Circular	with 512 grid points.

Elt	1: LensArray	: Conic	with Kc= 0.0000D+00, Kr=-8.0000D-01, nECo-ords = -6

Elt	2: Refractor	: Conic	with Kc= 0.0000D+00, Kr=-1.0000D+18, nECo-ords = -6

Elt	3: FocalPlane : Flat	with Kc= 0.0000D+00, Kr=-1.0000D+18, nECo-ords = -6

Entering Commands

Elt	4: FocalPlane : Flat	with Kc= 0.0000D+00, Kr=-1.0000D+18, nECo-ords = -6

Number of output coordinates is	5

Too many grid points. Resetting npts to 128 MACOS>

MACOS also allows commands to be stacked on a single input line. The commands must be separated by a space.

MACOS>old lens_array show 2

Enter file name: lens_array

Input file lens_array.in being loaded. Default used for SegXgrid

Optical Train Summary:

Tracing rays past	4 elements.

Aperture grid type = Circular	with 512 grid points.

Elt	1: LensArray	: Conic	with Kc= 0.0000D+00, Kr=-8.0000D-01, nECo-ords = -6

Elt	2: Refractor	: Conic	with Kc= 0.0000D+00, Kr=-1.0000D+18, nECo-ords = -6

Elt	3: FocalPlane : Flat	with Kc= 0.0000D+00, Kr=-1.0000D+18, nECo-ords = -6

Elt	4: FocalPlane : Flat	with Kc= 0.0000D+00, Kr=-1.0000D+18, nECo-ords = -6

Number of output coordinates is	5

Too many grid points. Resetting npts to 128 MACOS> show

Enter number of element (0=aperture): [0]: 2

| iElt= EltName= Element= Surface= | 2 rs Refractor Conic |  |
| --- | --- | --- |
| fElt= | 1.000000000D+18 |  |
| eElt= | 0.000000000D+00 |  |
| KrElt= | -1.000000000D+18 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 2.000000000D-01 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 1.000000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 0.000000000D+00 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 0.000000000D+00 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

MACOS>

MACOS>show;;

MACOS provides defaults for nearly all requested parameters or values. These are usu-ally computed from contextual information. The value of the default for a particular parameter is shown in the prompt line enclosed in square brackets. To indicate that the default is to be accepted, the user can simply hit return when the prompt is displayed. If typing ahead, a semicolon can also be used to indicate that a default should be accepted. In the example below, the default element number 0 is used to select the aperture.

Enter number of element (0=aperture): [0]:

ChfRayDir= 0.000000000D+00 0.000000000D+00 1.000000000D+00 ChfRayPos= 0.000000000D+00 0.000000000D+00 -1.000000000D+00 zSource= 1.000000000D+22

IndRef=		1.000000000D+00 Extinc=		0.000000000D+00 Wavelen=		1.000000000D-04 Flux=	1.000000000D+00

GridType=	Circular Aperture=	5.000000000D-01 Obscratn=	0.000000000D+00

nGridpts= 128

xGrid=	1.000000000D+00 0.000000000D+00 0.000000000D+00 yGrid=	0.000000000D+00 1.000000000D+00 0.000000000D+00

nElt=	4

MACOS>

#### Interrupting MACOS (Ctrl-C)

Pressing Ctrl-C at the MACOS> prompt -- or during a long trace, build, or propagation -- prints 'MACOS: interrupted' and exits with status 0.  Internally MACOS installs a SIGINT handler at startup that uses _exit(0) so it is async-signal-safe and fires regardless of what the runtime is doing.  This replaces the Intel-runtime 'forrtl: severe (69)' traceback that earlier MACOS versions produced on Ctrl-C.

#### Empty input

Pressing return at the MACOS> prompt with no command is a no-op -- useful for clearing the input buffer or scrolling history without re-running the last command.

#### Unknown commands

Unknown commands print a single-line warning rather than the verbose multi-line traceback that v3.2 produced.

### Graphics Display

MACOS provides in-line graphical displays on Unix machines using the PGPLOT package from CalTech (see installation notes for information on how to obtain PGPLOT). PGPLOT is started up at the first invocation of a MACOS graphics function, such as INTensity. PGPLOT prompts the user to define the desired form for the graph-ical output:

MACOS>stretch

Enter image stretch type (LINEAR, LOG10, SQRT): [LINEAR]: log10

MACOS>intensity

Enter number of element where data is to be generated: [9]: 9

Tracing	3210 rays and propagating	16384 grid points... Scalar FF Prop between Elt 8 and Elt 9 to WF 1:

z1=-9.1481D+01 dx1= 1.0581D-01 min= 1.0581D-01 max= 1.0581D-01 dev= 2.6446D-06 lin=100.00%

z2= 1.0000D+22 dx2=-1.0996D-03

Compute time was	1.4453 sec

Graphics device/type (? to see list, default /NULL): /xwindow

Starting /opt/local/bin/pgxwin_server.

The normal selection for Unix machines is “/XWindow.” PGPLOT then sets up a graph-ics window in which plots are displayed. Once selected, the graphical output option can-not be changed without quitting and restarting MACOS.

### Off-Line Graphics Display

MACOS graphics can also be displayed and analyzed using other applications. This is the only display option for Macintosh. It is also extremely useful for users on all plat-forms who wish to study their results in ways not provided by MACOS.

One such application is Matlab®, which provides an easy-to-use, powerful environment for manipulating and displaying data in many forms. Matlab is a product of The Math Works, Inc., 24 Prime Park, Natick, MA, USA (508)653-1415. We have included Mat-lab “.m-file” functions that provide some of the standard MACOS display types. These scripts import a specified datafile, generate a plot according to specified plot and stretch types, and return the data values. They functions also provide templates for modification or translation to other languages. By keeping the display application (e.g. Matlab) run-ning in one window, and MACOS running in another, nearly seamless generation and display of results can easily be accomplished.

Generating the data for off-line display is as simple as selecting a file output “plot type,” using the BINARY, FITS, TEXT or MATLAB commands (Section 7). Subsequent use of graphics-generating commands such as INTensity then create a file and store the requested data in that file. If the file already exists, the user is asked if the file is to be

PostScript and GIF Graphics Output

overwritten; if not, a new name is requested. Other applications can then access and dis-play the data. Repeating the previous example:

MACOS>binary

Plot type set to BINARY MACOS>intensity

Enter number of element where data is to be generated: [9]: 9

Tracing	3210 rays and propagating	16384 grid points... Scalar FF Prop between Elt 8 and Elt 9 to WF 1:

z1=-9.1481D+01 dx1= 1.0581D-01 min= 1.0581D-01 max= 1.0581D-01 dev= 2.6446D-06 lin=100.00%

z2= 1.0000D+22 dx2=-1.0996D-03

Compute time was	1.0508 sec Wavefront Propagation Data Summary:

Wavelength= 1.6280000D-04; Source Flux= 1.0000000D+00 u-v plane diam= 1.3438007D+01 du= 1.0581108D-01

x-y plane diam=-1.3965216D-01 dx=-1.0996233D-03

Peak intensity= 1.1584734D-01; Peak occurs at i=	65, j=	65 Sum of intensity= 7.6908687D-01

Writing Intensity, Elt=9

DIRECT ACCESS File=tmo.int9	Data file is “tmo.int9”

Binary array is of dimension	128 by	128. Magnitude of central pixel is 0.3161060810D-01

MACOS>

The output file name (in this case, “tmo.int9”) is nominally generated from the prescrip-tion file name, with the datatype (“int” for intensity) and element number appended after a period. Fits, text and Matlab format files have “.fit,” “.txt,” or “.mat” appended as well, respectively.

The Matlab command required to read the file, generate the plot (at log10 stretch), and return the data for further analysis is:

>>intensity=macosIMG(‘gray’,’tmo.int9’,’log10’);

Here intensity is the returned intensity data, in this case a 128 by 128 matrix, ‘gray’ is the plot type (gray scale), ’tmo.int9’ is the file name, and ’log10’ is the stretch option. The result is a plot functionally identical to what MACOS would produce with the same settings.

Other applications and environments that you may find useful for analyzing and display-ing MACOS data include include SAOImage (available free from the Smithsonian Astrophysical Observatory for Unix only), Spyglass Transform, MatrixX, IDL, plotting packages such as DeltaGraph, and spreadsheets such as Excel.

### PostScript and GIF Graphics Output

MACOS provides PostScript and GIF graphical file output directly from PGPLOT (Unix only). When PGPLOT is started up, at the first invocation of a MACOS graphics function, it prompts the user to define the desired form for the graphical output. Typing

? prints a list of PGPLOT output options:

Graphics device/type (? to see list, default /NULL): ?

PGPLOT v5.0.0 Copyright 1994 California Institute of Technology Legal PGPLOT device types are:

/GIF	(Graphics Interchange Format file, landscape orientation)

/VGIF (Graphics Interchange Format file, portrait orientation)

/NULL (Null device, no output)

/PS	(PostScript file, landscape orientation)

/VPS	(PostScript file, portrait orientation)

/CPS	(Colour PostScript file, landscape orientation)

/VCPS (Colour PostScript file, portrait orientation)

/TEK4010 (Tektronix 4010 terminal)

/RETRO (Retrographics VT640 Tek emulator)

/XTERM (XTERM Tek terminal emulator)

/XWINDOW (X window window@node:display.screen/xw)

/XSERVE (A /XWINDOW window that persists for re-use)

/XDISP (pgdisp or figdisp server)

Graphics device/type (? to see list, default /NULL):

The GIF options generate separate GIF files for each graphic, named pgplot.gif, pgplot.gif_2, etc. in sequence.

The PS options generate a single file, pgplot.ps, containing all of the plots generated in a single session. It can be printed (on Unix machines) with the command:

% lpr pgplot.ps

The file can be renamed if you wish to save it.

Screen capture utilities and applications, such as SAOimage or XV, can also be very useful in recording MACOS graphical outputs.

### MACOS Files

#### .In-File

An optical system prescription created in MACOS is saved in a “filename.in” file. This file stores the prescription in a text format which can be edited offline and reused later. The MACOS prescription files are generated by the NEW command (Section 4).

#### .Out-File

MACOS no longer generates a “filename.out” file for recording each result generated in a session.

#### Glass Tables

Glass data is provided in a text file, named macos.glass, which is located in a special MACOS data directory (see next section). The form and content of this file is discussed in Section 4.4.3.

Alternatively, glass data can be provided as part of the optical prescription “.in-file.”

#### MACOS Data Directory

Beginning with Version 2.8, certain MACOS data files, such as glass tables, are being placed in a special directory. On Unix systems, the path to this directory can be specified by the user by defining an environment variable, MACOS_DATA. This is done in response to the Unix shell prompt, as in:

> setenv MACOS_DATA /home/macos/data/

MACOS Files

The above command establishes the directory /home/macos/data as the location of the data files for subsequent MACOS sessions. The user may specify any directory as MACOS_DATA.

If MACOS_DATA is not specified, 3 other directories will be searched for data files. In order these are:

- Directory macosData in the user’s home directory

- Directory /home/comp/macosData

- The current directory

If the required data file is not found in any of these directories, a warning message is issued. In this event, the user is required to supply all relevant data, such as glass coeffi-cients, through the prescription file.

#### Data Export

The EXPort command writes certain variables to a file, in several different formats (Section 7 discusses writing graphics data to files). The variables are useful when link-ing linear models to thermal or structural codes (see Section 8).

The following variables can be exported:

      - CMAtrix: optical sensitivity matrix in global or local coordinates

      - CmDM: deformable mirror sensitivity matrix

      - RayPos: current position 3-vectors for each ray

      - RayDir: current ray direction 3-vectors for each ray

      - CumRayL: current optical pathlengths for each ray Files may be saved in the following formats:

      - BINary: data is saved to a Fortran binary file, filename.bin

      - TEXt: data is saved to an ASCII text file, filename.txt

      - MFILE: data is saved to a Matlab file, filename.m

The following is a sample EXPort session.

MACOS>build ;

Enter terminal element number: [1]:

Tracing 12669 rays but not BUILDing linear model. BUILD limited to	500 rays. MOD npts and recompute. MACOS>export

Current element is	1

Enter EXPORT mode (BINary, TEXt, MFIle, ISM, H or Quit): [BINARY]: tex

Exporting data to lens_array.txt

Export data for CHFRay only, ALLRays, or NOChfray? [CHFRay]: chfr

EXPORT (CMAtrix, CmDM, RAYPos, RAYDir, RAYL, H or Q): [QUIT]: raypos

% RayPos 3-vectors for	1 rays at Element	1

EXPORT (CMAtrix, CmDM, RAYPos, RAYDir, RAYL, H or Q): [QUIT]: q MACOS>

### Using Macros

MACOS has a scripting or macro capability. This is provided by keystroke recording and play-back commands, JOUrnal and EXEcute. Macros can call other macros, up to 10 deep. Comments (anything following a “%” on a line) and blank lines are ignored. Looping, conditionals and other “language” capabilities are not currently supported.

#### Journal Files

The JOUrnal command is used to start and stop a journal of all the commands the user inputs to the terminal. Typing the JOUrnal command initiates keystroke recording, which continues until toggled off by typing the JOUrnal command a second time.

JOUrnal allows the user to keep track of the commands entered in a MACOS session. The resulting .jou-file can later be used as a macro command (see Section 3.9.2). The

.jou-file can also be edited and modified offline.

The JOUrnal command has one argument, the filename. The filename entered by the user will automatically have the file extension .jou appended, to denote that the file is a JOUrnal file. If the file already exists, then the contents of the file are deleted before the JOUrnalizing begins. It is imperative that the user select a unique filename, if is it desired that old JOUrnal files be kept.

#### Executing Macros

The EXEcute command allows the user to designate a .jou-file from which further MACOS commands are read. Each command (including other EXEcute commands) is executed in sequence, until an END OF FILE symbol is read from the .jou-file.

MACOS then returns to interactive mode and the user can once again enter commands from the keyboard.

The EXEcute command has one argument, the filename of the batch file. The filename must have the file extension .jou to denote that the file is an journal file.

The user can edit a file produced with the JOUrnal comand to produce a macro. For example, if the .jou-file was created with an OPD at the 10th element, it is trivial to modify it to calculate the OPD at the 14th element. The user only needs to change the 10 in the .jou-file to a 14 using a text editor.

Examples are provided in the macosDemo directory distributed with the MACOS code.

### Memory Requirements

The MACOS application currently uses a fixed memory allocation set at compile time. Different run modules are supplied for different sized problems, depending on the machine and available memory. Table 4 summarizes requirements for specific plat-forms.

TABLE 4. Memory Requirements

| executable | Sun/Solaris | PowerMacintosh |
| --- | --- | --- |
| macos128 | 9.75 MB |  |
| macos256 | 18.3 MB |  |
| macos512 | 38.4 MB |  |
| macos1024 | 115 MB |  |

## SECTION 4: Describing Optical Systems

Systems

A first MACOS session typically begins by invoking the NEW command. This begins a dialog whose function is to define the data used by MACOS in its computations. This data comprises the optical prescription, and is saved by MACOS as an “.in-file,” which can be edited and reused later using the OLD command. The contents of the .in-file can be altered within a MACOS session using MODIFY, PERTURB or other commands, and saved as a new .in-file using the SAVE command. The .in-files can also be created and edited off-line using a text editor. The commands covered in this section are:

NEW DRAw

OLD (LOAd)

SUMmarize STAtus RESet SHOw

MODify BEAM SAVe

### Specifying the Light Source

The NEW dialog begins by asking the user for information about the light source, such as where it is located, in what direction it is oriented, and what its optical properties are. After the source is defined, similar questions are asked about each optical element in turn. This section is intended to help the user provide the appropriate answers to these questions. It is also intended to define MACOS variable names, which are used in the

.in-files to specify aspects of the prescription.

MACOS represents light sources as beams, consisting of a ray bundle and an associated complex amplitude matrix (or matrices). Beams can be collimated or focused, as per Figure 11. In addition, the illumination profile of a beam can be specified as uniform, gaussian, dipole or cosine-to-a-power, using the BEAM command discussed in a later sec-tion. Collimated beams are composed of a bundle of parallel rays, originating at a plane in front of the entrance aperture. Collimated beams are used to represent the light received from a point-source at an infinite distance from the system. They are also used when collimated light sources, such as some lasers, are to be modeled. Focused beams are bundles of rays converging to or diverging from a point in space and represent the light from a nearby point source. Focused beams are defined on a spherical surface whose center of curvature is at the source point.

The properties that must be specified by the user include:

- Source position and direction, defined by the position and orientation of the “chief” ray (really the gut ray – the ray that lies in the center of the beam)

- Aperture size (collimated source) or angle (point source)

- Aperture shape (circular, square, hexagonal, pie, flower)

- Illumination profile (flat, gaussian, dipole or cosine-to-a-power)

- Wavelength

- Flux through the defined aperture

- Polarization state (if using polarization ray-trace analysis functions)

- Segmentation (unsegmented, hexagonal segments, or “pie” segments)

- Coordinates to define the orientation of the beam to the optical elements

#### Source Position and Orientation

In MACOS, a light source is a surface but not an element. The surface is defined by a single wavefront emitted from a single point source. The zSource parameter is the dis-tance from the origin of the light to the light source surface, as defined by the location of the chief ray, ChfRayPos (see Figure 11). Collimated sources are specified by zSource=1d22. For point sources, the convention is that the sign of zSource is positive for a converging beam and negative for an expanding beam.

ChfRayPos

ChfRayPos	ChfRayPos

zSource=1d22

Collimated beam

zSource<0	zSource>0

Diverging beam	Converging beam

FIGURE 11 Light sources

The location and orientation of the light source is determined from the chief ray. The MACOS “chief ray” is the central or gut ray of the beam, as discussed in Section 2.2. It is the center ray of the incident beam bundle. It is normally a true chief ray as well, pass-ing through the center of the system stop. This condition is enforced by the user, using the STOP or CENTER commands, as described later.

To define the source, the user must specify in the global coordinate system:

- Chief ray position vector

- Chief ray direction vector

These are set by the 3-vector inputs ChfRayPos and ChfRayDir. If the source is colli-mated, then every ray in the beam starts out with its direction equal to ChfRayDir. The position of each ray is calculated from ChfRayPos, the shape and type of aperture, and the specified ray density. If the source is a point source, appropriate position and direc-tion is computed so that each ray starts from the surface of a sphere centered at the source point.

NOTE: Converging or diverging sources should not be located at the source point, or, for exam-ple, divide-by-zero errors will result when performing diffraction analysis.

#### Refractive Index and Extinction Coefficient at the Source

The refractive index is, in general, a complex number. The real part is the conventional index, giving the speed of light in the particular medium, and so defining the direction

changes a ray experiences at refractive interfaces. The imaginary part is the extinction

Specifying the Light Source

coefficient, which defines the attenuation of the field in the medium. Only the index (real part) is significant in MACOS scalar ray-tracing calculations. The extinction coef-ficient is used only when polarization ray-trace is selected, by use of the POL command.

- The real part of the refractive index of the source medium is specified by IndRef. If ChfRayPos is in a vacuum, then IndRef=1. For many applications it is sufficient to approximate the index of refraction of air as 1, although the exact value depends on the pressure and temperature.

- The negative of the imaginary part of the refractive index, the extinction coefficient, of the source medium is specified by Extinc. For lossless media such as vacuum and air, set ExtInc=0. For perfect metals, set ExtInc=1d22. This parameter is used only when polarization ray-tracing is turned on.

The user must specify the index explicitly for the source only. For other elements, par-ticular glasses can be specified, and the index will be varied as a function of the wave-length of the beam. The source IndRef and Extinc are not changed automatically with changes in wavelength. The user can implement such changes as needed using the MOD-IFY command.

#### Wavelength

The wavelength is specified by the wavelen variable. For most calculations, MACOS uses one wavelength at a time. Multiple-wavelength images can be generated using the COMPOSE command to define detector parameters, using use the MODify command to change the wavelen (and Flux) variables (see Section 4.10.2), then using the ADD com-mand to separately propagate and accumulate different wavelength and flux level images at the detector. This procedure can be automated by using “.filt” filter files and the MULtispec command.

#### Flux

The total flux of the source illumination through the aperture area (defined by the Aper-ture, Obscuration and GridType parameters) is specified by Flux.This parameter

is used in MACOS to facilitate the radiometric modeling of systems. It is useful when constructing composite images of multiple sources or at multiple wavelengths. This pro-cedure can be automated by using “.filt” filter files and the MULtispec command.

#### Aperture

The aperture size is set by the Aperture parameter, whose interpretation is different for collimated or point sources. If the source is collimated (zSource=1d22), Aperture is the diameter of the input beam. If it is a point source(zSource<1d22), Aperture is the full angle (entered in radians like all angles in MACOS) subtended by the two marginal rays (see Figure 12). Thus, the numerical aperture of the system is

N.A. = n*sin(Aperture)	(4.1)

where n is the index of refraction.

Obscurations can be defined in the source beam if desired. Obscuration size is specified in the same way as the aperture, with the parameter Obscratn setting the diameter or angle of a central obscuration.

Aperture

Collimated beam

Aperture

Diverging beam	Converging beam

(zSource = )

(zSource < 0)

(zSource >0)

FIGURE 12 Aperture sizing

NOTE: This aperture obscuration option can be useful, but is not the best way to represent most beam-train obscurations. It is better to explicitly place obscurations on the appropriate elements as discussed in Section 4.6.

The entrance aperture can have several shapes, as shown in Figure 13, depending on whether the system is monolithic – in which all optics are of one piece – or segmented – in which at least one mirror is split into separate segments. For monolithic systems, the aperture shape can be round or square, as set by the GridType parameter. For segmented systems the options are hexagonal array, round with 6 pie-shaped outer segments, or round with an arbitrary number of outer segments. For computational efficiency, the segmentation is defined at the light source.

GridType=Circular	GridType=Rectangular

GridType=Flower

GridType=Hex	GridType=Pie

FIGURE 13 Aperture shapes

It is not necessary to segment the aperture to use lenslet arrays, as discussed later.

NOTE: Registration of a segmented aperture on a segmented mirror may shift when the beam inci-dence angle changes, or when intervening optics are perturbed. This can cause rays that nominally strike one mirror segment to move to another. In these conditions, the user should check to ensure proper registration.

#### Ray Grids

The density of rays across the beam and the orientation of the ray grid are set by three

parameters:

Example Light Source

- Number of ray grid points across the aperture is set by nGridPts

- Ray grid coordinate vectors xGrid and yGrid determine the global orientation of the ray grid and so the beam coordinates

The ray grid is laid out on the surface that defines the light source, according to a coor-dinate system for which zGrid equals the negative chief ray direction ChfRayDir and xGrid is as specified by the user (see Figure 14). The number of rays across the aperture is set by nGridPts.

yGrid

Coordinates

yGrid=zGrid x xGrid

xGrid

zGrid

Chief ray

xGrid

nGridPts=7

FIGURE 14 Ray grid layout

### Example Light Source

MACOS obtains the information it needs to build an optical prescription (an “.in-file” dataset) through the NEW file definition dialog. The first questions in this dialog are about the light source and the latter questions pertain to the optical elements.

For the first example, we start the prescription for a Cassegrain telescope, Casseg-rain.in, shown in Figure 15. The light source is a point source at infinity so the wave-front incident on the telescope is collimated. In the example dialog, the prompts are exactly as MACOS gives them. The correct user response is indicated in bold text, and the name of the variable being set is in italic text.

Secondary Mirror

Detector

Reference Surface

Primary Mirror

FIGURE 15 Cassegrain telescope layout

Prompt	Response	VariableName

MACOS>new

Enter file name: Cassegrain

File Cassegrain.in being created.	filenam

Enter chief ray input direction vector (x,y,z) [0.,0.,1.0]:0,0,1  ChfRayDir Enter chief ray location vector (x,y,z) [0.,0.,0.]:0,0,-5	ChfRayPos Enter distance from source [1.000000E+22]:1d22		zSource

Enter chief ray index of refraction [1.00000]:1	IndRef

Enter chief ray extinction coefficient [0.]:0	Extinc

Enter monochromatic wavelength [6.000000E-07]:1d-6	Wavelen

Enter intensity (flux over aperture area) [1.00000]:1	Flux

+	+

| Source ray grid types:	|

|	Circular	Square	Hex	|

|	Pie	Flower	|

+	+

Enter source grid type [Circular]: Circular	GridType

Enter aperture diameter or angle [1.00000]:4	Aperture

Enter number of grid points [65]:15	nGridPts Enter input plane x-axis vector (x,y,z) [1.00000,0.,0.]:-1,0,0		xGrid Enter input plane y-axis vector (x,y,z) [0.,1.00000,0.]:0,-1,0		yGrid Enter number of elements [no default]: 5			nElt

This dialog produces the following lines in the prescription file Cassegrain.in: ChfRayDir= 0.000000000D+00 0.000000000D+00 1.000000000D+00

ChfRayPos= 0.000000000D+00 0.000000000D+00 -5.000000000D+00 zSource= 1.000000000D+22

IndRef=		1.000000000D+00 Extinc=		0.000000000D+00 Wavelen=		1.000000000D-06 Flux=	1.000000000D+00

GridType=	Circular Aperture=	4.000000000D+00 Obscratn=	0.000000000D+00

nGridpts=	15

xGrid= -1.000000000D+00 0.000000000D+00 0.000000000D+00 yGrid=	0.000000000D+00 -1.000000000D+00 0.000000000D+00

nElt=	5

The Cassegrain.in example is finished in Section 4.8.

Now consider the case of a non-collimated input beam, zSource not equal to infinity. A Cassegrain telescope has been turned into a beam projector. The light source is now located where the detector was previously located as shown in Figure 16.

MACOS>new

Enter file name: zsourc

File zsourc.in being created.

Enter chief ray input direction vector (x,y,z) [0.,0.,1.0]:0,0,1  ChfRayDir Enter chief ray location vector (x,y,z) [0.,0.,0.]:0,0,-1	ChfRayPos Enter distance from source [1.000000E+22]:-0.1		zSource

Enter chief ray index of refraction [1.00000]:1	IndRef

Enter chief ray extinction coefficient [0.]:0	Extinc

Enter monochromatic wavelength [6.000000E-07]:1d-3	Wavelen

Enter intensity (flux over aperture area) [1.00000]:1	Flux

+	+

| Source ray grid types:	|

|	Circular	Square	Hex	|

|	Pie	Flower	|

+	+

Enter source grid type [Circular]: Circular	GridType

Enter aperture diameter or angle [1.00000]:0.1076	Aperture

Enter number of grid points [65]:25	nGridPts

Enter input plane x-axis vector (x,y,z) [1.00000,0.,0.]:-1,0,0	xGrid

Specifying Optics

-1.1 -1.0

0.15

1.5

Z

ChfRayPos

FocalPln

zSource

3.0

FIGURE 16 Non-collimated light source example

Enter input plane y-axis vector (x,y,z) [0.,1.00000,0.]:0,-1,0	yGrid

Enter number of elements [no default]: 6	nElt

This results in the following lines in the prescription file zsource.in:

ChfRayDir= 0.000000000D+00 0.000000000D+00 1.000000000D+00 ChfRayPos= 0.000000000D+00 0.000000000D+00 -1.000000000D+00 zSource= -1.000000000D-01

IndRef=		1.000000000D+00 Extinc=		0.000000000D+00 Wavelen=		1.000000000D-03 Flux=	1.000000000D+00

GridType=	Circular Aperture=	1.076000000D-01 Obscratn=	0.000000000D+00

nGridpts=	25

xGrid= -1.000000000D+00 0.000000000D+00 0.000000000D+00 yGrid=	0.000000000D+00 -1.000000000D+00 0.000000000D+00

nElt=	6

Notice that the Aperture entry has changed significantly, since Aperture now repre-sents an angle, instead of the diameter of the input beam. The ChfRayPos is now a small distance in front of the source, so zSource is now negative to indicate an diverging beam.

### Specifying Optics

The next part of the new dialog is the specification of the optical elements. The first step in defining the beam train is actually the last step in the source dialog: specifying the number nElt of optical elements in the beam train. The number of elements is equal to the number of active optical surfaces, plus any reference (dummy) surfaces, plus 2 return surfaces for an exit pupil, if desired.

The new dialog continues by prompting the user for all data needed to define each optic. Optics are specified one at a time, in the sequence in which they are encountered by the light from the source (except nonsequential and segmented optics). Some optics, such as mirrors, are realized by a single element, or optically active surface. Other optics require

two or more elements. Lenses require two refracting elements, one for each surface (see the example in Section 4.4.2). Corner cubes require three non-sequential reflecting ele-ments. Each element in multiple-element optics is entered separately.

The first step in specifying an element is to define its type (e.g., Reflector) and its sur-face type (e.g., Conic). Table 5 lists the elements currently supported by MACOS, and Table 6 lists the surface types. Further data required to define the surfaces may be requested, depending on specified surface type.

TABLE 5. Element types

| Element Type (Element=...) | Allowed Surface Types | Description |
| --- | --- | --- |
| Reflector | all | reflective surfaces (e.g., mirrors) |
| Refractor | all | refractive surfaces (e.g., lenses) |
| LensArray | conics or aspheres | lens arrays |
| Grating | all | reflective diffraction gratings |
| HOE | all | reflective holographic optical ele-ments |
| NSReflector | conics | non-sequential reflective surfaces |
| NSRefractor | conics | non-sequential refractors |
| Segment | all | segmented mirrors |
| ObsSrf | conics | obscuring surfaces |
| FocalPln | flats | focal planes |
| RefSrf | conics | reference surfaces |
| ReturnSrf | conics | return surfaces |

TABLE 6. Surface types

| Surface Type (Surface=.. .) | Surface Description |
| --- | --- |
| Flat | Planar |
| Conic | Conicoid of revolution. Precise shape is defined by conic constant KcElt and radius KrElt parameters (or fElt and eElt) |
| Toric | Toric or cylindrical shapes, defined by separate radius parameters in x and y directions. |
| Anamorphic | “Saddle” shapes, defined by separate conic and radius parameters in x and y directions. |
| Aspheric | Base conicoid, with 10th-order aspheric deviation. |
| Monomial | Base conicoid, with 14th-order cartesian polynomial deviation. |
| Zernike | Base conicoid, with deviation defined by Zernike polynomials. |
| Interpolated | Base conicoid, with deviation defined by X-Y-Z data triplets |
| GridData | Base conicoid, with deviation defined by square matrix pixels |
| User-defined | Deformable mirrors defined using influence functions. |

The second step is to define element location, orientation and rotation point, as dis-cussed in Section 2.1:

- Element location is specified by the VptElt vector. This is a three-vector in global coordinates giving the location of the vertex of the element.

Element Types

- Element orientation is specified by the psiElt vector. This is a unit magnitude three-vector in global coordinates giving the direction of the principal axis of the element.

- Element rotation point, RptElt, defines the pivot point for rotations of the element implemented using the Perturb command. It also defines the coupling between rotation and translation for the Build command. Together with any element coordi-nates, Telt, RptElt governs the element response to perturbations.

The third step is to define any aperture or obscurations that might occur at the element. Apertures define the margins of the element, and can be circular or rectangular. Obscu-rations are circular or rectangular regions on the surface of the element where light is completely absorbed. Negative obscurations “un-absorb” the light, countering the effect of an obscuration in regions where they overlap. Obscurations and apertures can be placed anywhere on the surface.

The fourth step is to define the zElt and PropType diffraction propagation parameters, which are needed if the element is at the start or end of a diffraction propagation.

The final step is to specify element coordinates (Telt), if they are desired.

In the next several subsections, each of these steps is discussed more carefully, and examples are shown.

### Element Types

Element type and surface shape are each most critical in the determination of the prop-erties of an optic. In this subsection we define the MACOS element types and illustrate their use through a sequence of specific examples.

#### Reflectors

Mirrors are entered as type Reflector. Reflectors can have any surface type or shape; the user will be prompted for different parameters depending on which surface type is chosen. No element-specific parameters are required for reflectors unless polarization ray-tracing is turned on. In that case, Extinc and IndRef should be carefully specified, as they determine the transmission of the beam past the reflective interface.

This example shows how to enter a flat mirror in MACOS:

Enter Element	1 Data:

Enter element name (no blanks) [Elt]: flatMirror	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :reflect	Element

+	+

| ¦ Surface types: |  |  | ¦ |
| --- | --- | --- | --- |
| ¦ Flat | Conic | Aspheric | ¦ |
| ¦ Anamorphic | Zernike | Monomial | ¦ |
| ¦ Interpolated | UserDefined |  | ¦ |

+	+

Enter surface type :flat	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1	psiElt Do you want an aperture on this element? [NO]: no

Enter number of obscurations [0]:0

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]: no

Enter element index of refraction [1.00000]:1	IndRef

Enter element extinction coefficient [1.000000E+22]:1d22	Extinc

This results in the following lines in the prescription file:

| iElt= EltName= Element= Surface= | 1 flatMirror Reflector Flat |  |
| --- | --- | --- |
| fElt= | 1.000000000D+22 |  |
| eElt= | 0.000000000D+00 |  |
| KrElt= | -1.000000000D+22 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 1.000000000D+22 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 1.000000000D+22 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

#### Refractors

A refracting element is the interface between two media of differing index of refraction, such as air and glass. Rays traversing the interface are, of course, bent at the interface. lenses, windows and other refracting optics are actually modeled using 2 optical ele-ments, both of type Refractor. The first element defines the front surface of the optic and sets the index for the ray traveling inside the optic. The second element defines the back surface, and resets the index to the value outside the optic.

Some refracting elements, such as prisms, use more than two elements, and may require use of nonsequential reflectors and refractors as well – see Section 4.4.13. Arrays of len-slets are covered in Section 4.4.4.

As with reflectors, refractors in MACOS may have any surface shape. The index of refraction IndRef can be set directly in the prescription, or indirectly, by specifying a particular glass for the element. To set the index directly, select FixedIndex glass type and then specify the index. No other element-specific parameters are required, unless polarization ray-tracing is turned on. In that case, Extinc must be specified as well.

This example lens has a spherical front surface and a planar rear surface, as illustrated in Figure 17:

Element Types

Enter Element	1 Data:

Enter element name (no blanks) [Elt]: lens_front	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	TrGrating |

|	RfPolarizer	TrPolarizer	Obscuring |

+	+

Enter element type :refractor	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	Grid data |

|	Toric	InfluenceFcn	|

+	+

Enter surface type :conic	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1	psiElt Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:1	fElt

Enter element eccentricity [0.]:0	eElt

Do you want an aperture on this element? [NO]: no

Enter number of obscurations [0]:0

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]: no

Enter glass type [FixedIndex]: BK7	GlassElt

Enter Element	2 Data:

Enter element name (no blanks) [Elt]: lens_back	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	TrGrating |

|	RfPolarizer	TrPolarizer	Obscuring |

+	+

Enter element type :refractor	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	Grid data |

|	Toric	InfluenceFcn	|

+	+

Enter surface type :flat	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0.1	VptElt Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1	psiElt Do you want an aperture on this element? [NO]: no

Enter number of obscurations [0]:0

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]: no

Enter glass type [FixedIndex]: FixedIndex	GlassElt

Enter element index of refraction [1.00000]:1	IndRef

Enter element extinction coefficient [0.00000]:0	Extinc

NOTE: It is not uncommon to “lose” rays, if they miss the properly defined area of a lens, that is, if the rays miss the region where the 2 surfaces of the lens are sequential. If rays extend beyond the intersection point of the two elements, the rays become undefined and are not traced further as shown in Figure 17.

This ray hits the elements out of sequence, becomes undefined

lens_back

lens_front

FIGURE 17 Ray bundle must pass within surface intersection points of the refracting elements for all rays to be traced properly

#### Glass Tables

Version 2.8 of MACOS comes with a single glass table, which incorporates the Schott glass types as of May 1999 (from http://schottglass.com). This table is a text file, macos.glass, located in the MACOS data directory. See Section 3.8.4 for details on how to specify this directory.

The user may specify the index of glasses or other optical materials in several ways:

      - By selecting from the materials in the provided macos.glass file, as in the first part of the example in Section 4.4.2.

      - By using a different macos.glass file, located in the MACOS data directory. The user can easily add other materials to a private copy of macos.glass, using the MACOS_DATA environment variable to define its location.

      - By directly specifying the 6 Sellmeier glass coefficients in the prescription. These coefficients are B1, B2, B3, C1, C2, C3 from the manufacturers catalog. Note that the MACOS SAVE command saves the coefficients of each glass in the prescription file, even if the glass data was originally derived from the MACOS glass table.

Element Types

- By declaring a material to be FixedIndex, and then specifying the index, in the pre-scription. This approach uses a single value of the index for any wavelength of light.

The form of the glass table is simple. Each glass is defined in a single row of the file, with the first entry being the name of the glass, followed by the 6 Sellmeier coefficients. The following shows the first few entries from macos.glass:

| Air 0d0 0d0 0d0 | 0d0 0d0 0d0 |  |  |  |
| --- | --- | --- | --- | --- |
| BaF13 1.57068590E+00 02 1.13934126E+02 | 1.51295152E-01 | 1.07638026E+00 | 9.39377487E-03 | 4.49479866E- |
| BaF3 1.32064267E+00 | 1.33572683E-01 | 8.85521821E-01 | 8.87798715E-03 | 4.20290346E- |
| 02 1.11729167E+02 |  |  |  |  |
| BaF4 1.37492590E+00 | 1.46354500E-01 | 9.12643359E-01 | 9.29554343E-03 | 4.40426654E- |
| 02 1.16607899E+02 |  |  |  |  |

#### Lens Arrays

Lens arrays (a.k.a. lenticular arrays or lenslet arrays) are refracting optics. Usually, a lens array will have one flat surface and one surface figured into a mosaic of regularly distributed lenslets (Figure 18). Lens arrays are used most often in Hartmann wavefront sensors. An example adaptive optics wavefront sensor is provided in Appendix A.4.

MACOS uses a LensArray element type to model the bumpy surface of a lens array. LensArray elements are specified much as Refractor elements are, with the addition of a few new parameters. LensArray elements can have conic or aspheric lenslet shapes. Enter shape parameters that describe a single lenslet, and all the other lenslets will be automatically replicated. The location and orientation parameters VptElt and psiElt define the plane containing all of the lenslet vertices, and specify the orientation of each lenslet.

The first new parameter is LensArrayType, which can have two values: LensArray-Typ=1 for hexagonal arrays; and LensArrayTyp=2 for rectangular arrays. The second new parameter is LensArrayWidth, which is the width of a single lenslet (flat to flat for hexagonal lenslets) (Figure 18).

yMon

yMon

LensArrayWidth

pMon

Len

xMon	xMon

Top View, rectangular array (LensArrayTyp=1)

sArrayWidth

Top View, hex array (LensArrayTyp=2)

Side View of Center Row

FIGURE 18 Hexagonal lens array

LensArray elements use a surface coordinate system to define the location and orienta-tion of the lenslet grid on the underlying plane. The variable pMon is the center of the coordinate system, defining the vertex position of the “central” lenslet. pMon can differ

from VptElt, though only the projection of pMon onto the plane is significant. The vari-ables xMon, yMon, and zMon define the surface coordinate frame as shown in Figure 27.

An additional example of the use of lenslet arrays is provided in Appendix A.4.

Enter element name (no blanks) [Elt]: LensArray	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :LensArray	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,1	psiElt Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:2

Enter Conic Constant [0.]:0	KcElt

Enter Radius [-1.000000E+22]:-0.8	KrElt

Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [0.800000]:1d22	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

Enter glass type [FixedIndex]: FixedIndex	GlassElt

Enter element index of refraction [1.00000]:1.5	IndRef

Enter element extinction coefficient [0.]:0	Extinc

Enter surface reference point location (x,y,z) [0.,0.,0.]:0,0,0	pMon Enter surface x-axis unit vector (x,y,z) [1.00000,0.,0.]:1,0,0	xMon Enter surface y-axis unit vector (x,y,z) [0.,1.00000,0.]:0,1,0	yMon Enter surface z-axis unit vector (x,y,z) [0.,0.,1.00000]:0,0,1	zMon Orthoganalized zMon= 0. 0.  1.0000000000000

Enter lenslet width [1.00000]:0.1	LensArrayWidth

Enter lens array type (1=hex, 2=square) [1]:1	LensArrayType

This results in the following lines in the prescription file:

iElt=	1

EltName= LensArray Element= LensArray Surface= Conic

fElt= 8.000000000D-01 eElt= 0.000000000D+00 KrElt= -8.000000000D-01 KcElt= 0.000000000D+00

psiElt= 0.000000000D+00 0.000000000D+00 1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 RptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00

IndRef= 1.500000000D+00

Element Types

Extinc= 0.000000000D+00

pMon= 0.000000000D+00 0.000000000D+00 0.000000000D+00 xMon= 1.000000000D+00 0.000000000D+00 0.000000000D+00 yMon= 0.000000000D+00 1.000000000D+00 0.000000000D+00 zMon= 0.000000000D+00 0.000000000D+00 1.000000000D+00

LensArrayType= 1 LensArrayWidth= 1.000000000D-01

nObs=	0

ApType=	None

zElt=	1.000000000D+22

PropType=	Geometric nECoord=	-6

#### Reflective Gratings

Diffraction gratings are surfaces that have been ruled into a highly regular array of grooves. The rulings scatter light in all directions. The periodicity of the ruling causes there to be coherence in the light scattered in particular discrete directions. These direc-tions correspond to various “diffraction orders,” which are functions of the ruling direc-tion, the ruling width, and the wavelength of the incident beam (ref. Hecht). The amount of power scattered into each order is a more subtle matter, depending on the the detailed shape of the rulings. MACOS provides grating elements that model the beam diffracted into any specified order, but does not predict the intensity of the diffracted beams.

MACOS gratings can be defined on any surface shape. Enter surface parameters to define the underlying element, just as you would to define a regular mirror. Three addi-tional element-specific parameters are required to define the grating geometry. These are:

      - OrderHOE: The diffraction order you wish to trace off of the grating surface.

      - h1HOE: A unit 3-vector perpendicular to the ruling direction and the psiElt vector. This vector is rotated and translated appropriately if the element is perturbed using the MACOS Perturb command.

      - RulingWidth: The fixed distance between rules as projected to a flat plane underly-ing the surface. Distance between rules along the curved surface can vary if the sur-face shape is curved, which will be the case with a conic or aspheric surface type.

The following example shows the new dialog used to specify a grating on a flat surface. The complete prescription, with a collimated source illuminating the grating at normal incidence, is provided in Appendix A.2. Figure 19 shows the diffracted beam at a cou-ple of orders.

Order=1	Order=-2

FIGURE 19 Diffraction grating showing diffraction orders 1 and -2.

Enter Element	1 Data:

Enter element name (no blanks) [Elt]: Grating

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Grating

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Flat

Enter element vertex location (x,y,z) [no default]: 0,0,0

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0

Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1 Do you want an aperture on this element? [NO]: no

Enter number of obscurations [0]:0

Enter Fresnel propagation distance [1.000000E+22]:1d22

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric

Do you wish to use element-based coordinates?: [NO]: no

Enter element index of refraction [1.00000]:1

Enter element extinction coefficient [1.000000E+22]:1d22

Enter diffraction order [1.00000]:1		OrderHOE Enter spacing of grating ruling [1.00000]:18.97366596d-7	RuleWidth Enter vector perp to grating ruling (x,y,z) [no default]: 0,1,0			h1HOE

This dialog produced the following prescription data:

iElt=  1

EltName= Grating Element= Grating Surface= Flat

KrElt= -1.000000000D+22 KcElt= 0.000000000D+00

psiElt= 0.000000000D+00 0.000000000D+00 -1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 RptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 IndRef= 1.000000000D+00

Extinc= 1.000000000D+22

h1HOE= 0.000000000D+00 1.000000000D+00 0.000000000D+00 OrderHOE= 1.000000000D+00

RuleWidth= 1.897366596D-06

nObs=	0

ApType=	None

zElt=	1.000000000D+22

PropType=	Geometric nECoord=	-6

Element Types

#### Transmissive Gratings

The transmissive grating elements, like the reflective gratings of the previous section, use surfaces that have been ruled into a highly regular array of grooves to diffract light. The periodicity of the ruling causes there to be coherence in the light scattered in partic-ular discrete directions. These directions correspond to various “diffraction orders,” which are functions of the ruling direction, the ruling width, and the wavelength of the incident beam (ref. Hecht).

MACOS grating elements model the beam diffracted into any specified order, but do not predict the intensity of the diffracted beams.

MACOS gratings can be defined on any surface shape. Enter surface parameters to define the underlying element, just as you would to define a regular mirror. Three addi-tional element-specific parameters are required to define the grating geometry. These are:

      - OrderHOE: The diffraction order you wish to trace off of the grating surface.

      - h1HOE: A unit 3-vector perpendicular to the ruling direction and the psiElt vector. This vector is rotated and translated appropriately if the element is perturbed using the MACOS Perturb command.

      - RulingWidth: The fixed distance between rules as projected to a flat plane underly-ing the surface. Distance between rules along the curved surface can vary if the sur-face shape is curved, which will be the case with a conic or aspheric surface type.

#### Reflective Holographic Optical Elements (HOEs)

Holgraphic optical elements (HOEs) are diffractive elements that can be used to pro-duce a more complex output beam than is possible with straight-ruled gratings. They are typically formed by interfering two coherent beams on the surface of the underlying optic. The surface is covered with photo-sensitive material, so the interference is recorded as areas of exposed and unexposed fringes. The piece is then etched to turn the interference photograph into permanent features on the surface of the optic. Subsequent illumination by one of the 2 reference beams produces a diffracted beam in the form and direction of the second beam. The result is a diffraction grating. MACOS currently sup-ports reflective HOEs only.

MACOS HOEs can be defined on any surface shape. Enter surface parameters to define the underlying element, just as you would to define the shape of a regular mirror. The HOE grating is then defined using additional element-specific parameters. These parame-ters set up the original HOE interference geometry, so the grating on any point of the surface can be computed from the intended reference beams. The reference beams are rotated and translated appropriately if the element is perturbed using the MACOS Per-turb command. The parameters are:

      - h1HOE: The first is the input reference beam point source location in x,y,z. A colli-mated reference beam is located at infinity (1d22). Note that the reference beams have spherical (or flat) wavefronts only.

      - h2HOE: The second is the ouput reference beam point source location in x,y,z.

      - OrderHOE: The third is the diffraction order to be used to define the grating and to be ray-traced through the rest of the optical system. MACOS currently does not allow you to trace a different order than was used to define the grating.

- WaveHOE: The last is the construction or reference wavelength. The beam is traced using the source wavelength (Wavelen) which can be different.

The following example shows the new dialog used to specify a HOE on a parabolic sur-face. The complete prescription, with a collimated source illuminating the HOE at nor-mal incidence, is provided in Appendix A.2. Figure 20 shows the output beam for both the diffracted beam and the reflected beam. Both will be present in an actual system, illustrating one use of the HOE: a means for splitting light off the same surface.

Diffracted beam	Reflected beam

FIGURE 20 Holographic optical element (HOE), showing diffracted and reflected beams.

Enter Element	1 Data:

Enter element name (no blanks) [Elt]: HOEonMirror

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :HOE

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic

Enter element vertex location (x,y,z) [no default]: 0,0,0

Enter element rotation point (x,y,z) [0.,0.,0.]:

Enter element principal axis (x,y,z) [0.,0.,-1.00000]: Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:3 Enter element eccentricity [0.]:1

Do you want an aperture on this element? [NO]: Enter number of obscurations [0]:

Enter Fresnel propagation distance [3.00000]:1d22

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric

Do you wish to use element-based coordinates?: [NO]: no

Enter element index of refraction [1.00000]:1

Enter element extinction coefficient [1.000000E+22]:1d22

Element Types

Enter input reference point (x,y,z) [no default]: 0,0,-1d22	h1HOE

Enter output reference point (x,y,z) [no default]: 1,0,-3	h2HOE

Enter diffraction order [1.00000]: 1	OrderHOE

Enter reference wavelength [1.000000E-07]:6e-7	WaveHOE

This dialog produced the following prescription data:

| iElt= EltName= Element= Surface= | 1 HOEonMirror HOE Conic |  |
| --- | --- | --- |
| KrElt= | -6.000000000D+00 |  |
| KcElt= | -1.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 1.000000000D+22 |  |
| h1HOE= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+22 |
| h2HOE= | 1.000000000D+00 | 0.000000000D+00 -3.000000000D+00 |
| OrderHOE= | 1.000000000D+00 |  |
| WaveHOE= | 6.000000000D-07 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 1.000000000D+22 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

#### Reference Surfaces or Dummy Elements

Reference surfaces are “dummy” elements, with no direct effect on the light. They are used in MACOS to define the start and stop points of diffraction propagations. They are also used to provide a screen on which to project the beam for viewing using ray-trace or diffraction commands. They have many other uses as well.

As discussed in Section , reference surfaces can be aligned with the nominally spherical (or flat) wavefront of a particular beam. When done correctly, the reference surface then defines the nominal reference wavefront, or the wavefront the beam would have if it were perfect. The OPD of the actual beam wavefront on the surface (Section 5.2.5) then represents the deviation from perfection of the beam wavefront. These differences drive the MACOS diffraction propagators. This approach greatly reduces the sampling den-sity required to avoid undersampling a wavefront for diffraction (ref. Siegman and Szicklas).

MACOS provides functions that help assure that reference surfaces used for diffraction calculations are correctly set up. These are:

      - ORS: Optimizes a reference surface to provide best sampling of a beam (Section 6.3.1).

      - SRS: Slaves one reference surface to another to properly set up a near-field diffrac-tion propagation (Section 6.3.2).

Figure 21 illustrates a typical scenario, with a reference surface inserted in the focused beam of a lens. The reference surface has its center located at the focus of the lens, and so is aligned with the ideal (spherical) wavefront. The Cassegrain example (see Appen-dix A.1) provides another example; indeed all of the examples in the Appendix use ref-erence surfaces.

Reference Surface

fElt

FIGURE 21 Reference surface correctly aligned with wavefront.

Enter element name (no blanks) [Elt]: ref_surf	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Reference	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,1	psiElt Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:5.4	fElt

Enter element eccentricity [0.]:0	eElt

Do you want an aperture on this element? [NO]: no

Enter number of obscurations [0]:0

Enter Fresnel propagation distance [5.40000]:5.4	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]: no

This dialog produced the following prescription data:

iElt=	3

EltName= ref_surf Element= Reference Surface= Conic

KrElt= -5.400000000D+00 KcElt=	0.000000000D+00

psiElt= 0.000000000D+00 0.000000000D+00 1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 RptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 IndRef= 1.000000000D+00

Extinc= 0.000000000D+00

nObs=	0

ApType=	None

zElt=	5.400000000D+00

PropType=	Geometric

nECoord=	-6

Element Types

#### Focal Planes

The focal plane is a reference element, but one that always has a flat surface. The only difference between MACOS FocalPlane and Reference elements is in the way that linear models are computed for them. Use focal planes to end a prescription that is to be used for linear models! As a a reference element, a focal plane element has no direct optical effect. Focal planes are commonly used to terminate far-field diffraction propa-gations.

Enter element name (no blanks) [Elt]: focal_plane	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :FocalPlane	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Flat	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,2	VptElt

Enter element rotation point (x,y,z) [0.,0.,2.00000]:0,0,2	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,1	psiElt Do you want an aperture on this element? [NO]: no

Enter number of obscurations [0]:0

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

This dialog produced the following prescription data:

iElt=	5

EltName= focal_plane Element= FocalPlane Surface= Flat

KrElt= -1.000000000D+22 KcElt=	0.000000000D+00

psiElt= 0.000000000D+00 0.000000000D+00 1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 2.000000000D+00 RptElt= 0.000000000D+00 0.000000000D+00 2.000000000D+00 IndRef= 1.000000000D+00

Extinc= 0.000000000D+00

nObs=	0

ApType=	None

zElt=	1.000000000D+22

PropType=	Geometric nECoord=	-6

#### Return Surfaces

The return surface is another element which is used for calculations. It has no effect on wavefront propagation. Specifically, two return surfaces in sequence are used to find the position of the exit pupil of the system in the FEX command. This discussed more thor-oughly in Section 5.3.

Enter element name (no blanks) [Elt]: return	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Return	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,1	VptElt

Enter element rotation point (x,y,z) [0.,0.,1.00000]:0,0,1	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,1	psiElt Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:5	fElt

Enter element eccentricity [0.]:0	eElt

Do you want an aperture on this element? [NO]: no

Enter number of obscurations [0]:0

Enter Fresnel propagation distance [5.00000]:5	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]: no

This dialog produced the following prescription data:

iElt=  1

EltName= return Element= Return Surface= Conic

KrElt= -5.000000000D+00 KcElt= 0.000000000D+00

psiElt= 0.000000000D+00 0.000000000D+00 1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 1.000000000D+00 RptElt= 0.000000000D+00 0.000000000D+00 1.000000000D+00 IndRef= 1.000000000D+00

Extinc= 0.000000000D+00

nObs=	0

ApType=	None

zElt=	5.000000000D+00

PropType=	Geometric nECoord=	-6

Element Types

#### Hex and Pie Segmented Mirrors

Segmented systems (e.g. telescopes with segmented primary mirrors) are defined at the light source using GridType=hex or GridType=pie. This section describes entering the segmented surfaces. Figures 22 and 23 show the geometry for a segmented pie-shaped aperture and a large array of hexagonal segments, respectively. In both cases, the pannel coordinates are given by three variables (X,L,R). The segments must be entered in as elements in the same order they are listed in SegCoord. Segmented surfaces also require

      - nSeg: number of segments

      - gap: spacing between segments

      - width: width of segments (flat to flat)

R

L

S5

S4

S6

-2

S3

S1

X

S7

-2

S2

-2

-

FIGURE 22 Hexagonal coordinates X,L,R for a pie-shaped segmented mirror.

More than 1 segmented mirror can be used in a single prescription. In the case of 2 seg-mented mirrors, the segments are required to be lined up, so that the segmentation defined at the source is valid for both beams.

An example of a segment mirror .in-file (seg.in) is in Appendix A.5. The panels are numbered in the same order as they are in the Figure 23.

Input data:

+	+

| Source ray grid types:	|

|	Circular	Square	Hex	|

|	Pie	Flower	|

+	+

Enter source grid type [Circular]: Pie	GridType

Enter aperture diameter or angle [1.00000]:3.65	Aperture

Enter number of grid points [128]:31	nGridPts

22,22,22

|  |  |  |  |  | 67 |  | 66 |  | 65 |  | 64 |  | 63 |  | 62 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | -5,5,10 |  | -3,6,9 |  | -1,7,8 |  | 1,8,7 |  | 3,9,6 |  | 5,10,5 |  |  |  |  |
|  |  |  |  | 68 |  | 42 |  | 41 |  | 40 |  | 39 |  | 38 |  | 91 |  |  |  |
|  |  |  |  | -6,3,9 |  | -4,4,8 |  | -2,5,7 |  | 0,6,6 |  | 2,7,5 |  | 4,8,4 |  | 6,9,3 |  |  |  |
|  |  |  | 69 |  | 43 |  | 23 |  | 22 |  | 21 |  | 20 |  | 61 |  | 90 |  |  |
|  |  |  | -7,1,8 |  | -5,2,7 |  | -3,3,6 |  | -1,4,5 |  | 1,5,4 |  | 3,6,3 |  | 5,7,2 |  | 7,8,1 |  |  |
|  |  | 70 |  | 44 |  | 24 |  | 10 |  | 9 |  | 8 |  | 37 |  | 60 |  | 89 |  |
|  |  | -8,-1,7 |  | -6,0,6 |  | -4,1,5 |  | -2,2,4 |  | 0,3,3 |  | 2,4,2 |  | 4,5,1 |  | 6,6,0 |  | 8,7,-1 |  |
| 71 |  |  | 45 |  | 25 |  | 11 |  | 3 |  | 2 |  | 19 |  | 36 |  | 59 |  | 88 |
| -9,-3,6 |  | -7,-2,5 |  | -5,-1,4 |  | -3,0,3 |  | -1,1,2 1,2,1 |  |  |  | 3,3,0 |  | 5,4,-1 |  | 7,5,-2 |  | 9,6,-3 |  |
| 72 | 46 |  | 26 |  | 12 |  | 4 |  | 1 |  | 7 |  | 18 |  | 35 |  | 58 |  | 87 |
| -10,-5,5 -8,-4,4 |  |  | -6,-3,3 |  | -4,-2,2 |  | -2,-1,1 |  | 0,0,0 |  | 2,1,-1 |  | 4,2,-2 |  | 6,3,-3 |  | 8,4,-4 |  | 10,5,-5 |
| 73 |  | 47 |  | 27 |  | 13 |  | 5 |  | 6 |  | 17 |  | 34 |  | 57 |  | 86 |  |
| -9,-6,3 |  | -7,-5,2 |  | -5,-4,1 |  | -3,-3,0 |  | -1,-2,-1 |  | 1,-1,-2 |  | 3,0,-3 |  | 5,1,-4 |  | 7,2,-5 |  | 9,3,-6 |  |
| 74 |  |  | 48 |  | 28 |  | 14 |  | 15 |  | 16 |  | 33 |  | 56 |  | 85 |  |  |
| -8,-7,1 |  |  | -6,-6,0 |  | -4,-5,-1 |  | -2,-4,-2 |  | 0,-3,-3 |  | 2,-2,-4 |  | 4,-1,-5 |  | 6,0,-6 |  | 8,1,-7 |  |  |
| 75 |  |  |  | 49 |  | 29 |  | 30 |  | 31 |  | 32 |  | 55 |  | 84 |  |  |  |
| -7,-8,-1 |  |  |  | -5,-7,-2 |  | -3,-6,-3 |  | -1,-5,-4 |  | 1,-4,-5 |  | 3,-3,-6 |  | 5,-2,-7 |  | 7,-1,-8 |  |  |  |
| 76 |  |  |  |  | 50 |  | 51 |  | 52 |  | 53 |  | 54 |  | 83 |  |  |  |  |
| -6,-9,-3 |  |  |  |  | -4,-8,-4 |  | -2,-7,-5 |  | 0,-6,-6 |  | 2,-5,-7 |  | 4,-4,-8 |  | 6,-3,-9 |  |  |  |  |
| 77 |  |  |  |  |  | 78 |  | 79 |  | 80 |  | 81 |  | 82 |  |  |  |  |  |
| -5,-10,-5 |  |  |  |  |  | -3,-9,-6 |  | -1,-8,-7 |  | 1,-7,-8 |  | 3,-6,-9 |  | 5,-5,-10 |  |  |  |  |  |

FIGURE 23 Hexagonal coordinates X,L,R for a large array

Enter input plane x-axis vector (x,y,z) [1.00000,0.,0.]:1,0,0	xGrid Enter input plane y-axis vector (x,y,z) [0.,1.00000,0.]:0,1,0	yGrid Enter number of segments [no default]: 7		nSeg

Enter segment width: [3.65000]:1.2	width

Enter gap between segments: [0.]:0	gap

Enter 3 segment 1 hex coords [no default]: 0,0,0	SegCoord

Enter 3 segment 2 hex coords [no default]: 1,-1,-2

Enter 3 segment 3 hex coords [no default]: 2,1,-1

Enter 3 segment 4 hex coords [no default]: 1,2,1

Enter 3 segment 5 hex coords [no default]: -1,1,2

Enter 3 segment 6 hex coords [no default]: -2,-1,1

Enter 3 segment 7 hex coords [no default]: -1,-2,-1

Enter segment plane x-axis vector (x,y,z) [1.00000,0.,0.]:1,0,0	SegXgrid

Element data:

Enter Element 1 Data:

Enter element name (no blanks): (Elt): s1	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Segment	Element

+	+

| Surface types:	|

Element Types

| ¦ | Flat | Conic | Aspheric | ¦ |
| --- | --- | --- | --- | --- |
| ¦ | Anamorphic | Zernike | Monomial | ¦ |
| ¦ | Interpolated | UserDefined |  | ¦ |

+	+

Enter surface type :Conic	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1	psiElt Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:1.46	fElt

Enter element eccentricity [0.]:1	eElt

Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [1.46000]:1.46	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

Enter element index of refraction [1.00000]:

Enter element extinction coefficient [1.000000E+22]:

| iElt= | 1 |  |
| --- | --- | --- |
| EltName= | s1 |  |
| Element= | Segment |  |
| Surface= | Conic |  |
| fElt= | 1.460000000D+00 |  |
| eElt= | 1.000000000D+00 |  |
| KrElt= | -2.920000000D+00 |  |
| KcElt= | -1.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 1.000000000D+22 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 1.460000000D+00 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

Is this correct? [YES]:

Enter Element	2 Data:

Enter element name (no blanks) [Elt]: s2	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Segment	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

| ¦ | Anamorphic | Zernike | Monomial | ¦ |
| --- | --- | --- | --- | --- |
| ¦ | Interpolated | UserDefined |  | ¦ |

+	+

Enter surface type :Conic	Surface

Enter element rotation point (x,y,z) [0.,0.,0.]:.6124997330,-1.060881274,

.2569563356

Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [1.46000]:1.46	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

Enter element index of refraction [1.00000]:

Enter element extinction coefficient [1.000000E+22]:

iElt=	2

EltName= s2 Element= Segment Surface= Conic

fElt= 1.460000000D+00 eElt= 1.000000000D+00 KrElt= -2.920000000D+00 KcElt= -1.000000000D+00

psiElt= 0.000000000D+00 0.000000000D+00 -1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 RptElt= 6.124997330D-01 -1.060881274D+00 2.569563356D-01 IndRef= 1.000000000D+00

Extinc= 1.000000000D+22

nObs=	0

ApType=	None

zElt=	1.460000000D+00

PropType=	Geometric nECoord=	-6

Is this correct? [YES]:

...

Enter Element	7 Data:

Enter element name (no blanks) [Elt]: s7	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Segment	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic	Surface

Enter element rotation point (x,y,z) [0.,0.,0.]:-.612500534,-1.060880811,

.2569563356

Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [1.46000]:1.46	zElt

Element Types

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

Enter element index of refraction [1.00000]:

Enter element extinction coefficient [1.000000E+22]:

iElt=	7

EltName= s7 Element= Segment Surface= Conic

fElt= 1.460000000D+00 eElt= 1.000000000D+00 KrElt= -2.920000000D+00 KcElt= -1.000000000D+00

psiElt= 0.000000000D+00 0.000000000D+00 -1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 RptElt= -6.125005340D-01 -1.060880811D+00 2.569563356D-01 IndRef= 1.000000000D+00

Extinc= 1.000000000D+22

nObs=	0

ApType=	None

zElt=	1.460000000D+00

PropType=	Geometric nECoord=	-6

Is this correct? [YES]:

#### Generating segmented prescriptions: SegMirMaker

Hand-editing a .in file to define every segment of a 7- or 19-segment mirror is tedious.  MACOS v4.00 ships with a separate tool, SegMirMaker, in MACOS_resources/segmirmaker/, that generates a .presc (suitable for inclusion in a .in file) and an Hx.m edge-sensor measurement matrix from a parent prescription.

SegMirMaker accepts:

A MACOS .in parent prescription.  The parent element can be any conic or FreeForm surface.

The number of rings (nRing = 1 for 7-segment, 2 for 19-segment, etc.).

The axis orientation psi, segment size or aperture diameter, inter-segment gap, and segment standoff.

3-DOF (piston/tip/tilt) or 6-DOF segments.

For FreeForm parents, SegMirMaker copies the parent's lFF, FFCoef, FF coordinate frame, grid data, and grid frame verbatim into every segment, so all segments share the parent's underlying FreeForm shape.  Each segment's Mon slot is left empty (MonZernCoef = 0) but with lMon = L2 (half the segment width); the user can later add per-segment figure error via the Mon Zernike coefficients without re-running SegMirMaker.

SegMirMaker is the modernized successor to the SMPGe ('Segmented Mirror Prescription Generator') VAX Fortran tool from 1992; the original is preserved in MACOS_resources/segmirmaker/Archive/SMPGe.for.  See MACOS_resources/segmirmaker/README.md for full usage.

#### Flower Segmented Mirrors

Flower segmented mirrors consist of a central segment surrounded by “petal” segments. As such, they are like the “pie” segmented mirrors of the previous section, except flower segmentation is not limited to a hexagonal geometry. Flower segmentation is defined at the light source using GridType=Flower. Two other parameters are used to define the segmentation:

      - nPetals: number of petals surrounding the central segment

      - radCtr: radius of the largest inscribed circle defining the size of the center segment

Only 1 segment coordinate is needed per segment, to specify its location in sequence around the central segment, with the 0th segment being the center segment, the 1st seg-ment having one edge along the SegXgrid axis and the other along a line rotated 360°/

nPetals towards the SegYgrid axis, and the 2nd and subsequent segments located in the same sequence around the center segment. See Fig. 24 for an illustration.

SegYgrid

SegXgrid

GridType=Flower

FIGURE 24Flower segmentation, showing segment coordinates

#### Reflective Non-Sequential Surfaces

In most optical systems, all rays strike one element before passing on to the next ele-ment. There are exceptions, such as corner cube reflectors, where some rays will strike an element before others. These are modeled in MACOS using non-sequential surfaces. There is no limit on the number of non-sequential elements and the order that the non-sequential elements are entered into MACOS does not matter. However, there must be sequential surfaces before and after the group of non-sequential surfaces. For the pur-pose of the following discussion, we call them the first and second surfaces, respec-tively. After the ray passes the first sequential surface, MACOS solves for the next surface that the ray strikes which can be any of the group of non-sequential surfaces or the second sequential surface. If the ray hits a non-sequential surface, the procedure continues until the second sequential surface is encountered. After the second sequential surface is encountered the ray trace proceeds normally. Figure 25 shows the corner cube reflector example. The .in-file is in Appendix A.3

NOTE: Geometric propagation only is supported through non-squential elements.

Ray B

Ray A

Face1

Face2

FIGURE 25 Non-sequential surface: Ray A has Face2 as first element, Ray B’s first element is Face1.

Corner cube example

Enter element name (no blanks) [Elt]: Face1	EltName

+	+

| ¦ Element types: |  | ¦ |
| --- | --- | --- |
| ¦ Reflector | NSReflector | Segment ¦ |
| ¦ Refractor | NSRefractor | LensArray ¦ |
| ¦ FocalPlane | Reference | Return ¦ |

Element Types

|	HOE	Grating	Obscuring |

+	+

Enter element type :NSReflector	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Flat	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:-.7071,0,-.7071 psiElt Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

Enter element index of refraction [1.00000]:1d0 Enter element extinction coefficient [1.000000E+22]:

Enter element name (no blanks) [Elt]: Face2	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :NSReflector	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Flat	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:.7071,0,-.7071  psiElt Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

| ¦ Geometric | GeomUpdate | FarField | ¦ |
| --- | --- | --- | --- |
| ¦ NFSpherical | NFS1surf |  | ¦ |
| ¦ NFPlane | NFP1surf |  | ¦ |

| ¦ | NF1 | NF2 | ¦ |
| --- | --- | --- | --- |
| ¦ | SpatialFilter | SF1surf | ¦ |

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

Enter element index of refraction [1.00000]:1d0 Enter element extinction coefficient [1.000000E+22]:

#### Refractive Non-Sequential Surfaces

Refractive non-sequential surfaces are very similiar to reflective non-sequential surface (see Section 4.4.13). An example illustrating their application is a Luneberg Lens Antenna is provided in Appendix A.3. The Luneberg Lens is a stepped-gradient index lens consisting of concentric spherical shells, which grow denser as they decrease in radius. A point source on the surface produces rays that follow nearly elliptical trajecto-ries, emerging on the other side of the lens in a collimated beam. Using multiple point sources distributed on the surface, the Luneberg Lens can provide instantaneous, solid-state beam coverage over nearly 4 steradians.

### Element Surface Types

#### Flat Surfaces

Flat surfaces are completely defined by a point (the “vertex” VptElt) on the surface, and the unit vector normal to the surface at that point (psiElt). Technically, the vertex of a plane or sphere is undefined; the user is free to pick any point on the surface as the vertex. In MACOS, a flat surface is a spherical surface with an infinite radius of curva-ture, KrElt=1d22. See Section 4.4.1 for an example of entering flat surface data.

#### Conic Surfaces

Conic surfaces – really conicoids of revolution – include the most common optical shapes: spheres, ellipsoids, paraboloids, hyperboloids. Flat surfaces are spheroids with an infinite radius of curvature. MACOS specifies the shape of a conic surface using two surface-specific parameters: the conic constant KcElt and the “radius,” KrElt. Table 7 lists the common names associated with special values of the conic constant.

TABLE 7. Conic coefficients

| Conic coefficient | Conic section |
| --- | --- |
| KcElt<-1 | hyperboloid |
| KcElt=-1 | paraboloid |
| -1<KcElt<0 | ellipsoid with major axis on the principal axis (also known as prolate spheroid), psiElt |
| KcElt=0 | sphere |
| KcElt>0 | oblate ellipsoid |

MACOS also accepts input of conic parameters in terms of the geometric focal length (which is not the same as optical focal length of a spherical mirror) and eccentricity which are denoted fElt and eElt, respectively. Eccentricity and focal length are related to the conic constant and radius by:

–KcElt

eElt =

(4.1)

Element Surface Types

fElt =

----------–---K----r---E----l--t----------

(4.2)

1 + –KcElt

The “sag equation” for a conic surface, which is to say, the equation which defines all points on the surface, is:

Zh =

h

-----------------------------------------------------------------------------------------------

(4.3)

KrElt +  KrElt2 – 1 + KcElt h2

Here h is the radial distance to the projection of the point of incidence onto a plane per-pendicular to the principal axis and including the surface vertex; and Z is the height of the surface above that plane (Figure 26).

psiElt

Point of incidence vector

Optical surface

Z(h)

h

FIGURE 26 Conic mirror parameters

Note that the vertex of a conic refers to the geometric vertex except in the case of an oblate ellipsoid. For an oblate ellipsoid (KcElt>0), the “vertex” is taken as the point where the minor axis intersects the surface.

See Section 4.4.2 for an example of entering conic surface data.

#### Generalized (10th-Order) Aspheric Surfaces

Generalized aspheric surfaces use a conic surface as a base, and then impose a deviation on it, defined by additional, higher order, symmetric terms in radius h. In MACOS, gen-eralized aspheric surfaces take the form

h2	4	6	8	10

Zh =

----------------------------------------------------------------------------------------------- + Ah + Bh + Ch + Dh

(4.1)

KrElt +  KrElt2 – 1 + KcElt h2

where h is the radial pupil coordinate (the magnitude projection of the of the point of incidence vector onto the plane perpendicular to psiElt as shown in Figure 29), KrElt is the radius of curvature of the element, KcElt is the conic constant, and A, B, C, D are the aspheric coefficients, AsphCoef(i), with AsphCoef(1) = A, AsphCoef(2) = B, AsphCoef(3) = C, and AsphCoef(4) = D.

Note: The A,B,C,D parameters are used by optical design codes as well, except that the signs of the coefficients may differ in some cases. The sign of the coefficients in some codes is a function of the direction of the incident ray. MACOS references surfaces to a global coordinate system and does not assume a particular direction of the beam in defining the optical train. In MACOS the coefficients are positive in the direction of the element psiElt vector.

This example shows how to enter a higher order aspheric reflector.

Enter element name (no blanks) [Elt]: gen_asphere	EltType

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Reflector	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Aspheric	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt

Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,1	psiElt

Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:2

Enter Conic Constant [0.]:-1	KcElt

Enter Radius [-1.000000E+22]:5	KrElt

Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [2.50000]:5	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

Enter element index of refraction [1.00000]:

Enter element extinction coefficient [1.000000E+22]:

Enter aspheric coefficients A,B,C,D [0,0,0,0]:1e-9,2e-19,3e-17,4e-20	AsphCoef

This dialog produced the following prescription data:

| iElt= EltName= Element= Surface= | 1 gen_asphere Reflector Aspheric |  |  |
| --- | --- | --- | --- |
| KrElt= | -5.000000000D+00 |  |  |
| KcElt= | -1.000000000D+00 |  |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 | 1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 | 0.000000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 | 0.000000000D+00 |
| IndRef= | 1.000000000D+00 |  |  |
| Extinc= | 1.000000000D+22 |  |  |
| AsphCoef= | 1.000000000D-09 | 2.000000000D-19 | 3.000000000D-17 4.00000000D-20 |
| nObs= | 0 |  |  |
| ApType= | None |  |  |
| zElt= | 5.000000000D+00 |  |  |
| PropType= | Geometric |  |  |
| nECoord= | -6 |  |  |

#### Zernike Surfaces

Zernike surfaces add a deformation—described by a Zernike polynomial—to the sur-face of a conicoid. The deformation is entered as the “excess sag” of the surface (not the wavefront error). MACOS allows up to the first 45 Zernike terms to be added to a coni-coid base surface shape. The Zernike polynomial set that is used is that defined by Mal-

Element Surface Types

acara in his book, “Optical Shop Testing.” The ordering of the polynomials used in MACOS, and well as two other popular orderings, are shown in Table 8. In the next release of MACOS, the user will be able to select from these three orderings, as well as only enter the first 36 terms if so desired (for backwards compatibility with previous versions of MACOS).

TABLE 8. Zernike polynomials

| MACOS (Malacara) | Code V (Noll) | FRINGE (Fringe) | Polynomial |
| --- | --- | --- | --- |
| 1 | 1 | 1 | 1 |
| 2 | 3 | 3 | R sin(A) |
| 3 | 2 | 2 | R cos(A) |
| 4 | 6 | 6 | R2 sin(2A) |
| 5 | 5 | 4 | 2R2-1 |
| 6 | 4 | 5 | R2 cos(2A) |
| 7 | 10 | 11 | R3 sin(3A) |
| 8 | 9 | 8 | (3R3-2R) sin(A) |
| 9 | 8 | 7 | (3R3-2R) cos(A) |
| 10 | 7 | 10 | R3 cos(3A) |
| 11 | 15 | 18 | R4sin(4A) |
| 12 | 14 | 13 | (4R4-3R2) sin(2A) |
| 13 | 13 | 9 | 6R4-6R2+1 |
| 14 | 12 | 12 | (4R4-3R2) cos(2A) |
| 15 | 11 | 17 | R4 cos(4A) |
| 16 | 21 | 27 | R5 sin(5A) |
| 17 | 20 | 20 | (5R5-4R3) sin(3A) |
| 18 | 19 | 15 | (10R5-12R3+3R) sin(A) |
| 19 | 18 | 14 | (10R5-12R3+3R) cos(A) |
| 20 | 17 | 19 | (5R5-4R3) cos(3A) |
| 21 | 16 | 26 | R5 cos(5A) |
| 22 | 28 |  | R6 sin(6A) |
| 23 | 27 | 29 | (6R6-5R4) sin(4A) |
| 24 | 26 | 22 | (15R6-20R4+6R2) sin(2A) |
| 25 | 25 | 16 | 20R6-30R4+12R2+1 |
| 26 | 24 | 21 | (15R6-20R4+6R2) cos(2A) |
| 27 | 23 | 28 | (6R6-5R4) cos(4A) |
| 28 | 22 |  | R6 cos(6A) |
| 29 | 36 |  | R7 sin(7A) |
| 30 | 35 |  | (7R7-6R5) sin(5A) |
| 31 | 34 | 31 | (21R7-30R5+10R3) sin(3A) |
| 32 | 33 | 24 | (35R7-60R5+30R3-4R) sin(A) |
| 33 | 32 | 23 | (35R7-60R5+30R3-4R) cos(A) |

TABLE 8. Zernike polynomials

| MACOS (Malacara) | Code V (Noll) | FRINGE (Fringe) | Polynomial |
| --- | --- | --- | --- |
| 34 | 31 | 30 | (21R7-30R5+10R3) cos(3A) |
| 35 | 30 |  | (7R7-6R5) cos(5A) |
| 36 | 29 |  | R7 cos(7A) |
| 37 | 45 |  | R8 sin(8A) |
| 38 | 44 |  | (8R8-7R6) sin(6A) |
| 39 | 43 |  | (28R8-42R6+15R4) sin(4A) |
| 40 | 42 | 33 | (56R8-105R6+60R4-10R2) sin(2A) |
| 41 | 41 | 25 | 70R8-140R6+90R4-20R2+1 |
| 42 | 40 | 32 | (56R8-105R6+60R4-10R2) cos(2A) |
| 43 | 39 |  | (28R8-42R6+15R4) cos(4A) |
| 44 | 38 |  | (8R8-7R6) cos(6A) |
| 45 | 37 |  | R8 cos(8A) |

KcElt and KrElt (or eElt and fElt) are the parameters that describe the conicoid. The deviations occur only in the z-direction of a special coordinate frame. The variable pMon is the center of the coordinate frame. The variables xMon, yMon, and zMon define the spe-cial coordinate frame as shown in Figure 27.The variable lMon is the semi-diameter of the circle to which the Zernike deviations are scaled (remember that Zernikes are defined on a unit circle).

z Zernike deviations

psiElt

lMon

occur only in z direction

zMon

Top View

xMon

mirror segment

Exaggerated side view with coordinate description

FIGURE 27 Specialized coordinate frame

Zernike polynomials, listed in Table 8, are orthogonal over a unit circle. The polar coor-dinates (R, A) are related to the coordinates of the projection of the point of incidence vector onto the plane perpendicular defined by xMon and yMon (X, Y) by X=R lMon cos(A) and Y=R lMon sin(A). The more common Seidel aberrations can be calculated from the Zernike coefficients (ref. Malacara).An example using a Zernike surface to simulate deformations on the primary mirror of a small telescope is provided in Appen-dix A.7. The following dialog illustrates the responses the NEW command expects in entering a Zernike reflector:

Enter element name (no blanks) [Elt]: zern_surface	EltName

+	+

| ¦ Element types: |  | ¦ |
| --- | --- | --- |
| ¦ Reflector | NSReflector | Segment ¦ |
| ¦ Refractor | NSRefractor | LensArray ¦ |
| ¦ FocalPlane | Reference | Return ¦ |

Element Surface Types

|	HOE	Grating	Obscuring |

+	+

Enter element type :Reflector	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Zernike	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,0	VptElt

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1	psiElt Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:2

Enter Conic Constant [0.]:0	KcElt

Enter Radius [-1.000000E+22]:1d22	KrElt

Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

Enter element index of refraction [1.00000]:1

Enter element extinction coefficient [1.000000E+22]:

Enter Zernike coefficients 1-6 [0.,0.,0.,0.,0.,0.]:.0001,0,.00002,0,0, .00001

ZernCoef

Enter Zernike coefficients 7-12 [0.,0.,0.,0.,0.,0.]:0,0,.000001,.00002,0,0 Zern-Coef

Enter Zernike coefficients 13-18 [0.,0.,0.,0.,0.,0.]:0,.000003,.000001,0,0, .000002

ZernCoef

EnterZernikecoefficients19-24[0.,0.,0.,0.,0.,0.]:.0000005,.0000001,0,0,.000002,.00004

ZernCoef

Enter Zernike coefficients 25-30 [0.,0.,0.,0.,0.,0.]:.0000001,0,0,0, .000001,0

ZernCoef

Enter Zernike coefficients 31-36 [0.,0.,0.,0.,0.,0.]:0,0,0,0,0,0	ZernCoef

Enter Zernike coefficients 37-42 [0.,0.,0.,0.,0.,0.]:0,0,0,0,0,0	ZernCoef

Enter Zernike coefficients 43-45 [0.,0.,0.]:0,0,0	ZernCoef Enter surface reference point location (x,y,z) [0.,0.,0.]:0,0,0		pMon Enter surface x-axis unit vector (x,y,z) [-1.00000,0.,0.]:1,0,0		xMon Enter surface y-axis unit vector (x,y,z) [0.,-1.00000,0.]:0,1,0		yMon Enter surface z-axis unit vector (x,y,z) [0.,0.,-1.00000]:0,0,1		zMon Orthoganalized zMon= 0. 0.  1.0000000000000

Enter scale length [1.00000]:1	lMon

This dialog produced the following prescription data:

iElt=	1

EltName= zern_surface Element= Reflector Surface= Zernike

KrElt= -1.000000000D+22 KcElt=	0.000000000D+00

psiElt=	0.000000000D+00 0.000000000D+00 -1.000000000D+00 VptElt=	0.000000000D+00 0.000000000D+00 0.000000000D+00

| RptElt= | 0.000000000D+00 0.000000000D+00 | 0.000000000D+00 |
| --- | --- | --- |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 1.000000000D+22 |  |
| ZernCoef= | 1.000000000D-04 0.000000000D+00 | 2.000000000D-05 0.000000000D+00 |

0.000000000D+00 1.000000000D-05

0.000000000D+00 0.000000000D+00 1.000000000D-06 2.000000000D-05

0.000000000D+00 0.000000000D+00

0.000000000D+00 3.000000000D-06 1.000000000D-06 0.000000000D+00

0.000000000D+00 2.000000000D-06

5.000000000D-07 1.000000000D-07 0.000000000D+00 0.000000000D+00

2.000000000D-06 4.000000000D-05

1.000000000D-07 0.000000000D+00 0.000000000D+00 0.000000000D+00

1.000000000D-06 0.000000000D+00

0.000000000D+00 0.000000000D+00 0.000000000D+00 0.000000000D+00

0.000000000D+00 0.000000000D+00

0.000000000D+00 0.000000000D+00 0.000000000D+00 0.000000000D+00

0.000000000D+00 0.000000000D+00

|  | 0.000000000D+00 0.000000000D+00 | 0.000000000D+00 |
| --- | --- | --- |
| pMon= | 0.000000000D+00 0.000000000D+00 | 0.000000000D+00 |
| xMon= | 1.000000000D+00 0.000000000D+00 | 0.000000000D+00 |
| yMon= | 0.000000000D+00 1.000000000D+00 | 0.000000000D+00 |
| zMon= | 0.000000000D+00 0.000000000D+00 | 1.000000000D+00 |
| lMon= | 1.000000000D+00 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 1.000000000D+22 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

#### Sparse Zernike input

In addition to the dense MonCoef form (120 coefficients in Cartesian monomial order), MACOS reads Zernike coefficients in a sparse 'modes + coefs' form:

MonZernType= BornWolf
    nMonZernCoef= 4
    MonZernModes= 4  9  16  25
    MonZernCoef= -4.819E+00  4.001E+00  1.197E+00  2.269E-03

MonZernType selects the Zernike ordering and normalization.  Supported types: ANSI, BornWolf, Fringe, NormANSI, NormBornWolf, NormFringe, NormNoll, Noll, NormAnnularNoll, NormHex, ExtFringe.  The Norm* variants use RMS normalization; the unprefixed variants are unnormalized.

Conversion to the internal monomial form (MonCoef) happens automatically at trace time.  The same scheme applies to FF Zernikes: FFZernType, nFFZernCoef, FFZernModes, FFZernCoef.  Modes are 1-indexed.

#### Monomial Surfaces

MACOS has a second polynomial-based deformed-surface type, evaluated as a sum of Cartesian monomials. The generating function is:

(4.1)

The user must specify 120 coefficients in the prescription file.

#### Toric Surfaces

Toric surfaces are surfaces that have different shapes in some specified orthogonal x-and y-axes.Cylindrical and toroidal surfaces are two examples. Toric surfaces are distin-guished from the anamorphic surfaces by there being a non-zero conic constant in ony 1 of the axes. MACOS toric surfaces are entered using the standard KrElt and KcElt for-mat, except that two sets of these conic parameters are entered: one set for each of the x-and y-axis directions. The variable AnaCoef holds the values in the following order: Krx, Kry, Kcx. In addition, a specialized coordinate frame must be used to define the local x- and y-axes for these variables. This is the same specialized coordinate frame used in Section 4.5.4 and is defined by pMon, xMon, yMon, and zMon as shown in Fig-ure 27. An example of an (cylindrical) anamorphic surface as defined in a .in-file is shown below

#### Anamorphic Surfaces

Anamorphic surfaces are surfaces that have different (conic) shapes in some specified orthogonal x- and y-axes. Cylindrical optics and astigmatic plates are two examples. MACOS anamorphic surfaces are entered using the standard KrElt and KcElt format, except that two sets of these conic parameters are entered: one set for each of the x- and

Element Surface Types

y-axis directions. The variable AnaCoef holds the values in the following order: Krx, Kry, Kcx, Kcy. In addition, a specialized coordinate frame must be used to define the local x- and y-axes for these variables. This is the same specialized coordinate frame used in Section 4.5.4 and is defined by pMon, xMon, yMon, and zMon as shown in Fig-ure 27. An example of an (cylindrical) anamorphic surface as defined in a .in-file is shown below.

iElt=	1

EltName= ana_surf Element= Refractor Surface= Anamorph

psiElt= 0.000000000D+00 0.000000000D+00 -1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 RptElt= 0.000000000D+00 0.000000000D+00 0.000000000D+00 IndRef= 1.000000000D+00

Extinc= 1.000000000D+22

AnaCoef= -1.700000000D+02 -1.000000000D+00 -1.000000000D+22 0.000000000D+00 pMon= 0.000000000D+00 0.000000000D+00 0.000000000D+00

xMon= -1.000000000D+00 0.000000000D+00 0.000000000D+00 yMon= 0.000000000D+00 1.000000000D+00 0.000000000D+00 zMon= 0.000000000D+00 0.000000000D+00 -1.000000000D+00

nObs=	0

ApType=	None

zElt=	1.000000000D+22

PropType=	Geometric nECoord=	-6

#### User-Defined Surfaces: Deformable Mirrors

MACOS comes with 5 built-in user-defined surfaces. These surfaces use influence func-tions to model standard deformable mirror configurations for adaptive optics applica-tions. They assume a grid of push-pull actuators spread out in regular patterns across the surface of the element. Coefficients that define the extension of each actuator are read from a file (or are directly modified by a calling program, if you are using SMACOS). The mirror coefficient file is specified in the prescription (or through the MODIFY com-mand) as:

UDSrfFile=’filename’

The supplied deformable mirror types are listed in order of UDSrfType:

- 349-actuator deformable mirror. Actuators are located on a 7-mm spacing square grid. Units are inches.

- 349-actuator deformable mirror. Actuators are located on a 7-mm spacing square grid. Units are cm.

- 397-actuator deformable mirror. Actuators are located on a 7-mm spacing hex grid. Units are cm.

- 349-actuator deformable mirror with squared-out influence function. Actuators are located on a 7-mm spacing square grid. Units are cm.

- 349-actuator deformable mirror with ideal actuator influence functions (bilinear approximation). Actuators are located on a 7-mm spacing square grid. Units are inches.

- 349-actuator deformable mirror with squared-out influence function. Actuators are located on a 7-mm spacing square grid. Units are cm.

An example of the use of deformable mirrors, as well as lenslet arrays and the ATMOS

atmospheric phase disturbance command, is provided in Appendix A.4.

#### Interpolated Surfaces Using Regularly Gridded Data

MACOS provides interpolated surfaces of 2 types. The first type uses regular square gridded data, in the form of a matrix, whose rows and columns are filled with values giving the surface height deviation from a base conicoid of revolution. This form of interpolated surface, described in this section, is compatible with the high-density infor-mation typically generated by optical test interferometers. It uses linear interpolation to determine the intercept point of each ray at the optical surface, rather than the more compute-intensive 3rd-order interpolation of the more general X-Y-Z interpolated sur-face discussed in the next section.

Grid-data surfaces are defined using 3 additional parameters. These are:

      - mGridMat: the dimension of the gridded data matrix (nGridMat by nGridMat)

      - GridSrfdx: spacing of the data grids in base units

      - GridFile: name of the text file containing the nGridMat by nGridMat surface matrix

The maximum number of grid-data surfaces, and the maximum number of points for grid-data surfaces, are limited at compile time, by setting the mGridSrf and mGridMat parameters in the param.cmn file.

The surface data file GridFile should be in ASCII text, with nGridMat rows and

nGridMat columns. The data is read in using the following Fortran READ sequence:

((GridMat(i,j,jGridSrf),i=1,nGridMat),j=1,nGridMat)

The i and j indices correspond to the xMon and yMon coordinates, respectively:. The matrix is assumed centered at the pMon point. Increasing index values represent steps of size GridSrfdx along that axis. The maximum extent of the surface is ±(nGridMat/ 2)*GridSrfdx in either direction. Rays that fall outside the covered area are passed, but using only the base conic to determine direction and pathlength.

The data is read in when the prescription is read. No setup computations are required (no SINT command). Interpolation data can also be provided to SMACOS directly from a calling program that includes the appropriate common statements (see Section 9.2.3).

#### Interpolated Surfaces Using X-Y-Z Data

Interpolated surfaces using X-Y-Z data triplets allow arbitrary perturbations to be applied to a base conic-of-revolution surface, using irregularly sampled data. Compared to the gridded-data surface of the previous section, the X-Y-Z interpolated surfaces are slower to compute, using a 3rd-order, triangular, continuous surface interpolation algo-rithm. They also require large amounts of RAM.

The maximum number of interpolated data surfaces, and the maximum number of points for interpolated data surfaces, are limited at compile time, by setting the mIntSrf and mDP parameters in the param.cmn file. These settings are shown in the command window as MACOS is started up.

The interpolation data is entered from an input file, which is not read until the user exe-cutes a SINT (Surface INTerpolator) command. The SINT command reads the data files defining the surface(s) and precomputes interpolation coefficients used in the ray trace.

Obscurations and Apertures

If rays are traced prior to executing the SINT command, the interpolated surfaces are converted to conic surfaces and the interpolation data is not used.

Interpolation data can also be provided to SMACOS directly from a calling program that includes the appropriate common statements (see Section 9.2.3).

The input data file can be in either binary or text data format. A binary surface data file is expected to have the name “filnam.srfN.bin,” where filnam is the .in-file root name and N is the element number. A text surface data file is expected to have the name “filnam.srfN.txt.” If no file with the expected name is found, the user is prompted for a different file name. If none is provided, the surface is not initialized and will be converted to a conic when rays are traced.

The first line of the file contains two integer numbers: nDP, the number of nodes for the optical element; and iComputeDZ, which indicates whether slope data is provided or not. If iComputeDZ=1, slopes are to be estimated from the height data; iComputeDZ=0 indicates that slopes are provided in the data file.

The nDP subsequent lines of the file contain the data for the interpolation. The data for one node is in one row in the file. Each row has 3 or 5 real numbers, giving the x and y coordinates of the data point, the surface height at that point, and then, if iComput-eDZ=0, the x and y slopes at that point. The grid defining the data does not need to be a regular grid. The grid points need not be in any particular order in the data file.

#### FreeForm Surfaces

FreeForm (SrfType = 14, Surface = FreeForm) is a single MACOS surface type combining up to four independent contributions:

z(rho) = z_conic(rho) + z_Mon(rho) + z_FF(rho) + z_grid(rho)

where:

z_conic is the conic of revolution defined by KrElt, KcElt, and the optical axis psiElt through the vertex VptElt.

z_Mon is a Cartesian-monomial sag in the local frame (pMon, xMon, yMon, zMon), normalized by aperture lMon, with coefficients MonCoef (or sparse MonZernCoef).  Active only when lMon > 0.

z_FF is a free-form Zernike sag in an independent frame (pFF, xFF, yFF, zFF), normalized by lFF, with coefficients FFCoef (or sparse FFZernCoef).  Active only when lFF > 0.

z_grid is an interpolated sag from a regularly-gridded data array, on an independent frame (pData, xData, yData, zData) with grid spacing GridSrfdx.  Active only when nGridMat > 0.

Mon, FF, and grid contributions can be enabled in any combination; setting the corresponding sentinel to zero (or omitting it from the prescription) disables that term.

Why three independent frames.  In segmented mirrors generated by SegMirMaker (§4.4.11), the FF coefficients describe the parent surface and the FF frame is parent-aligned (so all segments share the same FF Zernike set), while the Mon frame is per-segment (face-aligned) so each segment can carry its own figure error via MonZernCoef.  The grid frame is parent-aligned so the data array represents a global perturbation map.

Independence is enforced in the surface evaluation.  The FreeFormSrf routine treats Mon, FF, and grid sags as displacements along their own z-axes (zMon, zFF, zData).  Earlier versions assumed all three shared a single z-axis, which produced incorrect intersections when, for example, the segment Mon frame was face-aligned with zMon = -psi while zFF = +psi (a 180-deg mismatch).

Example.  Conic + Mon + FF + grid:

iElt=    1
    EltName= primary
    Element= Reflector
    Surface= FreeForm
      KrElt= -1.000D+02
      KcElt= -1.0
     psiElt=  0  0  1
     VptElt=  0  0  0
     RptElt=  0  0  0
       lMon=  50.0
       pMon=  0  0  0
       xMon=  1  0  0
       yMon=  0  1  0
       zMon=  0  0  1
    MonZernType= BornWolf
    nMonZernCoef= 16
    MonZernModes= 1 3 4 5 9 10 11 12 13 19 20 21 22 23 24 25
    MonZernCoef= 1.197E+0  ...
        lFF= 100.0
        pFF=  0  0  0
        xFF=  1  0  0
        yFF=  0  1  0
        zFF=  0  0  1
    FFZernType= BornWolf
    nFFZernCoef= 16
    FFZernModes= ...
    FFZernCoef=  ...
    nGridMat= 256
    GridFile= flat.txt
    GridSrfdx= 31.25
       pData= 0  0  0
       xData= 1  0  0
       yData= 0  1  0
       zData= 0  0  1

### Obscurations and Apertures

Apertures are the areas through which light can pass. MACOS supports circular and rectangular apertures as shown in Figure 28. When a wavefront is propagated, the inten-sity that does not go through the hole is set to zero. Likewise, obscurations block light that falls within their interior. Figure 29 shows the types of obscurations implemented in MACOS: circular, rectangular, annular, and triangular. Geometrical rays are unaffected by the obscurations; the rays are traced through the optical system regardless. However, the OBScuration command will allow one to view only the unobscured rays in the spot diagram.

When entering data for obscurations and apertures, fElt, eElt, and, psiElt are required inputs. This may seem unnatural, but including these properties allows curva-ture to be given to apertures and obscurations. If an aperture or obscuration is planar, then fElt=1d22 and eElt=0. In this case, psiElt is normal to the plane. However, if the obscuration or aperture is not planar, such as an obscuration caused by mirror support pads, fElt, eElt, and, psiElt are the same as they would be for that mirror. All obscuring apertures are defined on the plane defined by psiElt and VptElt and pro-jected onto the surface.

This section has three different examples. The first example is of an element represent-ing a square aperture in front of the primary mirror.

Enter Element	1 Data:

Enter element name (no blanks) [Elt]: Sq_Ap	EltName

+	+

| Element types:	|

FIGURE 28 Examples of apertures types.

Circle	Rectangle	Ring	Triangle

Composite obscurations

FIGURE 29 Examples of obscuration types and some composite obscurations.

| ¦ | Reflector | NSReflector | Segment ¦ |
| --- | --- | --- | --- |
| ¦ | Refractor | NSRefractor | LensArray ¦ |
| ¦ | FocalPlane | Reference | Return ¦ |
| ¦ | HOE | Grating | Obscuring ¦ |

+	+

Enter element type :Obscuring	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Flat	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,-5.4	VptElt Enter element rotation point (x,y,z) [0.,0.,-5.40000]:0,0,-5.4	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1	psiElt Do you want an aperture on this element? [NO]: yes

+	+

| Aperture types:	|

|	Circular	Rectangular	|

+	+

Enter aperture type [Circular]: Rectangular	ApType Enter aperture xmin,xmax,ymin,ymax [-0.500000,0.500000,-0.500000,0.500000]: - 1.5,1.5,-1.5,1.5		ApVec

Enter number of obscurations [0]:0	nObs

Enter element x-axis vector (x,y,z) [-1.00000,0.,0.]:1,0,0	xObs

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

Obscurations and Apertures

| ¦ | Geometric | GeomUpdate | FarField | ¦ |
| --- | --- | --- | --- | --- |
| ¦ | NFSpherical | NFS1surf |  | ¦ |
| ¦ | NFPlane | NFP1surf |  | ¦ |
| ¦ | NF1 | NF2 |  | ¦ |
| ¦ | SpatialFilter | SF1surf |  | ¦ |

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

This dialog produced the following prescription data:

| iElt= EltName= Element= Surface= | 1 Sq_Ap Obscuring Flat |  |
| --- | --- | --- |
| KrElt= | -1.000000000D+22 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 -5.400000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 -5.400000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 0.000000000D+00 |  |
| nObs= | 0 |  |
| ApType= | Rectangular |  |
| ApVec= | -1.500000000D+00 | 1.500000000D+00 -1.500000000D+00 1.50000000D+00 |
| zElt= | 1.000000000D+22 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

The second example shows an obscuration consisting of four circles (see Figure 29). This is similar to what was done to model the Hubble Space Telescope’s primary mirror which was drilled in three places for rubber retaining rings. This example is a Casseg-rain telescope example with four support pads on the primary mirror. fElt and eElt are the same as the primary mirror. NGridpts was also increased so the obscurations could be seen in the focal plane. Note that the support pads are located just in front of the pri-mary mirror.

Enter Element	1 Data:

Enter element name (no blanks) [Elt]: 4CircObs	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Obscuring	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,-.01	VptElt Enter element rotation point (x,y,z) [0.,0.,-1.000000E-02]:0,0,-.01  RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1	psiElt Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:5.4	fElt

Enter element eccentricity [0.]:1	eElt

Do you want an aperture on this element? [NO]: yes

+	+

| Aperture types:	|

|	Circular	Rectangular	|

+	+

Enter aperture type [Circular]: Circular	ApType

Enter aperture radius, x, y [0.500000,0.,0.]:2,0,0	ApVec

Enter number of obscurations [0]:4	nObs

+	+

| Obscuration types:	|

|	Circle	NegCircle	|

|	Rectangle	NegRectangle	|

|	Annulus	NegAnnulus	|

|	Triangle	NegTriangle	|

|	Ellipse	NegEllipse	|

+	+

Enter obscuration type [Circle]: Circle	ObsType

Enter obscuration radius, x, y [no default]: 0.2,1,1	ObsVec

Enter obscuration type [Circle]: Circle	ObsType

Enter obscuration radius, x, y [no default]: 0.2,1,-1	ObsVec

Enter obscuration type [Circle]: Circle	ObsType

Enter obscuration radius, x, y [no default]: 0.2,-1,1	ObsVec

Enter obscuration type [Circle]: Circle	ObsType

Enter obscuration radius, x, y [no default]: 0.2,-1,-1	ObsVec

Enter element x-axis vector (x,y,z) [-1.00000,0.,0.]:1,0,0	xObs

Enter Fresnel propagation distance [5.40000]:5.4	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

This dialog produced the following prescription data:

iElt=	1

EltName= 4CircObs Element= Obscuring Surface= Conic

KrElt= -1.080000000D+01 KcElt= -1.000000000D+00

psiElt= 0.000000000D+00 0.000000000D+00 -1.000000000D+00 VptElt= 0.000000000D+00 0.000000000D+00 -1.000000000D-02 RptElt= 0.000000000D+00 0.000000000D+00 -1.000000000D-02 IndRef= 1.000000000D+00

Extinc= 0.000000000D+00

nObs=	4

ObsType=	Circle

ObsVec=	2.000000000D-01 1.000000000D+00 1.000000000D+00

ObsType=	Circle

ObsVec=	2.000000000D-01 1.000000000D+00 -1.000000000D+00

ObsType=	Circle

ObsVec=	2.000000000D-01 -1.000000000D+00 1.000000000D+00

ObsType=	Circle

ObsVec=	2.000000000D-01 -1.000000000D+00 -1.000000000D+00 xObs=	1.000000000D+00 0.000000000D+00 0.000000000D+00

ApType=	Circular

ApVec=	2.000000000D+00 0.000000000D+00 0.000000000D+00 zElt=	5.400000000D+00

PropType=	Geometric nECoord=	-6

Obscurations and Apertures

The third example shows an obscuration produced by a secondary mirror and it’s sup-port spider: two rectangles and a circle (see Figure 29). These planar obscurations obscur the primary mirror.

Enter Element	1 Data:

Enter element name (no blanks) [Elt]: SpiderObs	EltName

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Obscuring	Element

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Flat	Surface

Enter element vertex location (x,y,z) [no default]: 0,0,-5.4	VptElt Enter element rotation point (x,y,z) [0.,0.,-5.40000]:0,0,-5.4	RptElt Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1	psiElt Do you want an aperture on this element? [NO]: yes

+	+

| Aperture types:	|

|	Circular	Rectangular	|

+	+

Enter aperture type [Circular]: Circular	ApType

Enter aperture radius, x, y [0.500000,0.,0.]:2,0,0	ApVec

Enter number of obscurations [0]:3	nObs

+	+

| Obscuration types:	|

|	Circle	NegCircle	|

|	Rectangle	NegRectangle	|

|	Annulus	NegAnnulus	|

|	Triangle	NegTriangle	|

|	Ellipse	NegEllipse	|

+	+

Enter obscuration type [Circle]: Circle	ObsType

Enter obscuration radius, x, y [no default]: 0.5,0,0	ObsVec

Enter obscuration type [Circle]: Rectangle	ObsType Enter obscuration xmin, xmax, ymin, ymax [no default]: -.25,.25,-2,2  ObsVec Enter obscuration type [Circle]: Rectangle	ObsType Enter obscuration xmin, xmax, ymin, ymax [no default]: -2,2,-.25,.25  ObsVec Enter element x-axis vector (x,y,z) [-1.00000,0.,0.]:1,0,0		xObs

Enter Fresnel propagation distance [1.000000E+22]:1d22	zElt

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|  NFPlane    NFP1surf	|

|  NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric	PropType

Do you wish to use element-based coordinates?: [NO]:

This dialog produced the following prescription data:

| iElt= EltName= Element= Surface= | 1 SpiderObs Obscuring Flat |  |
| --- | --- | --- |
| KrElt= | -1.000000000D+22 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 -5.400000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 -5.400000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 0.000000000D+00 |  |
| nObs= | 3 |  |
| ObsType= | Circle |  |
| ObsVec= | 5.000000000D-01 | 0.000000000D+00 0.000000000D+00 |
| ObsType= | Rectangle |  |
| ObsVec= | -2.500000000D-01 | 2.500000000D-01 -2.000000000D+00 2.00000000D+00 |
| ObsType= | Rectangle |  |
| ObsVec= | -2.000000000D+00 | 2.000000000D+00 -2.500000000D-01 2.50000000D-01 |
| xObs= | 1.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| ApType= | Circular |  |
| ApVec= | 2.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| zElt= | 1.000000000D+22 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

### Specifying Coatings

Multi-layer coatings are not supported in this release of MACOS.

### Example Telescope

In Section 4.2.2 the source data for the Cassegrain telescope example was entered. This section finishes the example adding the necessary optical elements: the primary mirror, secondary mirror, reference surface, and focal plane. Figure 15 shows the telescope.

Table 9 lists the optical design.

TABLE 9. Cassegraing telescope design

| Element | Focal length | Eccentricity | Diameter | Distance from primary |
| --- | --- | --- | --- | --- |
| Primary Mirror | 5.4 | 1.0 | 4.0 |  |
| Secondary Mirror | 1.338854098 | 1.634183595 | 1.0 | 4.061145901 |
| Focal Plane | 1d22 | 0.0 |  | 1.5 |

Continuation of MACOS input from Section 4.2.2.

Enter Element	1 Data:

Enter element name (no blanks) [Elt]: SecMirObs

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Obscuring

+	+

| ¦ Surface types: |  |  | ¦ |
| --- | --- | --- | --- |
| ¦ Flat | Conic | Aspheric | ¦ |
| ¦ Anamorphic | Zernike | Monomial | ¦ |

Example Telescope

|	Interpolated	UserDefined	|

+	+

Enter surface type :Flat

Enter element vertex location (x,y,z) [no default]: 0,0,-5.4 Enter element rotation point (x,y,z) [0.,0.,-5.40000]:0,0,-5.4 Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1 Do you want an aperture on this element? [NO]: yes

+	+

| Aperture types:	|

|	Circular	Rectangular	|

+	+

Enter aperture type [Circular]: Circular

Enter aperture radius, x, y [0.500000,0.,0.]:2,0,0 Enter number of obscurations [0]:1

+	+

| Obscuration types:	|

|	Circle	NegCircle	|

|	Rectangle	NegRectangle	|

|	Annulus	NegAnnulus	|

|	Triangle	NegTriangle	|

|	Ellipse	NegEllipse	|

+	+

Enter obscuration type [Circle]: Circle

Enter obscuration radius, x, y [no default]: 0.5,0,0

Enter element x-axis vector (x,y,z) [-1.00000,0.,0.]:1,0,0 Enter Fresnel propagation distance [1.000000E+22]:1d22

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric

Do you wish to use element-based coordinates?: [NO]:

| iElt= EltName= Element= Surface= | 1 SecMirObs Obscuring Flat |  |
| --- | --- | --- |
| fElt= | 1.000000000D+22 |  |
| eElt= | 0.000000000D+00 |  |
| KrElt= | -1.000000000D+22 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 -5.400000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 -5.400000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 0.000000000D+00 |  |
| nObs= | 1 |  |
| ObsType= | Circle |  |
| ObsVec= | 5.000000000D-01 | 0.000000000D+00 0.000000000D+00 |
| xObs= | 1.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| ApType= | Circular |  |
| ApVec= | 2.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| zElt= | 1.000000000D+22 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

Is this correct? [YES]:

Enter Element	2 Data:

Enter element name (no blanks) [Elt]: Primary

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :Reflector

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic

Enter element vertex location (x,y,z) [no default]: 0,0,0

Enter element rotation point (x,y,z) [0.,0.,0.]:0,0,0

Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1 Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:5.4 Enter element eccentricity [0.]:1

Do you want an aperture on this element? [NO]: Enter number of obscurations [0]:

Enter Fresnel propagation distance [5.40000]:5.4

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric

Do you wish to use element-based coordinates?: [NO]: Enter element index of refraction [1.00000]:

Enter element extinction coefficient [1.000000E+22]:

| iElt= EltName= Element= Surface= | 2 Primary Reflector Conic |  |
| --- | --- | --- |
| fElt= | 5.400000000D+00 |  |
| eElt= | 1.000000000D+00 |  |
| KrElt= | -1.080000000D+01 |  |
| KcElt= | -1.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 0.000000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 1.000000000D+22 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 5.400000000D+00 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

Is this correct? [YES]:

Enter Element	3 Data:

Enter element name (no blanks) [Elt]: Secondary

+	+

Example Telescope

| ¦ Element types: |  | ¦ |
| --- | --- | --- |
| ¦ Reflector | NSReflector | Segment ¦ |
| ¦ Refractor | NSRefractor | LensArray ¦ |
| ¦ FocalPlane | Reference | Return ¦ |
| ¦ HOE | Grating | Obscuring ¦ |

+	+

Enter element type :Reflector

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic

Enter element vertex location (x,y,z) [no default]: 0,0,-4.0611459018

Enter element rotation point (x,y,z) [0.,0.,-4.06115]:0,0,-5.4 Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,-1 Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:1.3388540982 Enter element eccentricity [0.]:1.6341835953

Do you want an aperture on this element? [NO]: Enter number of obscurations [0]:

Enter Fresnel propagation distance [1.33885]:1.3388540982

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric

Do you wish to use element-based coordinates?: [NO]: Enter element index of refraction [1.00000]:

Enter element extinction coefficient [1.000000E+22]:

| iElt= EltName= Element= Surface= | 3 Secondary Reflector Conic |  |
| --- | --- | --- |
| fElt= | 1.338854098D+00 |  |
| eElt= | 1.634183595D+00 |  |
| KrElt= | -3.526787502D+00 |  |
| KcElt= | -2.670556023D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 -1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 -4.061145902D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 -5.400000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 1.000000000D+22 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 1.338854098D+00 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

Is this correct? [YES]:

Enter Element	4 Data:

Enter element name (no blanks) [Elt]: Ref_surf

+	+

| ¦ Element types: |  | ¦ |
| --- | --- | --- |
| ¦ Reflector | NSReflector | Segment ¦ |
| ¦ Refractor | NSRefractor | LensArray ¦ |
| ¦ FocalPlane | Reference | Return ¦ |
| ¦ HOE | Grating | Obscuring ¦ |

+	+

Enter element type :Reference

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Conic

Enter element vertex location (x,y,z) [no default]: 0,0,-4.0601459018 Enter element rotation point (x,y,z) [0.,0.,-4.06015]:0,0,-4.0601459018 Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,1

Enter e&f or Kc&Kr (1=e&f,2=Kc&Kr)? [1]:1

Enter element focal length [1.000000E+22]:5.5601459018 Enter element eccentricity [0.]:0

Do you want an aperture on this element? [NO]: Enter number of obscurations [0]:

Enter Fresnel propagation distance [5.56015]:5.5601459018

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: FarField

Do you wish to use element-based coordinates?: [NO]:

| iElt= EltName= Element= Surface= | 4 Ref_surf Reference Conic |  |
| --- | --- | --- |
| fElt= | 5.560145902D+00 |  |
| eElt= | 0.000000000D+00 |  |
| KrElt= | -5.560145902D+00 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 -4.060145902D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 -4.060145902D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 0.000000000D+00 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 5.560145902D+00 |  |
| PropType= | FarField |  |
| nECoord= | -6 |  |

Is this correct? [YES]:

Enter Element	5 Data:

Enter element name (no blanks) [Elt]: foc_pln

+	+

| Element types:	|

|	Reflector	NSReflector	Segment	|

|	Refractor	NSRefractor	LensArray |

|	FocalPlane	Reference	Return	|

|	HOE	Grating	Obscuring |

+	+

Enter element type :FocalPlane

Editing .In-Files

+	+

| Surface types:	|

|	Flat	Conic	Aspheric	|

|	Anamorphic	Zernike	Monomial	|

|	Interpolated	UserDefined	|

+	+

Enter surface type :Flat

Enter element vertex location (x,y,z) [no default]: 0,0,1.5 Enter element rotation point (x,y,z) [0.,0.,1.50000]:0,0,1.5 Enter element principal axis (x,y,z) [0.,0.,-1.00000]:0,0,1 Do you want an aperture on this element? [NO]:

Enter number of obscurations [0]:

Enter Fresnel propagation distance [1.000000E+22]:0

+	+

| Propagation types:	|

|	Geometric	GeomUpdate	FarField	|

|	NFSpherical	NFS1surf	|

|	NFPlane	NFP1surf	|

|	NF1	NF2	|

|	SpatialFilter SF1surf	|

+	+

Enter propagation type [Geometric]: Geometric

Do you wish to use element-based coordinates?: [NO]:

| iElt= EltName= Element= Surface= | 5 foc_pln FocalPlane Flat |  |
| --- | --- | --- |
| fElt= | 1.000000000D+22 |  |
| eElt= | 0.000000000D+00 |  |
| KrElt= | -1.000000000D+22 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 1.500000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 1.500000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 0.000000000D+00 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 0.000000000D+00 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

nOutCord=	0

Tout=

Is this correct? [YES]:

Enter xOut (x,y,z) [-1.00000,0.,0.]:1,0,0

Enter yOut (x,y,z) [0.,1.00000,0.]:0,1,0

This completes the input example. The file is now saved as Cassegrain.in within the directory in which MACOS is running. The input file is included in Appendix A.1.

### Editing .In-Files

Once a prescription has been entered and stored as a .in-file, the user can edit and reuse it for further analyses. Editing an .in-file is a matter of opening the file in the text editor

of your choice. On Unix machines this can be vi, Emacs, TextEdit, or another text edi-tor. On the Macintosh, Qued-M, MPW Editor, BBEdit, Alpha, or other text or word pro-cessors can be used.

Files can be opened in directories other than the one MACOS is running in, by using appropriate path descriptors in specifying the file name. On Unix systems, the full name takes the form: “subdirectory/subdirectory/filename.” On Macintosh systems, the full name takes the form: “:subdirectory:subdirectory:filename.”

There are some simple rules for .in-files that must be followed to avoid ambiguities in the data. These are:

- The first data block must be for the source.

- Each element’s data must be in a block starting with ‘iElt=’ followed by the num-ber of that element. All data within that data block refers to that element. The end of the data block is indicated by the next ‘iElt=’ line or the ‘nOutCord=’ line.

- Ordering within data blocks is generally not important, except that nObs must pre-ceed the description of the obscurations.

- The last element data block must be followed by the ‘nOutCord=’ specifying the number of output coordinates and the output coordinate matrix.

- Blank lines can be used to separate data for clarity.

- Comments must be preceeded by a ‘%’ sign

- Data strings can be entered in free format. Up to 16 decimals of precision will be recognized.

Data for a single element must be grouped together, but the order of the data is not important as long as the ‘iElt=’ statement is first. When a new ‘iElt=’ statement is encountered, MACOS checks to make sure all the necessary data has been provided for the previous element. If data for an element has not been provided, MACOS will do one of two things. For some elements, MACOS will provide “default” data for that element. Other data is “necessary” for an .in-file and MACOS will print a warning that the .in-file was not loaded and the name of the omitted variable. A listing of the defaults provided for variables is included in Section 4.11.

### Commands

MACOS provides the user several commands to help setup the optical prescription, to modify or perturb the optical prescription, and to list parameters of the optical system. Commands to help Some of these are described here; others are mentioned here but described elsewhere.

#### OLD and LOAD

Previously created .in-files are loaded using the OLD command. This command has one argument, the name of the input file (minus the “.in” suffix). For instance, to load the Cassegrain.in prescription:

MACOS> old Cassegrain

LOAd also work identically:

MACOS> loa Cassegrain

Commands

#### MODIFY

The MODify command allows the user to change the current optical prescription without editing the .in-file. These changes are not saved in a .in-file unless the SAVe command is used. This allows the user to perturb elements, modify the prescription, and save the results if desired. Any of the .in-file variables listed in Section 4.4.1 can be changed.

MACOS>mod

MOD: (q): h

MACOS MODify Function MODify Syntax Examples:

Flux=3

ChfRayPos(1)=2-MOD 1st element of vector ChfRayPos(:)=2,0,0-MOD all elements of vector RptElt(3,2)=3-MOD 3rd element of vector for iElt=2 RptElt(:,2)=1.5,3,0-MOD all elements of vector for iElt=2

Quit: quit Modify function.

To use the MODify command, just type MOD. The user will then be prompted for the mod-ification. If a MODification is successful, it will be echoed at the terminal. For instance to change the eccentricity of element 2 to 1.03.

MACOS>mod

MOD: (q): eElt(2)=1.03 eElt( 2) = 1.030000000D+00 KcElt( 2) = -1.060900000D+00 KrElt( 2) = -2.842000000D+00

If a MOD is not successful, it will not be echoed.

MOD: (q): eeElt(2)=1.04

MOD: (q):

There are 2 ways to enter use MODify for vectors and arrays. The first is changing one element of the vector or array at a time:

MOD: (q): VptElt(3,2)=4.2

The second is to re-enter the entire vector:

MOD: (q): VptElt(:,2)=1.0,0.0,3.0

The SHOw command can be used to display the element’s data.

#### SAVE

The SAVe command writes a new .in-file containing the current optical prescription data to disk. Parameters changed as the result of a PERturb or MODify command will be writ-ten as modified. This command takes one argument: the new .in-file name (without the “.in” suffix). If the specified file already exists, the user is asked if it should be overwrit-ten.

#### SUMMARIZE

The SUMmarize command displays a brief summary of the loaded optical system, list-ing each element and showing its type, conic constant KcElt, and radius of curvature KrElt.

#### STATUS

The STAtus command displays information about some of the current states.

MACOS>status

Status for file cass_ap3:

Current element for OPD calculations= 4 Current element for WF/Spot calculations= 5 Obscuration option for ray-trace= 0 Current WF plot type=SLICE

REGRID option= F Composed image= T

Pixel location set by chief ray option= F

#### RESET

The RESet command reinitializes some of the current states (see below) to their default values.

MACOS>reset MACOS>status

Status for file cass_ap3:

Current element for OPD calculations= 0 Current element for WF/Spot calculations= 0 Obscuration option for ray-trace= 0 Current WF plot type=GRAY

REGRID option= F Composed image= F

Pixel location set by chief ray option= F

#### SHOW

The SHOw command displays all the prescription data for a particular element. It has one argument, the element number.

### Summary of Prescription Variables

Tables 10 through 12 provide a summary of the MACOS prescription variables and their default values. When MACOS loads an .in-file, these defaults are used variables, if the user has not supplied values. When a default is used, the user is notified, e.g.:

Default used for zSource

Not all variables have defaults, and these variables must be specified by the user. If one of these variables is not given a value in the .in-file, then the .in-file is not loaded and the user is notified; for instance:

VptElt(1) must be specified by user Input file not properly loaded

The variables are shown with their index types in parentheses. These indices are:

- iObs: which obscuration in an element (can be up to 6 obscurations per element).

- iElt: number of the element in the .in-file.

- iAxis: ranges over the x,y,z coordinates in the global coordinate system.

- iPert: ranges over x,y,z rotations, x,y,z translations in the u-vector.

- iLin: ranges over the x,y,z components of the ray direction perturbation, the x,y,z components of the ray transverse aberration and the pathlength perturbation in the linear state vector x.

- iAsCoef: A, B, C, and D coefficients described in Section 4.5.3.

Summary of Prescription Variables

- iAp: either radius,x,y, or xmin,xmax,ymin,ymax depending on ApType.

- iZern: the 36 Zernike coefficients.

- iLayer: ranges over the coating layers.

All variables can be changed with the MODify command.

TABLE 10. Light source variables

| Source Variables | Defaults | Description |
| --- | --- | --- |
| Aperture | 1.0D0 | input aperture |
| BeamType | 1 (uniform) | beam intensity |
| ChfRayDir(iAxis) | NO DEFAULT | chief ray direction |
| ChfRayPos(iAxis) | (0.0D0,0.0D0,0.0D0) | chief ray location |
| CosPower |  |  |
| ExtInc(0) | 0.0D0 | extinction coefficient |
| Ex0 |  | electric field strength of source along x-axis |
| Ey0 |  | electric field strength of source along x-axis |
| Flux | 1.0D0 | total intensity received across the unobscured aperture |
| gap | 0.0D0 | spacing between segments in seg-mented apertures |
| GridType | 1 (circular) | type of input aperture |
| IndRef(0) | 1.0D0 | real part of index of refraction of input medium |
| nElt | NO DEFAULT | number of elements |
| nGridPts | IFIX(mpts/2) | number of ray grid points across the aperture |
| nSeg | 0 | number of segments |
| Obscratn | 0.0D0 | input obscuration |
| rxBeam |  | beam diameter in x-direction |
| ryBeam |  | beam diameter in y-direction |
| SegCo-ord(xyz,iElt) | NO DEFAULT | segment location in the entrance pupil |
| Wavelen | 6.0D-7 | wavelength of light |
| width | Aperture | width of segments |
| xGrid(iAxis) | NO DEFAULT | orientation of input x-axis vector |
| yGrid(iAxis) | cross product of xGrid and ChRay-Dir | orientation of input y-axis vector |
| zGrid(iAxis) |  | orientation of input z-axis vector |
| Zelt(0) |  |  |
| zSource | 1.0D22 | distance of light source from point of origin |

TABLE 11. Element variables

| Element variables | Default values | Description |
| --- | --- | --- |
| AnaCoef(iAna,iElt) |  | anamorphic coefficients |
| ApType | 1 (circle) | aperture type |
| ApVec(iAp,iElt) | NO DEFAULT | aperture definition informa-tion |
| AsphCoef(iAsCoef,iElt) | (0.0D0,0.0D0,0.0D0, 0.0D0) | asphere coefficients (4) |
| CoatIndxElt(iLayer, iElt) |  | index of coating layers |
| CoatThkElt(iLayer,iElt) |  | thickness of coating layers |
| eElt(iElt) | 0.0D0 | element eccentricity |
| EltName(iElt) | ' ' (blanks) | element name |
| Element(iElt) | NO DEFAULT | element type |
| ExtInc(iElt) | 0.0D0 | extinction coefficient |
| fElt(iElt) | 1.0D22 | element focal length |
| h1HOE(iAxis,iElt) |  |  |
| h2HOE(iAxis,iElt |  |  |
| IndRef(iElt) | 1.0D0 | real part of refactive index for element |
| KcElt(iElt) | 0.0D0 | element conic constant |
| KrElt(iElt) | -1.0D22 | element conic radius |
| LensArrayType(iElt) | 1 | type of lens in array (hex, square) |
| LensArrayWidth(iElt) | 1 | width of a single lens in the array |
| lMon(iElt) | Aperture | scale length for monomial (Zernike) element calcula-tions |
| MonCoef(iMon,iElt) |  | monomial coefficients |
| nCoatElt(iElt) |  | number of coatings |
| nECoord(iElt) | -6 | number of element coordi-nates |
| nLayer |  | number of layers in coating, maximum 20 |
| nObs(iElt) | 0 | number of obscurations |
| ObsType(iObs,iElt) | 1 (circle) | obscuration type |
| ObsVec(iAp,iObs,iElt) | (0.0D0,0.0D0,0.0D0, 0.0D0,0.0D0,0.0D0) | obscuration definition infor-mation |
| OrderHOE(iElt) |  | diffraction order of HOE to trace |
| PinHole() |  | pin hole |
| pMon(iAxis,iElt) | VptElt | reference for monomial (Zernike) surface calcula-tions |

Summary of Prescription Variables

TABLE 11. Element variables

| Element variables | Default values | Description |
| --- | --- | --- |
| PropType(iElt) | 1 (Rays Only) | type of propagation from element iElt to element iElt+1 |
| psiElt(iAxis,iElt) | NO DEFAULT | element principal axis vec-tor |
| RptElt(iAxis,iElt) | VptElt | element rotation point |
| RuleWidth(iElt) |  | grating ruling width |
| Surface(iElt) |  | surface type |
| TElt(iPert,iElt-Cord,iElt) | (1.0D0,0.0D0,0.0D0, 0.0D0,0.0D0,0.0D0) (0.0D0,1.0D0,0.0D0, 0.0D0,0.0D0,0.0D0) (0.0D0,0.0D0,0.0D0, 1.0D0,0.0D0,0.0D0) (0.0D0,0.0D0,0.0D0, 1.0D0,0.0D0,0.0D0) (0.0D0,0.0D0,0.0D0, 0.0D0,1.0D0,0.0D0) (0.0D0,0.0D0,0.0D0, 0.0D0,0.0D0,1.0D0) | element local coordinates transformation matrix |
| UDSrfCoef(iCoef,iElt) |  | user defined surface coeffi-cients |
| UDSrfType(iElt) |  | user defined surface type |
| VptElt(iAxis,iElt) | NO DEFAULT | element vertex point |
| xMon(iAxis,iElt) | xGrid | x-axis for monomial (Zernike) surface calcula-tions |
| xObs(iAxis,iElt) | NO DEFAULT | orientation of x-axis for each element |
| yMon(iAxis,iElt) | cross product of psiElt and xMon | y-axis for monomial (Zernike) surface calcula-tions |
| WaveHOE(iElt) |  | recording wavelength for HOE |
| zElt(iElt) | fElt | Fresnel propagation distance |

TABLE 11. Element variables

| Element variables | Default values | Description |
| --- | --- | --- |
| ZernCoef(iZern,iElt) | (0.0D0,0.0D0,0.0D0, | Zernike coefficients |
|  | 0.0D0,0.0D0,0.0D0) |  |
|  | (0.0D0,0.0D0,0.0D0, |  |
|  | 0.0D0,0.0D0,0.0D0) |  |
|  | (0.0D0,0.0D0,0.0D0, |  |
|  | 0.0D0,0.0D0,0.0D0) |  |
|  | (0.0D0,0.0D0,0.0D0, |  |
|  | 0.0D0,0.0D0,0.0D0) |  |
|  | (0.0D0,0.0D0,0.0D0, |  |
|  | 0.0D0,0.0D0,0.0D0) |  |
|  | (0.0D0,0.0D0,0.0D0, |  |
|  | 0.0D0,0.0D0,0.0D0) |  |
|  | (0.0D0,0.0D0,0.0D0, |  |
|  | 0.0D0,0.0D0,0.0D0) |  |
|  | (0.0D0,0.0D0,0.0D0) |  |
| zMon(iAxis,iElt) | psiElt | z-axis for monomial (Zernike) surface calcula-tions |

TABLE 12. Output variables

| Output Variables | Defaults | Description |
| --- | --- | --- |
| Tout(iLin,iPert) | NO DEFAULT | the output coordinate transformation matrix |
| nOutCord | NO DEFAULT | number of output coordinates |

## SECTION 5: Ray-Trace Analysis

MACOS ray-trace commands define a bundle of rays at the light source, and then advance each ray element-by-element through the beam train. Individual rays can be followed through the system, or the entire bundle can be traced to display OPD maps or spot diagrams at particular elements. Placement of reference surfaces using ORS and SRS is introduced. The system exit pupil can be placed using the FEX command. Using the PERturb and MODify commands, individual elements can be altered and the analy-ses repeated. The commands defined in this section include:

DRAW	MAP

RAY	SPOt

OPD	OBS

PERturb	PREad

CENter	STOP

FEX	COORDinates

FFP	PFP

POLarization	NOPolarizaton SINT

### Background

Having loaded the .in-file prescription that describes the optical system to MACOS, the user can trace rays to analyze the system. MACOS provides ray-trace commands that produce useful graphical (or file) outputs, such as SPOT and OPD. As discussed earlier, spot diagrams are graphs of the pierce points of the beam rays on a particular surface. They provide a measure of the geometric beam or image shape and are extremely useful in gauging performance. OPD maps show the wavefront profile at a particular surface. When used with an appropriate reference or return surface, OPD generates the wave-front error map of the system.

MACOS also provides commands to help setup appropriate reference surfaces. These include FEX, ORS and SRS (ORS and SRS are discussed later, in Section 6.3). Both SPOT and OPD provide means to quantify system performance, whether for design, tolerancing or simulation, for instrument-level or system-level analyses.

Detailed summaries of the paths taken by single rays can be generated using the RAY command. The optical train can be visualized using the DRAW command. These com-mands are useful in verifying design layout and correctness, as well as for single-ray performance measurements.

In addition to these commands, which generate specific data products, MACOS pro-vides commands that can be used to change the system or the beam path in specific ways. The PERturb command implements a kinematically correct single-axis rotation and translation perturbation of an element or the beam source, and can be used to alter the system according to effects of misalignments or disturbances. The CENTER and STOP commands are used to set up the correct beam path through the optical train, by enforcing the system stop. The FEX command sets up return surfaces at exit pupils for OPD evaluation and for diffraction calculations. Commands such as FFP and PFP are

used to find the field angles that place the beam at particular points on an element, and are very useful in simulating imaging systems.

Before we describe the MACOS ray-trace commands, we will summarize how MACOS computes a ray-trace and introduce some of the internal MACOS variables that store ray-trace information. These variables are directly accessible to user programs that use the SMACOS include files.

A full beam ray-trace is initiated when the user issues an OPD or SPOT command (other commands, such as BUILD, INTensity or several of the other diffraction commands also initiate a full-beam ray-trace). Along with the command, the user specifies the ele-ment where the requested data is to be generated.

MACOS keeps track of the current position of the beam in the beam train. If the starting element for the trace is the source, then MACOS generates the starting ray states at the source point, using the source prescription data. Otherwise the trace starts where it left off previously. From the starting element, the beam is traced to the specified ending ele-ment. MACOS works in 2 loops to trace the beam from the starting element to the end element. The inner loop traces a single ray past each element until it reaches the speci-fied ending element. The outer loop traces successive rays, following the order illus-trated by the MAP command, until all rays have been traced.

In MACOS, “tracing rays” means updating specific ray-state variables so that they hold values appropriate for the specified end element. These variables include:

- RayPos(1:3,iRay) stores a 3-vector giving the position of the current segment of ray iRay in global coordinates. The current segment is the one that ends at the last element specified in an OPD, SPOt, BUild command. The last element after a dif-fraction propagation command (such as INTensity) depends on the type of propa-gation used just prior to the current element.

- RayDir(1:3,iRay) stores a unit magnitude 3-vector giving the direction of the next segment of ray iRay. If the current element is a mirror, RayDir gives the direc-tion of the reflected ray. If it is a refracting surface, RayDir gives the direction of the refracted ray.

- RayL(iRay) stores a scalar giving the geometric length (not the optical path length which includes the effect of the index of refraction) of the current segment of ray iRay.

- CumRayL(iRay) stores a scalar giving the cumulative optical length (includes effects of non-unity index of refraction) of the ray iRay up to and including the cur-rent segment. It gives the optical pathlength from the source to the current element.

- RayIndex(iRay)stores the current index of refraction of the ray. These definitions are illustrated in Figure 30.

### Ray-Trace Commands

    - DRAW

The DRAW command produces a drawing of the beam train, showing the optical element surfaces and the beam propagating through them. The beam is represented by eleven rays. The elements are drawn in cross-section, using the ray-intercept points from 100 rays across the beam to define the surfaces. The drawing is in a specified (xz, yz or xy) plane, and the surfaces and rays are drawn as the projection of the onto that plane. Tilted or folded systems can sometimes produce strange results! Different colors are used to distinguish between rays and several different element types.

Ray-Trace Commands

RayL2

RayL1	RayL3

CumRayL=RayL1*RayIndex1

+RayL2*RayIndex2

+RayL3*RayIndex3

x

RayIndex2

RayPos3

z

y

MACOS>draw

FIGURE 30 Ray state definitions

The user is asked to specify the starting element, the stopping element, and the viewing plane. The following example draws the ImageDemo.in prescription (see Appendix) beam train from the source (element 0) to element 11:

Enter first element to include: [0]: 0

Enter last element to include: [13]: 11

Enter drawing plane (XZ, YZ or XY in source coords): [XZ]: xz

The result is shown in Figure 31. The system is somewhat aberrated, so the beam does not come to a clean focus! A close-up of refractive elements 6-11 is specified by chang-ing the starting element:

MACOS>draw

Enter first element to include: [0]: 6

Enter last element to include: [13]: 11

Enter drawing plane (XZ, YZ or XY in source coords): [XZ]: xz

FIGURE 31 DRAW views of ImageDemo.in

    - MAP

Single rays are specified by their “ray ID” number, iRay. This number must be specified in tracing individual rays, printing individual ray partials, or in accessing ray state data directly using the SMACOS include files.

The relative location of each ray in the pupil can be displayed using the MAP command. When there are more than 15 rays across the aperture, only a subset of the ray numbers are shown. The chief ray is always ray number 1 (iRay=1) and is not displayed by MAP. For Cassegrain.in, the MAP command produces the following:

MACOS>map

| 0 0 0 0 0 0 0 150 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 0 0 0 143 144 145 146 147 148 149 |  |  |  | 0 | 0 | 0 | 0 |
| 0 0 0 134 135 136 137 138 139 140 141 |  |  |  | 142 | 0 | 0 | 0 |
| 0 0 123 124 125 126 127 128 129 130 131 132 133 |  |  |  |  |  | 0 | 0 |
| 0 110 111 112 113 114 115 116 117 118 119 120 121 |  |  |  |  |  | 122 | 0 |
| 0 97 98 99 100 101 102 103 104 105 106 107 108 |  |  |  |  |  | 109 | 0 |

Map of rays at input aperture:

| 0 84 | 85 86 87 88 89 90 91 92 93 94 | 95 | 96 | 0 |
| --- | --- | --- | --- | --- |
| 69 70 | 71 72 73 74 75 76 77 78 79 80 | 81 | 82 | 83 |
| 0 56 | 57 58 59 60 61 62 63 64 65 66 | 67 | 68 | 0 |
| 0 43 | 44 45 46 47 48 49 50 51 52 53 | 54 | 55 | 0 |
| 0 30 | 31 32 33 34 35 36 37 38 39 40 | 41 | 42 | 0 |
| 0 0 | 19 20 21 22 23 24 25 26 27 28 | 29 | 0 | 0 |
| 0 0 | 0 10 11 12 13 14 15 16 17 18 | 0 | 0 | 0 |
| 0 0 | 0 0 3 4 5 6 7 8 9 0 | 0 | 0 | 0 |
| 0 0 | 0 0 0 0 0 2 0 0 0 0 | 0 | 0 | 0 |

MACOS>

MACOS>map

The lower left corner is the (x=0, y=0) point for the aperture coordinates defined by the ChfRayDir (=-zGrid) and xGrid vectors. The view is looking into the optical sys-tem from the source.

When using MAP with a segmented system, a map of the segments is also printed, show-ing which segment each ray hits. For SegmentDemo.in, the following map is pro-duced.

| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 684 | 686 | 688 | 690 | 692 | 694 | 696 | 698 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 645 | 647 | 649 | 651 | 653 | 655 | 657 | 659 | 661 | 663 | 0 | 0 | 0 |
| 0 | 0 | 597 | 599 | 601 | 603 | 605 | 607 | 609 | 611 | 613 | 615 | 617 | 619 | 0 | 0 |
| 0 | 544 | 546 | 548 | 550 | 552 | 554 | 556 | 558 | 560 | 562 | 564 | 566 | 568 | 570 | 0 |
| 0 | 489 | 491 | 493 | 495 | 497 | 499 | 501 | 503 | 505 | 507 | 509 | 511 | 513 | 515 | 0 |
| 0 | 431 | 433 | 435 | 437 | 439 | 441 | 443 | 445 | 447 | 449 | 451 | 453 | 455 | 457 | 0 |
| 0 | 373 | 375 | 377 | 379 | 381 | 383 | 385 | 387 | 389 | 391 | 393 | 395 | 397 | 399 | 0 |
| 0 | 313 | 315 | 317 | 319 | 321 | 323 | 325 | 327 | 329 | 331 | 333 | 335 | 337 | 339 | 0 |
| 0 | 255 | 257 | 259 | 261 | 263 | 265 | 267 | 269 | 271 | 273 | 275 | 277 | 279 | 281 | 0 |
| 0 | 197 | 199 | 201 | 203 | 205 | 207 | 209 | 211 | 213 | 215 | 217 | 219 | 221 | 223 | 0 |
| 0 | 142 | 144 | 146 | 148 | 150 | 152 | 154 | 156 | 158 | 160 | 162 | 164 | 166 | 168 | 0 |
| 0 | 0 | 93 | 95 | 97 | 99 | 101 | 103 | 105 | 107 | 109 | 111 | 113 | 115 | 0 | 0 |
| 0 | 0 | 0 | 49 | 51 | 53 | 55 | 57 | 59 | 61 | 63 | 65 | 67 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 14 | 16 | 18 | 20 | 22 | 24 | 26 | 28 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| return to continue of segments at input aperture: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 5 | 5 | 5 | 5 | 4 | 4 | 4 | 4 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 5 | 5 | 5 | 5 | 5 | 4 | 4 | 4 | 4 | 4 | 0 | 0 | 0 |
| 0 | 0 | 5 | 5 | 5 | 5 | 5 | 5 | 4 | 4 | 4 | 4 | 4 | 4 | 0 | 0 |
| 0 | 6 | 5 | 5 | 5 | 5 | 5 | 5 | 4 | 4 | 4 | 4 | 4 | 4 | 3 | 0 |
| 0 | 6 | 6 | 6 | 5 | 5 | 5 | 1 | 1 | 4 | 4 | 4 | 3 | 3 | 3 | 0 |
| 0 | 6 | 6 | 6 | 6 | 5 | 1 | 1 | 1 | 1 | 4 | 3 | 3 | 3 | 3 | 0 |
| 0 | 6 | 6 | 6 | 6 | 6 | 1 | 1 | 1 | 1 | 3 | 3 | 3 | 3 | 3 | 0 |
| 0 | 6 | 6 | 6 | 6 | 6 | 1 | 1 | 1 | 1 | 3 | 3 | 3 | 3 | 3 | 0 |
| 0 | 6 | 6 | 6 | 6 | 7 | 1 | 1 | 1 | 1 | 2 | 3 | 3 | 3 | 3 | 0 |
| 0 | 6 | 6 | 6 | 7 | 7 | 7 | 1 | 1 | 2 | 2 | 2 | 3 | 3 | 3 | 0 |
| 0 | 6 | 7 | 7 | 7 | 7 | 7 | 7 | 2 | 2 | 2 | 2 | 2 | 2 | 3 | 0 |
| 0 | 0 | 7 | 7 | 7 | 7 | 7 | 7 | 2 | 2 | 2 | 2 | 2 | 2 | 0 | 0 |
| 0 | 0 | 0 | 7 | 7 | 7 | 7 | 7 | 2 | 2 | 2 | 2 | 2 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 7 | 7 | 7 | 7 | 2 | 2 | 2 | 2 | 0 | 0 | 0 | 0 |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |

Map of rays at input aperture:

Hit Map

MACOS>

    - Undefined Rays

If a MAP command is used after a ray-trace or diffraction command such as OPD, SPOt or INTensity, some of the rays may disappear. This indicates that those rays became undefined because the rays:

      - missed an optical element

Ray-Trace Commands

- hit an optical element out of order

- hit two non-sequential surfaces simultaneously

as shown in Figure 32. Undefined rays are called out during the trace command. The surface where a ray became undefined can be pinpointed using the RAY command.

Misses element	Must go backwards to hit next element.

Hits elements simultaneously.

FIGURE 32 Example with defined rays (solid) and undefined rays (dashed)

    - RAY

The RAY command traces specific rays and prints the results. RAY has one argument, the number of the ray to be printed (iRay). The chief ray for Cassegrain.in is printed as:

MACOS>ray

Enter number of ray (1=chief ray, 0=quit):1 Ray	1 is the chief ray.

Ray	1 segment from Element	0 (InputRay) to Element	1 (SecMirOb): Starting point= 0.000000000D+00 0.000000000D+00 -6.000000000D+00

End point= 0.000000000D+00 0.000000000D+00 -5.400000000D+00 Direction= 0.000000000D+00 0.000000000D+00 1.000000000D+00

Segment Length= 6.000000000D-01 Total Length= 6.000000000D-01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray	1 is obscured by element 1

Ray	1 segment from Element	1 (SecMirOb) to Element	2 (Primary ): Starting point= 0.000000000D+00 0.000000000D+00 -5.400000000D+00

End point= 0.000000000D+00 0.000000000D+00 -8.881784197D-16 Direction= 0.000000000D+00 0.000000000D+00 1.000000000D+00

Segment Length= 5.400000000D+00 Total Length= 6.000000000D+00

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray	1 segment from Element	2 (Primary ) to Element	3 (Secondar): Starting point= 0.000000000D+00 0.000000000D+00 -8.881784197D-16

End point= 0.000000000D+00 0.000000000D+00 -4.061145902D+00 Direction= 0.000000000D+00 0.000000000D+00 -1.000000000D+00

Segment Length= 4.061145902D+00 Total Length= 1.006114590D+01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray	1 segment from Element	3 (Secondar) to Element	4 (Ref_surf):

Starting point= 0.000000000D+00 0.000000000D+00 -4.061145902D+00 End point= 0.000000000D+00 0.000000000D+00 -4.060145902D+00 Direction= 0.000000000D+00 0.000000000D+00 1.000000000D+00

Segment Length= 1.000000000D-03 Total Length= 1.006214590D+01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray 1 segment from Element 4 (Ref_surf) to Element 5 (foc_pln ): Starting point= 0.000000000D+00 0.000000000D+00 -4.060145902D+00

End point= 0.000000000D+00 0.000000000D+00 1.500000000D+00 Direction= 0.000000000D+00 0.000000000D+00 1.000000000D+00

Segment Length= 5.560145902D+00 Total Length= 1.562229180D+01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

This printout gives the start and end point of each segment of the ray in global coordi-nates. The direction is a unit vector in global coordinates. The segment length refers to the geometric distance between the ray start and end points; it is not scaled by index of refraction. The total length gives a running sum of the length of the ray that is scaled by the index of refraction. The index give the index of refraction for the previous and cur-rent element.

To print the lower marginal ray, we see from the MAP command that the iRay number is 2, and respond accordingly:

MACOS>ray

Enter number of ray (1=chief ray, 0=quit):2

Ray	2 has aperture coordinates	8 and	1.

WF coordinates are 129 and 122.

Adjacent rays are	0,	0,	0,	6.

Ray	2 segment from Element	0 (InputRay) to Element	1 (SecMirOb): Starting point= -4.440892099D-16 -2.000000000D+00 -6.000000000D+00

End point= -4.440892099D-16 -2.000000000D+00 -5.400000000D+00 Direction= 0.000000000D+00 0.000000000D+00 1.000000000D+00

Segment Length= 6.000000000D-01 Total Length= 6.000000000D-01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray	2 segment from Element	1 (SecMirOb) to Element	2 (Primary ): Starting point= -4.440892099D-16 -2.000000000D+00 -5.400000000D+00

End point= -4.440892099D-16 -2.000000000D+00 -1.851851852D-01 Direction= 0.000000000D+00 0.000000000D+00 1.000000000D+00

Segment Length= 5.214814815D+00 Total Length= 5.814814815D+00

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray	2 segment from Element	2 (Primary ) to Element	3 (Secondar): Starting point= -4.440892099D-16 -2.000000000D+00 -1.851851852D-01

End point= -1.110223024D-16 -4.999999999D-01 -4.096296297D+00 Direction= 7.951199381D-17 3.580901857D-01 -9.336870027D-01

Segment Length= 4.188888889D+00 Total Length= 1.000370370D+01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray	2 segment from Element	3 (Secondar) to Element	4 (Ref_surf): Starting point= -1.110223024D-16 -4.999999999D-01 -4.096296297D+00

End point= -1.098674950D-16 -4.947992095D-01 -4.038085968D+00 Direction= 1.975982230D-17 8.899032834D-02 9.960324902D-01

Segment Length= 5.844219807D-02 Total Length= 1.006214590D+01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray	2 segment from Element	4 (Ref_surf) to Element	5 (foc_pln ):

Ray-Trace Commands

Starting point= -1.098674950D-16 -4.947992095D-01 -4.038085968D+00 End point= -1.877761358D-26 -8.456685352D-11 1.500000000D+00 Direction= 1.975982230D-17 8.899032834D-02 9.960324902D-01

Segment Length= 5.560145902D+00 Total Length= 1.562229180D+01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

This printout is similar to the chief ray printout. However, the first three lines are new. They specify the ray grid coordinates, the coordinates of the diffraction amplitude matrix grid point with which the ray is associated, and the iRay numbers of the adjacent rays. The adjacent rays are listed in (-x, +x, -y, +y) order.

    - OPD

The commands that perform full-beam ray-trace analyses are SPOt and OPD. The OPD (for Optical Path Difference) command traces the full bundle of rays to a specified ele-ment. At that element it computes an OPD for each ray and plots the beam OPD map. It also computes and prints the RMS OPD for the full beam.

The optical path difference of a ray is the difference between the optical pathlength of that ray and the chief ray. A positive OPD indicates that the ray travelled farther than the chief ray.

The OPD map as plotted is always a square grid corresponding to the aperture ray grid printed out in the MAP command. To see the true locations of the rays the SPOt com-mand must be used. Rays which are obscured or vignetted by obscurations or apertures on the current or previous elements are not plotted by default. The OBScuration com-mand changes which rays are plotted and is discussed in Section 5.2.7.

OPD plots will be representative of the system wavefront error if evaluated at an exit pupil or other appropriate reference surface. Such a surface will generally be located in the near-field region of the beam, at a point where the ray grid is regular.

NOTE: OPD plots made in a caustic or other region where the beam sampling geometry is disrupted will not auccurately capture system wavefront error. Better results may usually be obtained by using a near- or far-field diffraction propagation to the point of interest, and then plotting the phase of the complex amplitude (see Section 6).

The OPD command has a single argument iElt which is the number of the element where the OPD map or spot diagram is to be computed. The element numbering is as specified when creating the .in-file.

OPD plots are not affected by STRETCH setting. OPD plots are always on a linear scale.

The default plot type is GRAy scale. The plot type can be changed with the following commands: WIRe, SLIce, GRAy, COLumn, ROW, CONtour, TEXt, MATlab,FITs, or BINary. These plotting commands are discussed in more detail in Section 7.3.

The following examples show output of the OPD command with the GRAY, WIRe, and

BINary plot types for Cassegrain.in.

MACOS>opd

Enter element where OPD is to be evaluated [0]:3 Tracing	150 rays...

Compute time was	0.1445 sec RMS OPD error is 1.609556E-02

Graphics device/type (? to see list, default /NULL): /xw

FIGURE 33 OPD GRAy scale plot

MACOS>wire

Plot type set to WIRE

MACOS>opd

Enter element where OPD is to be evaluated [3]:3 RMS OPD error is 1.609556E-02

Type <RETURN> for next page:

FIGURE 34 OPD ROW plot

MACOS>bin

Plot type set to BINARY

MACOS>opd

Enter element where OPD is to be evaluated [3]:3 RMS OPD error is 1.609556E-02

Writing OPD Map, Elt=3

DIRECT ACCESS File=Cassegrain.OPD3

Ray-Trace Commands

Binary array is of dimension 128 128 0. Magnitude of central pixel is 0.0000000000D+00

    - SPOT

Like the OPD command, the SPOt command traces the full bundle of rays to a specified element. At that element it computes and plots a spot diagram.

A spot diagram is an x-y plot across the beam. It puts a dot at the point of incidence of each ray as projected onto a plane across the specified element (see Figure 35).

Rays which are obscured or vignetted by obscurations or apertures on the current or pre-vious elements are not plotted by default. The OBScuration command changes which rays are plotted and is discussed in Section 5.2.7.

The SPOt command has the argument iElt: the number of the element where the spot diagram is to be computed. In addition, the user is asked to select which output coordi-nates to use to define the x and y axes. The choices are Tout, Enter or Beam (see Sec-tion 4). SPOt plots an x-y scatter format unless plot types TEXt, BINary, or MATlab are selected, in which case a “.spot” file is written to disk. Figures 4 and 36 show spot diagrams for Cassegrain.in.

Element

Projection plane

FIGURE 35 Spot on the projection plane

MACOS>spot

Enter element where spot diagram is to be computed [0]:4 Enter output coordinate option (Tout, Enter or Beam): [Tout]: Tracing 150 rays...

Compute time was 0.3086 sec

Graphics device/type (? to see list): /xw

Chief ray location: x= 0.0000000D+00 y= 0.0000000D+00 Centroid of spot diagram: x=-5.2080625E-18 y= 0.0000000E+00 MACOS>

    - OBSCURATION

By default, the SPOt and OPD commands do not plot rays which are obscured or vignetted by obscurations or apertures on the current or previous elements. The OBScu-ration command is used to change this default.

OBS has three settings. OBS=ALL plots every ray -- even rays that are obscured. OBS=POSITIVE, the default, plots only rays that are not obscured on the current or on any previous element. OBS=NEGATIVE plots only those rays that are obscured some-

FIGURE 36 Exit pupil and detector SPOt diagram for Cassegrain.in.

where upstream of the specified element. Of course, rays that become undefined upstream are not plotted under any of these options.

Figure 37 shows spot diagrams of the Cassegrain telescope, for comparison with Figure

36. The spot diagram on the left is plotted with OBS=ALL, showing all rays. The spot dia-gram on the right is plotted with OBS=NEGATIVE, showing only obscured rays.

MACOS>spot

Enter element where spot diagram is to be computed [4]:5 Enter output coordinate option (Tout, Enter or Beam): [Tout]: Tracing 150 rays...

Compute time was 0.1211 sec

Chief ray location: x= 0.0000000D+00 y= 0.0000000D+00 Centroid of spot diagram: x=-1.1055660E-16 y=-1.1055660E-16 Type <RETURN> for next page:

MACOS>obs

Enter ray-trace obscuration option (ALL, POSITIVE, NEGATIVE) [POSITIVE]: ALL

All rays plotted on spot diagrams

MACOS>spot

Enter element where spot diagram is to be computed [5]:5 Enter output coordinate option (Tout, Enter or Beam): [Tout]:

Chief ray location: x= 0.0000000D+00 y= 0.0000000D+00 Centroid of spot diagram: x= 7.1761210E-17 y= 7.1761210E-17 Type <RETURN> for next page:

MACOS>

OBS=ALL	OBS=NEGATIVE

FIGURE 37 Exit pupil Spot diagrams with alternate OBS settings

Beam Set-Up Commands

#### SEGRAYTRACE

Trace a single ray that passes through the geometric center of a chosen segment element (its RptElt), printing the ray state at every downstream surface.  Useful for verifying that segmented-mirror prescriptions place segments where you expect.

MACOS> SEGRAYTRACE
    Enter segment number (0=quit): [0]: 5
    Ray  1 segment from Element 0 (InputRay) to Element 5 (Seg5):
       Starting point=  ...
       End point     =  ...   (should be RptElt(:,5))
       Direction     =  ...
    ...
    Enter segment number (0=quit): [0]: 0
    MACOS>

The source ray's position is automatically offset from ChfRayPos so the resulting ray (parallel to ChfRayDir for a collimated source, or aimed from ChfRayPos for a point source) lands exactly on the segment's recorded RptElt.  The segment-dispatch path is forced to fire only on the chosen segment via RayToSegMap; other segments are skipped.

The trace endpoint at the segment's element should match RptElt to within SFFZPSolve's tolerance (~1e-11 mm).  Larger mismatches diagnose generator bugs:

~180-deg rotational mismatch (e.g. Seg2's endpoint matches Seg5's RptElt): the segment-numbering psi-flip in the generator didn't fire when it should have.

~mm radial mismatch in the surface-normal direction: the generator computed RptElt on the conic only, missing the FF Zernike sag.

The command requires an Rx loaded with at least one Segment element; otherwise it prints 'Element N is not a Segment' and re-prompts.

#### Per-ray status reporting

Every ray in a trace records its terminal status:

OK (0) -- successfully terminated at last requested element

Obscured (1) -- blocked by an obscuration or aperture

Miss (2) -- missed a surface (no intersection found)

Bracket (3) -- root-finding bracket failure on a composite (FreeForm / aspheric / grid) surface

MaxIter (4) -- root-finder hit its iteration limit

Undef (5) -- initial state, never updated

The end-of-trace WARN summary breaks down failures by category and records the offending element index for the first failure of each ray.  Bracket / iter messages are throttled after the first 20 to keep the terminal usable when many rays fail.

### Beam Set-Up Commands

The next several commands are designed primarily to help the user correctly set up par-ticular computations, such as computation of beam wavefront error, or diffraction com-putations. These commands help the user implement perturbations of the optical train to evaluate performance impacts of misalignments, different field angles, or “zoom” set-tings. The STOP command specifies the system stop, either at an element or at a point in global coordinates, to help in defining the beam. It is required by other commands, such as PERTURB, FFP, PFP and FEX, that might move the source. The CENTER command finds source positions that cause the beam to be centered at a specified element. The FFP and PFP commands find source field angles that place the beam at a particular point on a particular surface, such as on a detector. The FEX command finds the system exit pupil and places a return surface at that point. These commands, together with the ORS and SRS commands discussed in the next section, provide a full capability for correctly set-ting up most ray-trace and diffraction analyses.

    - STOP

The lateral position of a beam propagating through a system is constrained by the pres-ence of a system stop. The system stop is an aperture somewhere in the system through which the beam passes. It defines the beam position and (usually its shape), as per Fig-ure 38. Certain commands (PERturb, FFP, PFP and FEX) require knowledge of the stop position to enforce the correct beam geometry.

The system stop is specified using the STOp command. STOp does not center the beam in the stop (CENTEr can be used to do this). It merely defines the system stop position. This information is used to constrain the beam when setting exit pupils or perturbing the source.

New source position, angle

System stop aperture defines beam position for changing field angles

Changing field angle moves image position

FIGURE 38 System stop

STOp has 2 or 3 arguments, depending on whether the stop is to be specified at an ele-ment or at a point in object space. The first argument is a character string specifying whether the stop is to be tied to a particular element (ELT) or to a particular point in object space (OBJ).

If the former, the next parameter requested is the element number. This is followed by a request for an offset vector: a 2-vector in local beam coordinates giving the center of the stop with respect to the vertex of the element (VptElt).

If the stop is to be specified in object space, the next request is for the 3-vector coordi-nates of the stop position.

MACOS>stop

The following examples illustrate the setting of stops for the Cassegrain.in example pre-scription. The first example sets the stop at the nominal location in 3-space of the pri-mary mirror vertex:

Stop at ELT or OBJect point? [OBJ]: object

Enter stop position in object space (x,y,z): [0.,0.,0.]:0,0,0 MACOS>

MACOS>stop

The second example puts the stop at the secondary mirror. The STOp command com-putes the equivalent object-space stop position:

Stop at ELT or OBJect point? [OBJ]: elt Enter system stop element number: [1]:3 Enter offset vector (dx,dy): [0.,0.]:0,0

Computed StopPos= 0.000000000D+00 0.000000000D+00 1.637981902D+01 MACOS>

    - CENTER

The CENTEr command solves for a new source position that places the beam at a speci-fied point on a specified element. This command is useful for enforcing the system stop; it has other applications as well. This command has two arguments: the element number of the element where the beam is to be placed; and the offset of the center of the beam from the vertex of the element. The CENTEr comand changes the source chief ray posi-tion ChfRayPos but not the field angle.

The following example uses the Cassegrain.in example to center a perturbed beam, with the result shown by tracing the chief ray to the specified element:

MACOS>center

Enter system stop element number: [1]:2 Enter offset vector (dx,dy): [0.,0.]:0,0

Beam centered in	2 iterations, error= 3.231486333D-15 MACOS>ray

Enter number of ray (1=chief ray, 0=quit): [0]: 1

Ray	1 is the chief ray.

Ray	1 segment from Element	0 (InputRay) to Element	1 (SecMirOb): Starting point= 6.055112155D-17 5.000007357D-03 -5.000005690D+00

End point= 6.055112155D-17 4.200001400D-03 -4.200000000D+00 Direction= 0.000000000D+00 -9.999998333D-04 9.999995000D-01

Segment Length= 8.000060899D-01 Total Length= 8.000060899D-01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

Ray	1 is obscured by element 1

Ray	1 segment from Element	1 (SecMirOb) to Element	2 (Primary ): Starting point= 6.055112155D-17 4.200001400D-03 -4.200000000D+00

End point= 6.055112155D-17 -3.230922474D-15 0.000000000D+00	Zeroed!

Direction= 0.000000000D+00 -9.999998333D-04 9.999995000D-01

Segment Length= 4.200002100D+00 Total Length= 5.000008190D+00

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

(etc...)

    - COORD

The COOrdinates command computes beam coordinates at any specified element. Beam coordinates are the projection onto that element of the xGrid and yGrid axes defined at the source element (see previous section). The COORDinates command takes

a single argument: the element number iElt. It prints out two orthogonal unit vectors in

Beam Set-Up Commands

MACOS>coord

response. These vectors are the projection onto the surface at the chief ray of the xGrid

and yGrid axes.

These coordinates are computed using four differential chief rays. The first two differ slightly in x- and y-angle from the nominal chief ray. In the near-field of the source, such as at a pupil element, the incidence points of these rays on the element differ enough to indicate the projected axes directions. The second two differential chief rays differ in x- and y-position from the nominal chief ray. They are used to indicate axes direction in the far-field of the beam, such as at a focal plane.

Note: The beam coordinates may undergo a sign change in the vicinity of a focus. The unit vectors will be correct and consistent, however, their signs may abruptly flip. Use common sense in inter-preting the results of the COORD command!

The following example uses the Cassegrain.in example, determining the coordinates projection onto the secondary obscurations (iElt=1), the exit pupil (iElt=5), and the detector (iElt=6):

Enter element number: [1]:1

Computed xLocal= -1.000000000D+00 0.000000000D+00 0.000000000D+00 Computed yLocal= 0.000000000D+00 -1.000000000D+00 0.000000000D+00 Computed zLocal= 0.000000000D+00 0.000000000D+00 1.000000000D+00

MACOS>coord 5

Enter element number: [1]: 5

Computed xLocal= -9.999999998D-01 0.000000000D+00 2.229180881D-05 Computed yLocal= 0.000000000D+00 -9.999999998D-01 2.229180881D-05 Computed zLocal= 2.229180880D-05 2.229180880D-05 9.999999995D-01

MACOS>coord 6

Enter element number: [1]: 6

Computed xLocal= -1.000000000D+00 0.000000000D+00 0.000000000D+00 Computed yLocal= 0.000000000D+00 -1.000000000D+00 0.000000000D+00 Computed zLocal= 0.000000000D+00 0.000000000D+00 1.000000000D+00

MACOS>pert 6

    - FFP

The find field point (FFP) command finds the field angles for which the chief ray inter-sects a specified element at a specified point. It resets both the source position and angle (ChfRayAng and ChfRayPos) to both satisfy the system stop condition and to place the beam at the given point.

The FFP command has two arguments: the element number of the surface at which the chief ray intercept is specified; and the offset of the chief ray from the vertex of the tar-get element in global coordinates. The STOP command must be used prior to invoking FFP.

The following example uses the Cassegrain.in example, finding the beam that offsets an image by (1e-3, -2e-3):

MACOS>stop

Stop at ELT or OBJect point? [Elt]: obj 0,0,0

Enter stop position in object space (x,y,z): [0.,0.,0.]: 0,0,0 MACOS>ffp 6

Enter element number: [6]:

Enter offset vector in global units (dx,dy): [0.,0.]:1e-3,-2e-3 Field angle finder results:

Did	1 iterations, error= 6.936774704D-11

Old dx= 0.000000000D+00 New dx= 1.000000031D-03 0.000000000D+00	-2.000000062D-03

Old crd= 0.000000000D+00 New crd= 4.458354140D-05 0.000000000D+00	-8.916708280D-05

1.000000000D+00		9.999999950D-01 Old crp= 0.000000000D+00 New crp= -2.229177070D-04 0.000000000D+00	4.458354140D-04

-5.000000000D+00	-4.999999975D+00

Accept the new chief ray? [YES]: yes

MACOS>ray 1	Trace ray to show new incidence point...

Enter number of ray (1=chief ray, 0=quit): [0]: 1 Ray	1 is the chief ray.

Ray	1 segment from Element	0 (InputRay) to Element	1 (SecMirOb): Starting point= -2.229177070D-04 4.458354140D-04 -4.999999975D+00

End point= -1.872508748D-04 3.745017496D-04 -4.200000000D+00 Direction= 4.458354140D-05 -8.916708280D-05 9.999999950D-01

Segment Length= 7.999999791D-01 Total Length= 7.999999791D-01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

(etc...)

Ray	1 segment from Element	5 (ExitPupi) to Element	6 (Detector): Starting point= 1.812075347D-04 -3.624150694D-04 -4.060145887D+00

End point= 1.000000031D-03 -2.000000062D-03 1.500010000D+00 Direction= 1.472607007D-04 -2.945214014D-04 9.999999458D-01

Segment Length= 5.560156189D+00 Total Length= 1.462230217D+01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

MACOS>

    - PFP

The pixel field point (PFP) command is the same as the FFP command, in that it finds the field angles for which the chief ray intersects a specified element at a specified point. It differs in that the incidence point is specified in pixel coordinates. PFP resets both the source position and angle (ChfRayAng and ChfRayPos) to both satisfy the system stop condition and to place the beam at the target point.

The PFP command has two arguments: the element number of the surface at which the chief ray intercept is specified; and the offset of the chief ray from the vertex of the tar-get element in global coordinates. The STOP command must be used prior to invoking PFP.

The following example uses the Cassegrain.in example, finding the beam that puts an image at pixel location (20, -100):

MACOS>PFP 6

Enter element number: [6]:6

Enter pixel size for placing image on pixel array: [0.]:15e-4 Enter image position in pixel units (dx,dy): [0.,0.]:20,-100 Field angle finder results:

Did	2 iterations, error= 1.581159622D-12

Old dx= 1.000000031D-03 New dx= 3.000000000D-02

-2.000000062D-03	-1.500000000D-01 Old crd= 4.458354140D-05 New crd= 1.337301749D-03

-8.916708280D-05	-6.686508743D-03

9.999999950D-01		9.999767508D-01 Old crp= -2.229177070D-04 New crp= -6.686508743D-03 4.458354140D-04	3.343254372D-02

-4.999999975D+00	-4.999883754D+00

Accept the new chief ray? [YES]:

Beam Set-Up Commands

MACOS>ray 1	Trace ray to show new incidence point...

Enter number of ray (1=chief ray, 0=quit): [0]: 1 Ray	1 is the chief ray.

Ray	1 segment from Element	0 (InputRay) to Element	1 (SecMirOb): Starting point= -6.686508743D-03 3.343254372D-02 -4.999883754D+00

End point= -5.616797930D-03 2.808398965D-02 -4.200000000D+00 Direction= 1.337301749D-03 -6.686508743D-03 9.999767508D-01

Segment Length= 7.999023513D-01 Total Length= 7.999023513D-01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

(etc...)

Ray	1 segment from Element	5 (ExitPupi) to Element	6 (Detector): Starting point= 5.436452666D-03 -2.718226333D-02 -4.060076800D+00

End point= 3.000000000D-02 -1.500000000D-01 1.500010000D+00 Direction= 4.416714741D-03 -2.208357371D-02 9.997463720D-01

Segment Length= 5.561497351D+00 Total Length= 1.462402454D+01

Index(i-1)= 1.000000000D+00 0.000000000D+00 Index(i)= 1.000000000D+00 0.000000000D+00

MACOS>

    - FEX

The Find EXit pupil (FEX) command computes the location of the system exit pupil and places an optimized spherical reference surface at that point. By “optimized,” we mean that the spherical reference surface is correctly set up to perform a far-field diffraction propagation to the next element, which is usually a detector.

To use FEX, the prescription must be set up with two return surfaces followed by a detector focal plane or reference surface. The first return surface is placed at the location of the focal plane detector, and the second is placed in the vicinity of the exit pupil. FEX locates the second precisely at the exit pupil location and computes its orientation and curvature based on the position of the detector element.

FEX works for systems where the exit pupil is located in front of the detector. It is not useful for telecentric systems (where the exit pupil is at infinity) or for cases where the exit pupil is located behind the detector.

FEX traces two rays: the nominal chief ray and a differential chief ray. The field angle of differential chief ray differs by 1 arcsecond from the nominal chief ray angle. Both rays are traced to the detector return surface and then backwards to find their intersection. The intersection of the nominal and differential chief rays defines the center of the exit pupil. The parameters of the exit pupil (the second return surface) are set as follows:

      - vertex, VptElt, to the center of the exit pupil

      - radius of curvature, KrElt, to the distance between the exit pupil and detector

      - principal axis, PsiElt, to the chief ray direction, ChfRayDir

The result is a reference sphere aligned with the spherical wavefront converging to the point where the chief ray hits the detector. This configuration correctly models diffrac-tion images at that point field point. If the optical system is perturbed, the FEX com-mand must be rerun.

FEX requires two arguments: the number iElt of the exit pupil return surface and the location in global coordinates of the vertex of the aperture stop.

For example, we added two return surfaces to the Cassegrain.in prescription just before the detector using a text editor (see Section 4.9). The first is a return surface at the detector location. The second is close to the exit pupil. The entire .in-file is in Appendix A.1.4.

MACOS>show 5

Enter number of element (0=aperture): [0]: 5

| iElt= EltName= Element= Surface= | 5 rtn1 Return Conic |  |
| --- | --- | --- |
| fElt= | 1.000000000D+22 |  |
| eElt= | 0.000000000D+00 |  |
| KrElt= | -1.000000000D+22 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 1.500000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 1.500000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 0.000000000D+00 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 0.000000000D+00 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

nECoord=	-6

MACOS>show 6

Enter number of element (0=aperture): [0]: 6

| iElt= EltName= Element= Surface= | 6 rtn2 Return Conic |  |
| --- | --- | --- |
| fElt= | 6.000000000D+00 |  |
| eElt= | 0.000000000D+00 |  |
| KrElt= | -6.000000000D+00 |  |
| KcElt= | 0.000000000D+00 |  |
| psiElt= | 0.000000000D+00 | 0.000000000D+00 1.000000000D+00 |
| VptElt= | 0.000000000D+00 | 0.000000000D+00 -4.500000000D+00 |
| RptElt= | 0.000000000D+00 | 0.000000000D+00 -4.500000000D+00 |
| IndRef= | 1.000000000D+00 |  |
| Extinc= | 0.000000000D+00 |  |
| nObs= | 0 |  |
| ApType= | None |  |
| zElt= | 6.000000000D+00 |  |
| PropType= | Geometric |  |
| nECoord= | -6 |  |

Then the exit pupil is optimized:

MACOS>fex

Enter number of exit pupil return surface: [5]:6 Tracing	173 rays...

Chief ray location: x= 0.0000000D+00 y= 0.0000000D+00 z=-

4.5000000D+00

Centroid location: x= 7.9797280D-17 y= 8.6736174D-17 z=-

4.4875916D+00

Centroid offset from chief ray: x=-7.9797280D-17 y=-8.6736174D-17 z=-1.2408352D-02

Exit pupil finder results:

Old f= 6.000000000D+00	New f= 6.790667844D+00

Beam Set-Up Commands

Old z= 6.000000000D+00	New z= 6.790667844D+00 Old psi= 0.000000000D+00 New psi= 9.936451891D-17 0.000000000D+00	1.080049119D-16

1.000000000D+00		1.000000000D+00 Old Vpt= 0.000000000D+00 New Vpt= 0.000000000D+00 0.000000000D+00	0.000000000D+00

-4.500000000D+00	-5.290667844D+00

Accept the new element? [YES]:

MACOS>

    - PERTURB

The PERturb command allows the user to change the current optical prescription by rotating or translating the light source or any of the optical elements. PERturb accepts as arguments rotation and translation perturbation vectors, which it applies directly to the prescription data. Rotations of an element are taken about the prescription rotation point RptElt. Subsequent ray traces or propagations will use the new prescription.

PERturb differs from MODify in that it changes only the position and orientation of ele-ments (MODIFY can be used to change any parameter in the prescription). PERturb is also different from MODify in that PERturb uses the rotation point and proper kinemat-ics to apply rotational perturbations (PERturb rotations are relative to the old orienta-tion) and MODify specifies a absolute location. PERturb is not limited to small motions.

To reset the prescription to its unperturbed condition, it is necessary to explicitly reverse the PERturb commands executed previously or to reload the prescription using the OLD command. Due to the noncommutativity of rotations, the latter method is to be preferred if more than one rotation has been applied.

The PERturb command takes arguments specifying the element to be perturbed and the perturbation vectors. The element is specified by number iElt which is in the range: 0 (the source) to nElt (the number of elements in the active prescription). Units for rota-tion perturbations are radians. The units of the translation perturbations are in the cho-sen base units (meters, inches, whatever).

Rotations are applied using Euler axis/angle parameters (Ref. Hughes). The axis of the rotation is given by the direction of the rotation perturbation vector. The angle is given by its magnitude. This approach is used in preference to Euler angles or quaternions as it provides a smooth transition to the usual small-angle linear approximations.

If iElt=0, indicating the source is to be perturbed, the user is prompted for the location of the aperture stop in global coordinates. For instance to look 0.5 deg (8.72664d-3 rad) off the x-axis, a rotation about the y-axis is commanded:

MACOS>perturb

Enter element to be perturbed: [0]:0

Enter rotational perturbation vector (x,y,z): [0.,0.,0.]:0,8.72664d-3,0 Enter translational perturbation vector (x,y,z): [0.,0.,0.]:0,0,0 Computing new perturbed system parameters

MACOS>

If iElt>0, indicating a regular element, the user is asked whether the perturbation vec-tors are in local element coordinates or in global coordinates. If in local coordinates, the user is prompted for a single 6-vector perturbation in those coordinates. If in global

coordinates, the user is prompted for a rotational perturbation 3-vector, then a transla-tional 3-vector. For instance, to translate the secondary mirror in Cassegrain.in:

MACOS>pert

Enter element to be perturbed:3

Enter coordinate system for perturbation (ELEMENT or GLOBAL): [GLOBAL]: Enter rotational perturbation vector (x,y,z): [0.,0.,0.]:0,0,0

Enter translational perturbation vector (x,y,z): [0.,0.,0.]:0,0,1d-6 Computing new perturbed system parameters

MACOS>

    - PREAD

The PREad command reads element perturbations from a specified datafile. MACOS assumes that the name of the perturbation file is infilename.pert. If this file does not exist, MACOS prompts for ther perturbation file name.

The perturbation file must be in the following all numeric format. The perturbations must be in global coordinates. The order of the perturbation vectors in the .pert-file must be identical to the order of the elements (the first perturbation vector corresponds to the first element, the sixth perturbation vector corresponds with the sixth element). Each element must have a perturbation vector. If an element is unperturbed, then a string of zeroes should be entered as its perturbation vector. The perturbation vectors are rota-tional perturbations and translational perturbations. The 6 components of the perturba-tion vector must be separated by spaces.

MACOS>pre

File Cassegrain.pert read. MACOS>

    - ATMOS

MACOS provides an atmospheric phase disturbance function with the ATMos com-mand. ATMos imposes a ray pathlength and direction perturbation on the beam at a spec-ified element. The ray perturbations are derived from a Kolmogorov phase disturbance model, parameterized by the atmospheric turbulence coherence length r0, and driven by a random number generator. The seed for the random number generator can be set using the SEED command.

ATMos provides two options for specifying the atmospheric grid. The RAY option treats each ray as an independant but correlated random variable, determining the phase dis-turbance (and so the direction and OPD) for each ray directly from the Kolmogorov generating function. The ATM option implicitly defines a coarser grid of “atmospheric states” than the ray grid provides. Essentially, the phase disturbance is determined for cells whose size is specified by the user, but which will be in general larger than the ray grid cell size. Ray direction and OPD values are computed by interpolating from the surface defined by the atmospheric phase states to the ray grid cells.

ATMOS takes four or five arguments, depending on how the grid is to be specified (RAY or ATM). An example of the use of ATMOS in an adaptive optics system model is pro-vided in Appendix A.4. A sample dialog showing use of ATMOS follows:

MACOS>atm

Enter number of atmosphere phase screen element: [0]:1 Enter atmosphere r0: [12.4492]: 24

Enter atmosphere wavelength: [2.854331E-05]: 2.854331E-05 Enter atmosphere tilt participation (0-1): [1.00000]:0 Use RAY grid or separate ATMospheric grid? [RAY]: atm

Computed xLocal= 0.000000000D+00 -1.000000000D+00 0.000000000D+00 Computed yLocal= -1.000000000D+00 0.000000000D+00 0.000000000D+00 Computed zLocal= 0.000000000D+00 0.000000000D+00 -1.000000000D+00

Beam Set-Up Commands

Tracing 12113 rays... Compute time was	0.2266 sec

Enter grid spacing: [1.56840]:20 MACOS>

Here the “tilt participation” regulates the percentage of the Kolmogorov that is to be included in the phase screen. Tilt is explicitly removed when this factor is set to zero, and increases proportionally as the factor goes to 1, which signifies full Kolmogorov tilt. An example, showing a typical atmospheric wavefront from AOexample.jou (Appendix A.4), is shown in Figure 39.

FIGURE 39 Atmospheric disturbances (AOexample).

    - POLARIZATION

The POLarization command turns on polarization ray tracing. The default is no polarization ray tracing.

    - NOPOLARIZATION

The NOPOlarization command turns off polarization ray tracing. This is the default setting.

    - SINT

The Surface INTerpolate (SINT) command activates the surface interpolation of ele-ments. Prior to using the SINT command, the surface is assumed to be a conic-of-revo-lution. MACOS assumes that the interpolation input file has the name in-file.srf#, where # is the number of the element the data describes. If that file does not exist, MACOS prompts the user for the name of the file.

MACOS>SINT

Computing interpolated surface data... Compute time was	5.7871 sec

MACOS>

## SECTION 6: Diffraction Analysis

This section presents commands for physical optics modeling. Most optical systems are adequately modeled using a single-plane, exit-pupil diffraction model. Other systems are better modeled using multiple planes of diffraction. This section describes how to set up .in-files and other necessary data for both cases. The commands discussed in this section are:

ORS	SRS

BEAm	REGrid

SCAlar	VECtor

The diffraction modeling routines of MACOS can accurately compute both near-field

(Fresnel diffraction) and far-field (Fraunhoffer diffraction) effects. Roughly speaking, near-field propagation takes the beam from pupil to pupil, regions where the beam is large compared to a wavelength. Far-field propagations take the beam from a pupil to a focal point, or from a focal point to a pupil. In the vicinity of a focus, the beam diameter approaches a wavelength.

Examples of systems where near-field diffraction is important include laser beam trans-fer systems, or spatial filters, where the beam propagates from pupil to pupil via a far-field plane where an aperture or obscruation is placed. Examples of far-field diffraction include imaging systems and high-gain directional antennas. In some cases both types of diffraction are important, and multiple planes of diffraction should be used.

### Single- versus Multi-plane Diffraction

The first step in setting up a problem is to decide whether the important diffraction effects can be captured using a single-plane model, or whether multi-plane diffraction is neccessary. Most imaging systems are adequately modeled using the single-plane approach, with one far-field diffraction being used to create the image. Section 6.2.2 dis-cusses the set up of the .in-file for a single far-field propagation.

Systems for which diffraction effects of beam truncation due to apertures or obscura-tions may be significant may be best modeled using multiple planes of diffraction.

Examples are systems with spatial filters and coronographic systems. Examples of sys-tems requiring near-field propagation, whether multi-plane or no, include telecentric systems, defocused systems, or any system for which intermediate-plane intensities are desired.

If a multi-plane model is necessary, the first step is to lay out the optical system and determine the sequence of propagations. Consider the Cassegrain telescope with relay optics (including a fold near the focus of the main telescope) shown in Figure 40. Figure 41 shows reference surfaces added for multiplane diffraction calculations. The beam is partially obscured at the main and relay telescope secondary mirror supports (“spi-ders”). There are 4 diffraction propagations which are clearly shown in the diffraction schematic, Figure 42. Note that points A and D are used in the model twice. The first time each is used to model the spider obscurations. The second time, the points are used as a reference surfaces for diffraction calculations.

Spiders

Primary

Secondary

Spiders

Fold mirror

Relay secondary Relay primary Focal plane

FIGURE 40 Optical schematic of Cassegrain telescope and relay optics example

The first near-field propagation, PropType=5, occurs between the main telescope pri-

3	zElt6

A 5 6	7  B

zElt4

zElt3

zElt8	zElt11

8 C

zElt10

zElt13 D 14

FIGURE 41 Reference surfaces for multi-plane diffraction in Cassegrain telescope and relay optics example

iElt 1 PropType

2	3	4	5	6	7	8	9	10 11 12 13 14

A	A	B	C	D	D

FIGURE 42 Diffraction schematic for Cassegrain telescope and relay optics example

mary and secondary mirror (between reference surfaces 3 and 4 which are located close to these optics). The reference surfaces share a common center of curvature at point A. Since this is a converging beam, the zElt parameters are positive for both reference sur-faces (see Section 6.1.4).

The second near-field propagation occurs between the secondary mirror and the relay camera’s secondary mirror support “spiders.” It is a “through-focus” propagation, with zElt=6 positive (converging beam) and zElt=8 negative (diverging beam). The refer-ence point for this propagation is point B. The fold mirror is included in the propaga-tion.

The third propagation is at the converging beam between the relay primary and second-ary. This near-field propagation is centered at point C.

Setting up Diffraction Propagations

The last propagation is a far-field propagation, PropType=3, to the focal plane. It is cen-tered at the focal plane, point D.

### Setting up Diffraction Propagations

To use MACOS diffraction features, the .in-file prescription must be set up correctly. A surface which does not change the wavefront (reference, obscuring, return, or focal plane elements) must ideally be placed at the start and end of each diffraction propaga-tion. Each pair of reference surfaces must be defined together—slaved to each other—so as to avoid spurious phase errors. Slaving reference surfaces to each other means ensur-ing that they share a common axis and a common center of curvature. The reference sur-faces should be optimized to minimize sampling errors. The beam must be adequately padded to avoid aliasing. In certain conditions, the rays that are used to determine geo-metric phase should be regridded.

MACOS provides several commands to help ensure that the prescription data is set up correctly. The FEX command (discussed earlier in Section 5.3.6) optimizes placement of a return surface at an exit pupil for far-field diffraction propagation. The ORS command optimizes a reference surface for minimum sampling error by matching it to the best-fit sphere and aligning it to the chief ray. The SRS command slaves one reference surface to another, solving for location, orientation, and radius of the starting (or ending) surface of a near- or far-field diffraction propagation as a function of the parameters of the end-ing (or starting) surface. Used together, FEX, ORS and SRS enable the correct setup of any MACOS diffraction propagation.

The FEX, ORS and SRS commands are run after a .in-file prescription is loaded. They refine the prescription for elements defined in the .in-file. The results can be saved in a new .in-file using the SAVe command. If a PERturb, MODify, FFP, PFP, CENTER, or other command changes the source position or angle, or the position, angle or figure of an ele-ment in the propagation path, FEX, ORS and SRS should be rerun.

The results of propagation commands (such as INtensity or PIXilate) are stored in particular internal MACOS arrays. These arrays are accessible through SMACOS com-mon statements and the SMACOS call line. These include the ray-trace data variables defined in Section 5.1. Additional diffraction-specific arrays include:

    - WFElt(mdttl,mdttl,mWF) holds complex amplitude matrices that store the results of propagation commands (such as INtensity). Here mdttl is the maximum dimension of the “wavefront” amplitude arrays, and is defined in the paramXXX.cmn include files.

    - PixArray(mPix,mPix) stores the results of a pixilated image generation command (such as PIXilate or ADD). Here mPix is defined in the paramXXX.cmn include files.

MACOS implicitly enforces a padding requirement of at least 50% at compile time through internal data array declaration statements. This is accomplished by restricting the ray grid, which defines the maximum extent of the ray beam, to occupy the central half of the WFElt(mdttl,mdttl,mWF) matrices (see Section 2.3).

Once the surfaces for the diffraction calculations are properly set up, the beam can be propagated through the system by invoking one of the propagation commands described in this section (such as INtensity). These commands use geometric and Fresnel propa-

gators to propagate the complex amplitude matrix to the specified element. They then plot the appropriate data product.

We list here the available diffraction propagators (Table 13).

TABLE 13 MACOS diffraction propagators

| PropType=... | Description |
| --- | --- |
| Geometric | Geometric propagator, updates phase only as ei |
| FarField | Fresnel/Fraunhaufer far-field propagator: sphere- or plane- to-plane |
| NFSpherical | Fresnel near-field propagator: sphere- to-sphere |
| NFPlane | Fresnel near-field propagator: plane- to-plane |
| NF1 | Part 1 of the near-field propagator: sphere- to-plane. Use when an obscuration is to be imposed at an intermediate focus |
| NF2 | Part 2 of the near-field propagator: plane-to-sphere |
| GeomUpdate | Geometric propagator, forces update at each surface (don’t use) |
| NFS1surf | Near-field propagator: sphere- to-sphere, using end ref surf only |
| NFP1surf | Near-field propagator: plane- to-plane, using end ref surf only |
| SpatialFilter | Near-field propagator: sphere- to-sphere, with spatial filter |
| SF1surf | Near-field propagator with spatial filter, using end ref surf only |

#### Choosing Propagator Type

For imaging systems, it is usually clear whether a particular diffraction propagation is best modeled using the near-field or far-field propagator. If the light is to be evaluated at a focus, the far-field propagator should be used. Otherwise, the near-field propagator should be used. For far-field propagations to focal planes, it is important to place the focal plane element at the physical detector location, which may not coincide with the center of curvature of the exit pupil, which is to say, the system might be defocused (see Figure 43).

end

D

end  z2

start

z1

start

Using the near-field propagator	Using the far-field propagator

FIGURE 43 Imaging a defocused beam

Some systems require near-field propagation to model image formation. For example, spacecraft fine guidance sensors are defocused to spread a star image over multiple pix-els to improve centroiding accuracies (see Figure 43). These images can be accurately generated using the near-field propagators followed by a geometric propagation to the focal plane. The intensity in each pixel can then computed using the PIXilate com-mand described in Section 7.2.2.

Setting up Diffraction Propagations

Other conditions requiring near-field propagations include the presence of a spatial fil-ter, occulting mask, or other intensity-altering object at an intermediate far-field point in the beam.

In some cases the choice of propagator will not be clear. Evaluating the Fresnel number Fn may help determine the choice. Consider the defocused beam of Figure 43. Here the first reference surface has diameter D and axial distance, i.e distance from the center of curvature, z1. The second reference surface is located at an axial distance z2. The Fresnel number Fn is

D

2

Fn = ------------------------

z2 – z1 

(EQ 1)

Both far- and near-field propagators should be tested when Fn is in the range (0.1 – 1.0).

Obvious aliasing effects may also be mitigated by changing the sampling of the pupil, by increasing the diffractiongrid size, the ray grid size, or both. This may require using larger MACOS executable sizes (e.g., macos512 if aliasing is seen with macos256), as well as larger values for nGridPts.

#### Far-Field

Consider the two cases of far-field diffraction shown in Figure 44. Both are typical of systems where a far-field diffraction to a focal plane is required to compute an image or point-spread function. This may be the only diffraction computation in the system (a single-plane model) or it may be the last of several propagations in a multi-plane model.

iElt=4

iElt=6

iElt=5

iElt=7, iElt=9

iElt=6

iElt=4

iElt=7

iElt=5

iElt=8

Example A	Example B

FIGURE 44 Far-field propagation examples

Example A in Figure 44 is set up for exit pupil diffraction. The light is incident on ele-ment 4 (the secondary mirror in some reimaging optics) and is reflected to two refractive surfaces (elements 5 and 6) which together form a lens. Finally, the light forms an image on element 7. For exit pupil diffraction, MACOS needs two additional surfaces in the model: elements 8 and 9. Elements 7 and 8 must be return surfaces with element 7 located at the focal plane and element 8 located at the exit pupil. From element 7, MACOS traces light backwards in image space (this is the way MACOS performs the calcualtion, light does not actually propagate in this fashion) until it reaches the exit pupil (element 8). The path length of this segment is negative for the purposes of geo-metric propagation. It then reverses direction until reaching the focal plane (element 9). Element 9 must be a focal plane surface.

The exit pupil surface must be a sphere with radius of curvature equal to the distance from the exit pupil to the image on the focal plane. The FEX command can be used to setup the exit pupil location, principal axis, and radius of curvature based on the chief ray. The Fresnel propagation distance for the exit pupil, zElt, must be set to the radius of curvature of the exit pupil element.

NOTE: The principal axis of the exit pupil should be set along the line connecting the center of the exit pupil and the center of the image. The FEX command sets it along the chief ray which is not the same for systems with a large amount of odd aberrations, such as coma. For some applications, there can be a significant difference.

We assign the geometric propagator (PropType=1) to all elements, except for the exit pupil (element 8) which is assigned the far-field propagator (PropType=3). This directs MACOS to use the far-field propagator to compute the amplitude matrix at the focal plane, based on the phases calculated at the exit pupil by the geometric propagator.

The main advantage of using the exit pupil for far-field diffraction is that the ray grid is usually well-formed there. If a system is significantly aberrated, sampling at non-pupil planes may give distorted ray grids which causes a poor match between the locations of the ray grid and the locations of the complex amplitude matrix grid used in the diffrac-tion calculations. However if the system is well corrected and the ray grid is regular, a different plane may be used. A spot diagram at the plane provides a good qualitative check. Another check is provided by the propagation diffraction summary, which gives information on the “dx” or ray spacing across the reference surface.

Example B illustrates a system that uses a reference surface located away from the exit pupil to mark the start of a far-field diffraction calculation. Two fewer surfaces are required in this case, as the reference surface is placed in the beam between the final optical element (element 6) and the focal plane. All elements are assigned PropType=1, except the reference surface which is assigned PropType=3 to mark the start of the far-field propagation. Again, the reference surface is be a sphere with radius of curvature equal to the distance from the reference surface to the point of incidence of the chief ray with the focal plane. The SRS command can be used to find the orientation and radius for the reference surface.

#### Near-Field

There are basically four cases of near-field diffraction (see Figure 45). The algorithm uses planar reference surfaces. The first three cases deal with converging/diverging beams and use spherical reference surfaces. They employ the angular spectrum propaga-tion algorithm for the paraxial equation and the Sziklas/Seigman algorithm for spherical wavefronts.The fourth case, nearly collimated light, uses the propagation algorithm based on the standard form angular spectrum propagation for the paraxial wave equa-tion(see, e.g. Goodman [2])

The first three types are distinguished by the signs of the Fresnel propagation distance z to the reference point, the center of curvature of both of the reference spheres. z is posi-tive when the light is moving towards the reference point, as in converging beam. In a diverging beam, the light is moving away from the reference point and z is negative.

Both z’s are infinite for a collimated beam.

WARNING: In aberrated systems, the ray grid can lose its regularity. This is likely to be more pronounced near to focus, but can occur anywhere. The grid regularity can be restored using the REGrid command described in Sec-tion 6.3.3. In some cases this should be accompanied by an OPD interpolation step, which is not included in this release of MACOS.

Table 14 list the values for reference parameters for near-field propagations. The Fresnel propagation distances are entered both as the zElt parameters for each reference sur-face and as the focal length, fElt (or the radius of curvature, KrElt) of the element. The sphere-to-sphere propagations are all specified by setting PropType=5 for the first reference surface. The end reference surface has PropType=1, indicating geometric

Setting up Diffraction Propagations

Converging beam z1 > 0, z2 > 0

z1	z2

z2	 z1

Through-focus beam z1 > 0, z2 < 0

z1

z2

Diverging beam z1 < 0, z2 < 0

FIGURE 45 Near-field diffraction examples

Collimated beam z1 =  , z2 = 

propagation following the surface. The plane-to-plane propagation has PropType=4. (Unpowered elements such as fold mirrors can be included within the scope of a propa-gation by specifying PropType=5 or 4.) The last element must have PropType=1.

NOTE: PropType=2 assumes no phase change from the previous element and should be used with extreme cau-tion.

TABLE 14. Reference surface parameters for near-field propagation

| Parameter | Referece Surface 1 | Reference Surface 2 |
| --- | --- | --- |
| zElt | z1 | z2 |
| fElt | ¦z1¦ | ¦z2¦ |
| eElt | 0 | 0 |
| KrElt | -¦z1¦ | -¦z2¦ |
| KcElt | 0 | 0 |
| PropType | 4,5 | 1 |

The locations and orientations of the reference surfaces must be specified.

- Vertex location, VptElt, should be close to the optical element at the appropriate extreme of the propagation, but not so close that the surface intersects the optical element which would cause some rays to become undefined.

- The principal axis, psiElt, should be set so that the chief ray is normal to the refer-ence surface.

Initial guesses can be refined later with the ORS and SRS commands.

### Commands

#### ORS

The Optimize Reference Surface (ORS) command is used to refine the prescription for a reference surface used in near-field diffraction. To use ORS, the prescription must be set up with a reference surface at the desired location (VptElt) and with an approximate principal axis (psiElt). ORS requires one argument: the number iElt of the reference surface to be optimized.

ORS first relocates the reference surface so that the chief ray intersects the vertex point (VptElt). Then ORS tilts the specified reference surface so that psiElt is in alignment with the chief ray. Finally, ORS iteratively solves for the value of fElt that minimizes the OPD over a 15 by 15 grid of rays. The solution is the radius of the reference sphere that best fits the wavefront at that location. This process usually converges quite quickly. If it doesn’t, a message may appear indicating a problem. The usual source of difficulty is that the surface has been put in backwards! Try changing the signs of psiElt using the MODify command and repeat.

NOTE: ORS may not find the optimal reference surface for systems with a large amont of odd aberrations, such as coma. In these systems, the minimum OPD is obtained when the reference sphere is tilted from the chief ray.

The ORS command prints the old and new values of the element parameters and asks the user if the new ones are acceptable. Enter yes to accept the new values are for subse-quent computations. Enter no to retain the previous values. The new element data will not be kept unless a SAVe command is used to store the data. For example, using the seg.in example in Appendix A.5:

MACOS>ors

Enter number of element to be optimized:12 Reference surface optimization results:

RMSmin= 1.673727619D-06 nCalls= 11

Old f= 2.450000000D-01	New f= 2.486198802D-01 Old z= 2.450000000D-01		New z= 2.486198802D-01 Old psi= 2.382657340D-01 New psi= 2.382657340D-01 0.000000000D+00	0.000000000D+00

9.712000000D-01	9.712000000D-01   Old Vpt= 7.630588700D-04 New Vpt= 7.630594517D-04 0.000000000D+00	0.000000000D+00

-1.495089150D+00	-1.495089150D+00

Accept the new element data? (YES):

MACOS>

#### SRS

The Slave Reference Surface (SRS) command is used to slave one of the two reference surfaces used in a diffraction propagation to the other reference surface. It applies these changes directly to the element prescription. To use SRS, the prescription must be set up with a reference surface at the start and end of the propagation. If the propagation is a far-field propagation, the second surface can be a focal plane. To use SRS, the slaved reference surface must be in the MACOS prescription at the desired location (VptElt) and with an approximate principal axis (psiElt).

SRS is to be used in conjunction with ORS. ORS is run first for the first of the two refer-ence surfaces, then SRS is run for the second. ORS requires two arguments: the number iElt of the reference surface to be slaved and the number iElt to which the reference surface is to be slaved.

SRS first relocates the reference surface so that the chief ray intersects the vertex point (VptElt). Then SRS tilts the specified reference surface so that psiElt is normal to the

Commands

MACOS>srs

surface. Finally, SRS solves for the propagation distance zElt based on the zElt of the surface it is being slaved to and the optical path length between the surfaces along the chief ray. It computes the other element parameters accordingly.

The SRS command prints the old and new values of the element parameters and asks the user if the new ones are acceptable. Enter yes to accept the new values for subsequent computations. Enter no to retain the previous values. The new element data will not be kept unless a SAVe command is used to store the data. Continuing with the seg.in example gives

Enter number of element to be slaved:13 Enter number of element to slave to:12 Reference surface recalculation results:

Old f= 1.237919100D-01	New f= 1.274117874D-01 Old z= 1.237919100D-01		New z= 1.274117874D-01 Old psi= 2.382657340D-01 New psi= 2.382657340D-01 0.000000000D+00	0.000000000D+00

9.712000000D-01	9.712000000D-01 Old Vpt= 2.964279300D-02 New Vpt= 2.964279465D-02

0.000000000D+00	0.000000000D+00

-1.377371850D+00	-1.377371850D+00

Accept the new element? (YES):

MACOS>

#### REGRID

This feature is undergoing revision and not currently supported.

#### SCALAR

The SCAlar command sets a toggle to use a scalar formula when calculating single-plane diffraction (see Section 6.1). The SCAlar command has no arguments.

Scalar diffraction is the default and the only mode when polarization ray tracing is off (see Section 5.3.10). Vector diffraction is the default with polarization on.

#### VECTOR

The VECtor command sets a toggle to use vector formula when calculating single-plane diffraction (see Section 6.1). The VECtor command has no arguments.

Scalar diffraction is the default and the only mode when polarization ray tracing is off (see Section 5.3.11). Vector diffraction is the default with polarization on.

#### Propagation commands

Several commands, such as INT, LOG, ADD, etc. cause the beam to be propagated to the specified element using MACOS combined ray-trace and diffraction computations.

These commands are described in the next section.

#### BEAM

The BEAm command allows the user to select a source beam profile. The default is a uni-form beam. Alternatives are Gaussian, cosine-to-a-power, and dipole beam profiles.

MACOS>in

If a Gaussian beam is choosen, the user is then prompted for the diameter of the beam in x and y, where the diameter is the standard deviation of the Gaussian profile.

Figure 46 shows the intensity for a beam expander using a uniform beam and the inten-sity for the same system using a Gaussian beam.

Enter number of element where data is to be generated: [25]: 25

Tracing 4641 rays and propagating 65536 grid points... NF/PP Prop between Elt 24 and Elt 25 to WF 1:

z1= 0.0000D+00 dx1= 1.4286D-01

z2= 1.0000D+06 dx2= 1.4286D-01 min= 1.4286D-01 max= 1.4286D-01 dev= 6.8863D-

15 lin=100.00%

Compute time was 23.8438 sec Wavefront Propagation Data Summary:

Wavelength= 8.0000000D-07; Transmission Distance= 0.0000000D+00 u-v plane diam= 3.6428574E+01 du= 1.4285715E-01

x-y plane diam= 3.6428574E+01 dx= 1.4285715E-01 Type <RETURN> for next page:

MACOS>beam

Enter beam type (UNIFORM, GAUSSIAN, COS**POWER, DIPOLE): [UNIFORM]: gau

Enter x beam waist radius: [1.00000]: 0.3 Enter y beam waist radius: [1.00000]: 0.3 MACOS>in

Enter number of element where data is to be generated: [25]: 25

Tracing 4641 rays and propagating 65536 grid points... NF/PP Prop between Elt 24 and Elt 25 to WF 1:

z1= 0.0000D+00 dx1= 1.4286D-01

z2= 1.0000D+06 dx2= 1.4286D-01 min= 1.4286D-01 max= 1.4286D-01 dev= 6.8863D-

15 lin=100.00%

Compute time was 20.4062 sec Wavefront Propagation Data Summary:

Wavelength= 8.0000000D-07; Transmission Distance= 0.0000000D+00 u-v plane diam= 3.6428574E+01 du= 1.4285715E-01

x-y plane diam= 3.6428574E+01 dx= 1.4285715E-01 Type <RETURN> for next page:

MACOS>

Uniform beam	Gaussian beam

FIGURE 46 Propagation examples with Gaussian and uniform beams

## SECTION 7: Beam Propagation and Image Simulation

Image Simulation

This section presents the MACOS image generation and plotting commands. The com-plex amplitude matrices can be plotted to show pupil functions, point images, or images of extended objects. Composed images can combine multiple objects or multiple colors. Pixel array detectors can be simulated. Noise effects and image blurring can be simu-lated. Plot types include wireframe, contour, grayscale, row, and columns. Text and binary outputs are also available. The commands discussed in this section are:

INtensity	GAIn

PIxilate	AMplitude

WINdow	COMpose

ADD	DADD

MULTISPEC	RFILT

NOISE	SEED

BLUR	LOG

REAl	IMAGinary

GRAy	WIRe

SLIce	CONtour

COLumn	ROW

TEXt	BINary

FITs	MATlab

GBLUR

### Background

MACOS propagation commands, such as INtensity, propagate the beam through the optics to produce a complex amplitude matrix or matrices representing the light beam as projected onto a particular surface. The commands described in this section generate the complex amplitude matrices, perform basic analyses on them, and plot the results. Plots of intensity, magnitude and phase, and the real and imaginary parts of the amplitude are available. Images, or intensity plots, can be digitized or pixilated, simulating the images received by a pixel array detector such as a CCD. Multiple-source images can be com-posed on a single pixel array, allowing the accumulation of multiple-color or multiple object images. Simulated noise can be added.

Propagation works as follows. If no propagation command has been executed on the current system, it starts at the source. The rays are initialized according to the aperture and source geometry. The complex amplitude matrices are initialized to zero for those grid points that are not associated with rays (e.g. padding in the wavefront array). The grid points which are associated with rays are initialized to achieve uniform amplitude with total magnitude equal to the value set by the Flux .in-file parameter. If a propaga-tion has been performed previously, to an element upstream from the current element, then the previous values of the complex amplitude matrix are the starting point for the current computations.

The light is then advanced surface-by-surface through the system. Rays are traced to determine the phases that drive the geometric and Fresnel propagators. The propagators

are invoked at the end of each specified diffraction to update the complex amplitude matrices. Depending on the compile-time parameter mWF, one or more of these are stored in the WFElt array. These are accessible to the image generation commands for plotting. The propagation terminates when it has computed the complex amplitude at the specified last element.

Propagation commands print a summary of the scaling calculations for Fresnel propaga-tion. These provide information on the regularity of the ray grid (the “lin” or linearity is displayed as a percentage) at each reference surface. Warnings are issued if rays become undefined (usually indicative of a problem with the .in-file prescription).

### Beam Propagation Commands

To introduce and motivate the various beam propagation and image simulation com-mands, we will use a couple of examples. The first example is a simple non-optimized coronagraph. As shown on Figure 47, this system consists of three telescopes in sequence. At the focus of the first telescope is an occulting mask (iElt=6), which is designed to suppress the light from the core of an on-axis star. The beam is then re-expanded. A second mask is placed at the system stop (iElt=11), which is located in the collimated beam between the second and third telescopes. This forms a Lyot stop, which suppresses high-frequency light scattered from the occulted axial star. The detected on-axis image (iElt=16) is a somewhat complex and chaotic PSF, with greatly reduced core intensity.

Off-axis images are passed with very little attenuation, yielding near-ideal PSFs. The net effect is to greatly increase the visibility of dim off-axis objects compared to bright on-axis objects. Coronographs can greatly decrease dynamic range between 2 very close objects, such as a star and a planet might provide, enabling easier observation of the dimmer object.

The prescription and macro for generating the results plotted in this section are coroEx-ample.in and coroExample.jou, respectively. These files are listed in Appendix A.6.

FIGURE 47 DRAW sketch of coroExample.in system.

#### INTENSITY

The INtensity command takes the complex amplitude matrix at the specified element, computes the modulus squared to determine the intensity, and plots or exports it accord-ing to the active plot option (see Section 7.3).

Beam Propagation Commands

The INtensity command only accepts one argument–the number of the surface at which the intensity is viewed. This surface must be a stored wavefront. In the default gray scale plot, the intensity is displayed with black and white corresponding to the maixmum and minimum intensities, respectively.

We begin the example by propagating the on-axis beam through the first part of a near-field diffraction propagation (PropType=NF1), observing the image at the focus of the first telescope, both with and without the occulting mask:

MACOS> mod

MOD: [q]: nObs(6)=0	Turns off the occulting mask

nObs( 6) =	0 MOD: [q]: q MACOS> int 6

Enter number of element where data is to be generated: [1]: 6 Tracing 12661 rays and propagating 262144 grid points...

Prop to reference point Elt 5 and Elt 6 to WF 1:

z1= 6.3293D+00 dx1= 8.8773D-03 min= 8.8703D-03 max= 8.8832D-03 dev= 3.7728D-06 lin= 99.96%

z2= 1.0000D+30 dx2= 1.3925D-06

Compute time was	8.7598 sec Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D+00 u-v plane diam= 4.5362873D+00 du= 8.8772746D-03

x-y plane diam= 7.1158918D-04 dx= 1.3925425D-06

Peak intensity= 3.7152264D-02; Peak occurs at i= 257, j= 257

Sum of intensity= 8.7709310D-01 MACOS>

FIGURE 48 Unocculted image at first focal plane (coronagraph example)

Continuing with the example, the beam intensity can be observed at the other points of interest in the coronagraph, simply by specifying the appropriate element numbers. At the pupil, the Lyot stop, and then at the detector, the intensity plots are generated by:

MACOS> int 10

Enter number of element where data is to be generated: [6]: 10 Tracing 12661 rays and propagating 262144 grid points...

Prop from reference point Elt 6 and Elt 7 to WF 1:

z1= 1.0000D+30 dx1= 1.3925D-06

z2=-6.3293D+00 dx2= 8.8773D-03 min= 8.8703D-03 max= 8.8832D-03 dev= 3.7728D-06 lin= 99.96%

Geometric Prop between Elt 6 and Elt 10 to WF 1 Compute time was	6.8867 sec

Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D+00 u-v plane diam= 0.0000000D+00 du= 0.0000000D+00

x-y plane diam= 1.6256657D+01 dx= 3.1813417D-02

Peak intensity= 7.6769075D-05; Peak occurs at i= 258, j= 297 Sum of intensity= 1.3185163D-01

MACOS> int 11

Enter number of element where data is to be generated: [10]: 11 Tracing 12661 rays and propagating 262144 grid points...

Geometric Prop between Elt 10 and Elt 11 to WF 1 Compute time was	0.4170 sec

Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D+00 u-v plane diam= 1.6256657D+01 du= 3.1813417D-02

x-y plane diam= 1.6094488D+01 dx= 3.1496063D-02

Peak intensity= 2.9045304D-05; Peak occurs at i= 267, j= 320 Sum of intensity= 4.4725472D-02

MACOS>

FIGURE 49 On-axis beam profile at a pupil; at the Lyot stop; and at the detector (coronagraph example)

#### PIXILATE

The PIxilate command is used to simulate discrete element detectors. MACOS calcu-lates wavefront propagation on an array size which is independent of the detector size. The physical dimension of the diffracted image pixels is in general different from the detector pixel size being used to sample the signal. The PIxilate command imposes a square detector area over the intensity pattern. It averages the INtensity pattern within each detector pixel and sets that average equal to the value within the pixel. This is shown schematically in Figure 50. The PIxilated grid is centered on the chief ray (unless the PLOcate command has been executed) and the image location is determined by the Fourier transform of the exit pupil OPD map. Residual tilt in the OPD will shift the image from the chief ray intercept on the focal plane.

The PIxilate command has three arguments: the number of pixels per side (of the detector), the pixel width, and the element where the signal is to be PIxilated.

Continuing our example, we use the pixilate command to get a closer look at the occulted and unocculted images at the first focus:

Beam Propagation Commands

Point-spread function	Pixel boundaries	Pixilated image

FIGURE 50 Pixilization of images

MACOS> pix 6

Enter number of element where data is to be generated: [6]: 6 Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D+00 u-v plane diam= 2.2394683D+00 du= 1.7633609D-02

x-y plane diam= 3.5613167D-04 dx= 2.8041864D-06 Enter number of pixels per side: [128]: 64

Enter size of pixel: [2.804186E-06]:

Peak intensity= 1.6217206D-01; Peak occurs at i=	65, j=	65 Sum of intensity= 9.0994070D-01

MACOS> mod

MOD: [q]: nObs(6)=1	Reinstates the occulting mask

nObs( 6) =	1

MOD: [q]: q

MACOS> pix 6 64 2.804186E-06

Enter number of element where data is to be generated: [16]: 6 Tracing	3210 rays and propagating	16384 grid points...

Prop to reference point Elt 5 and Elt 6 to WF 1:

z1= 6.3293D+00 dx1= 1.7634D-02 min= 1.7628D-02 max= 1.7637D-02 dev= 2.6054D-06 lin= 99.99%

z2= 1.0000D+30 dx2= 2.8042D-06

Compute time was	0.4272 sec Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D+00 u-v plane diam= 2.2394683D+00 du= 1.7633609D-02

x-y plane diam= 3.5613167D-04 dx= 2.8041864D-06 Enter number of pixels per side: [128]: 64

Enter size of pixel: [2.804186E-06]:

Peak intensity= 8.2110545D-04; Peak occurs at i=	65, j=	72

Sum of intensity= 1.1480418D-01	Note the reduced intensity

MACOS>

#### STRETCH

It is often important to be able to visualize very low-level details of an image or beam. This may require changing the stretch of the image: the scaling of the intensity. Plotting on a log10 or sqrt intensity scale greatly increases the dynamic range of an image. The MACOS STRetch command changes the intensity scaling, to log10, sqrt, or linear scale.

An example of setting the stretch is:

MACOS> stretch

FIGURE 51 Pixilated images at first focal plane (coronagraph example)

FIGURE 52 Pixilated images at log10 and sqrt stretch (coronagraph example)

Enter image stretch type (LINEAR, LOG10, SQRT): [LINEAR]: log10

#### WINDOW

Using PIXilate to simulate detector images has a drawback: if the FEX command is used correctly, the image will always be centered in the PixArray window, even at dif-fering field angles. The WINdow command provides a means of fixing the detector in space, so that changes in field angle result in appropriate motion of the image on the detector. WINdow sets PixArray equal to a specific region on the detector.

WINdow has four arguments. The first is a character string indicating the user’s choice of output coordinate options. The second is the pixel size. The third is the location of the element vertex in pixel coords (this is used to define the origin of a pixel coordinate sys-tem, whose axis directions are defined by the previously selected coordinates. The fourth is the location in pixel coordinates of the center of the window:

MACOS> win

Enter output coordinate option (Tout, Enter or Beam): [Tout]: Tout

Enter pixel size for placing window: [0.]: 1.392542d-6 Enter pixel coords of element vertex (x,y): [0.,0.]: 0,0 Enter window location in pixel coords (x,y): [0.,0.]: 0,0

#### COMPOSE

The COMpose command defines the pixel size and dimensions for a detector window. It also sets up an accumulate option, whereby images from a series of propagations, under

Beam Propagation Commands

arbitrarily different conditions, can be incoherently added. It is useful for creating com-posite images from multiple-source or multiple-wavelength objects.

COMpose images are created by successive applications of the ADD command. ADD prop-agates the current beam, pixilates the intensity to the COMpose grid, and adds it to what is already present. COMpose is also used with the NOIse and BLUR commands, to add simulated noise to an image. An example using the COMpose and ADD commands is pre-sented in Section 7.2.6.

COMpose has three arguments: the number of pixels per side (of the detector), the pixel width, and the element where the signal is to be COMposed.

Continuing the coronagraph example, a pixel array is defined at the detector element:

MACOS> compose

Enter element where image is to be composed: [16]: 16

Enter number of pixels per side: [512]: 64

Enter size of pixel: [1.000000E-03]: 1.392542E-06

#### ADD

The ADD command propagates the current beam, pixilates the intensity to the COMpose

grid, and adds it to the composed pixel array image, which is stored in PixArray.

Continuing the coronagraph example, the on-axis image is added to the detector pixel array:

MACOS> add

Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D+00 u-v plane diam= 4.5362873D+00 du= 8.8772746D-03

x-y plane diam= 7.1158918D-04 dx= 1.3925425D-06

Peak intensity= 3.8883158D-04; Peak occurs at i= 257, j= 257 Sum of intensity= 4.4725467D-02

Image added. Plot composed image? [YES]: yes

FIGURE 53 Axial image ADDed to begin composed image.

#### Propagating a Perturbed Beam

The next step in the coronograph example is to define an off-axis star and propagate it to the detector, adding it into the composed image. The result is an image with 2 PSFs on

it, allowing us to evaluate the effectiveness of the coronagraph in reducing dynamic range between neighboring objects.

MACOS> stop

Stop at ELT or OBJect point? [Elt]: elt Enter system stop element number: [1]: 11 Enter offset vector (dx,dy): [0.,0.]: 0,0

Computed StopPos= 0.000000000D+00 0.000000000D+00 -1.427723273D+02

MACOS> ffpˆ	Find field angles that put the off-axis Enter element number: [16]: 6		spot right next to the occulting disk Enter offset vector in global units (dx,dy): [0.,0.]: 2d-5,2d-5

Field angle finder results:

Did	1 iterations, error= 1.424113342D-15

Old dx= 0.000000000D+00 New dx= 2.000000000D-05 0.000000000D+00	2.000000000D-05

Old crd= 0.000000000D+00 New crd= 8.916721422D-07 0.000000000D+00	-8.916721422D-07

1.000000000D+00	1.000000000D+00    Old crp= 0.000000000D+00 New crp= 1.094726641D-04 0.000000000D+00	-1.094726641D-04

-2.000000000D+01	-2.000000000D+01

Accept the new chief ray? [YES]: yes

MACOS> ors	Find new reference surfaces for diffraction propagation

Enter number of element to be optimized: [1]: 5

Reference surface optimization results:

TTL OPD= 1.656171318D-09 nCalls= 18

Old f= 6.329334641D+00	New f= 6.329334641D+00 Old z= 6.329334641D+00	New z= 6.329334641D+00 Old psi= 0.000000000D+00 New psi= -2.730547038D-06 0.000000000D+00	2.730547038D-06

1.000000000D+00	1.000000000D+00    Old Vpt= 0.000000000D+00 New Vpt= 3.728254596D-05 0.000000000D+00	-3.728254596D-05

-4.829334641D+00	-4.829334641D+00

Accept the new element data? [YES]: yes

MACOS> ors

Enter number of element to be optimized: [1]: 7

Reference surface optimization results:

TTL OPD= 1.656171779D-09 nCalls= 18

Old f= 6.329334641D+00	New f= 6.329334641D+00 Old z= -6.329334641D+00		New z= -6.329334641D+00 Old psi= 0.000000000D+00 New psi= 2.730547038D-06 0.000000000D+00	-2.730547038D-06

-1.000000000D+00	-1.000000000D+00   Old Vpt= 0.000000000D+00 New Vpt= 2.717454041D-06 0.000000000D+00	-2.717454041D-06

7.829334641D+00	7.829334641D+00

Accept the new element data? [YES]: yes

MACOS> fex	Find new exit pupil as well

Enter number of exit pupil return surface: [15]: 15

Tracing	333 rays...

Chief ray location: x=-2.0000000D-05 y= 2.0000000D-05 z=

1.5300000D+01

Centroid location: x=-2.0042495D-05 y= 2.0042495D-05 z=

1.5300000D+01

Centroid offset from chief ray: x= 4.2495119D-08 y=-4.2495119D-08 z=-6.7501560D-14

Exit pupil finder results:

Old f= 6.329334641D+00	New f= 7.324539719D+00

| Old z= | 6.329334641D+00 New z= | 7.324539719D+00 |
| --- | --- | --- |
| Old psi= | 0.000000000D+00 New psi= | -2.736348769D-06 |
|  | 0.000000000D+00 | 2.736348769D-06 |
|  | 1.000000000D+00 | 1.000000000D+00 |
| Old Vpt= | 0.000000000D+00 New Vpt= | 1.263889675D-13 |
|  | 0.000000000D+00 | -1.263889675D-13 |

Beam Propagation Commands

8.970665359D+00	7.975460281D+00

Accept the new element? [YES]: yes

MACOS> mod	This object is 100 times dimmer than the on-axis object

MOD: [q]: flux=0.01 Flux =	1.000000000D-02 MOD: [q]: q

MACOS> stretch

Enter image stretch type (LINEAR, LOG10, SQRT): [LINEAR]: log10

MACOS> int	This image is shown in Figure 54

Enter number of element where data is to be generated: [16]: 6

Tracing 12661 rays and propagating 262144 grid points... Prop to reference point Elt 5 and Elt 6 to WF 1:

z1= 6.3293D+00 dx1= 8.8773D-03 min= 8.8703D-03 max= 8.8832D-03 dev= 3.7730D-06 lin= 99.96%

z2= 1.0000D+30 dx2= 1.3925D-06

Compute time was	8.9688 sec Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D-02 u-v plane diam= 4.5362873D+00 du= 8.8772746D-03

x-y plane diam= 7.1158918D-04 dx= 1.3925425D-06

Peak intensity= 3.7145605D-04; Peak occurs at i= 257, j= 257 Sum of intensity= 8.6867251D-03

FIGURE 54 Off-axis image at the first focus, showing closeness to occulting mask.

MACOS> add	Now the off-axis star is added to the composed image (Figure 55)

Tracing 12661 rays and propagating 262144 grid points... Prop from reference point Elt 6 and Elt 7 to WF 1:

z1= 1.0000D+30 dx1= 1.3925D-06

z2=-6.3293D+00 dx2= 8.8773D-03 min= 8.8703D-03 max= 8.8832D-03 dev= 3.7728D-06 lin= 99.96%

Scalar FF Prop between Elt 15 and Elt 16 to WF 1:

z1= 7.3245D+00 dx1= 1.0273D-02 min= 1.0265D-02 max= 1.0280D-02 dev= 4.3667D-06 lin= 99.96%

z2= 0.0000D+00 dx2= 1.3925D-06

Compute time was	14.0371 sec Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D-02

u-v plane diam= 5.2495594D+00 du= 1.0273110D-02 x-y plane diam= 7.1158918D-04 dx= 1.3925425D-06

Peak intensity= 2.2644562D-04; Peak occurs at i= 257, j= 257 Sum of intensity= 6.8026488D-03

Image added. Plot composed image? [YES]: yes

FIGURE 55 Off-axis image added to composed detector image. Note that the image dynamic range is about 1, while the objects differ by 100 in flux.

#### GBLUR

The GBLUR command is used to simulate “long-exposure” jitter effects or other phe-nomena that cause smooth spreading of the image. It propagates the beam to the speci-fied surface, and then convolves the resulting intensity with a gaussian. It adds the result to the pixel array buffer (PixArray) and optionally displays it. The blurring is done at the WF sampling density, rather than the pixel density, which will generally be more accu-rate if the blurring occurs prior to the digitization of the image at the detector. Before using GBLUR, the user must invoke the COMPose command to set the detector pixel geometry. The width of the blur kernel is defined in the function arguments.

The following example sets up a typical case. The detector is first defined, at element 21, by specifying the number and size of the pixels. Note that the pixel size is specified to be the same as the WF pixel size (dx) for this example, which shows the blurred image at its highest resolution.

The GBLUR function is then invoked, first with a kernel width much smaller than the pixel size, and then with a kernel size much larger. The results

MACOS> compose

Enter element where image is to be composed: [21]: 21

Enter number of pixels per side: [128]: 128

Enter size of pixel: [1.000000E-03]: 1.0007177D-05

MACOS> gblur

Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D+00 u-v plane diam= 1.6365868D-01 du= 1.2886510D-03

x-y plane diam= 1.2709115D-03 dx= 1.0007177D-05

Peak intensity= 9.5305603D-02; Peak occurs at i=	65, j=	65 Sum of intensity= 8.6054104D-01

Enter Gaussian blur kernel width in base units (0,0): [0.,0.]: 1d-6,1d-6

Image added. Plot composed image? [YES]: yes

Beam Propagation Commands

Type <RETURN> for next page:

MACOS> gblur

Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Source Flux= 1.0000000D+00 u-v plane diam= 1.6365868D-01 du= 1.2886510D-03

x-y plane diam= 1.2709115D-03 dx= 1.0007177D-05

Peak intensity= 9.5305603D-02; Peak occurs at i=	65, j=	65 Sum of intensity= 8.6054104D-01

Enter Gaussian blur kernel width in base units (0,0): [0.,0.]: 300d-6,300d-6

Image added. Plot composed image? [YES]: yes

Type <RETURN> for next page: MACOS>

FIGURE 56 GBLUR example.

#### DAD

The DAD command displays the current COMposed image. This is useful if you have forgotten what the composed image looks like, or if you want to view it using a different stretch or plot type. It has no arguments.

#### GAIN

The GAIn command computes the ratio of the light diffracted by the optical system to a uniformly radiating point source of the same intensity. It is used mainly to evaluate the far-field performance of radio-frequency antennas. This command has one argument, the surface on which to compute the gain. An example is provided by the Luneberg Lens Antenna (Appendix A.3.3), and shown in Figure 58.

#### AMPLITUDE and PHASE

Each element in the MACOS wavefront propagation matrix is a complex number which can be expressed in terms of its real and imaginary components (i.e., a + b*i where a and b are real numbers). This complex number is a vector in the complex plane with two components, real and imaginary as shown in Figure 59.

FIGURE 57 Composed detector image displayed at linear stretch, slice plot type.

FIGURE 58 Far-field gain of Luneberg Lens Antenna.

imaginary

(a + bi)

b

a	real

FIGURE 59 Graphing numbers in the complex plane

The AMplitude and PHASE commands directly plot the corresponding components of the complex amplitude matrix. Figure 60 provides an example. The first plot shows the

Beam Propagation Commands

MACOS>amp

amplitude or length of the vector. The second plot shows the phase of the vector, or the angle which the vector makes with the real axis modulo 360 degrees. The AMplitude command requires the wavefront element number nElt as input.

This example uses CassWithExitPupil.in in Appendix A.1.4.

Enter element where WF is to be plotted:5 Wavefront plane 1 is evaluated at this element. Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Transmission Distance= 5.5601459D+00 u-v plane diam= 8.4230375E+00 du= 3.3031520E-02

x-y plane diam= 1.6767097E-04 dx= 6.5753318E-07

Amplitude	Phase

FIGURE 60 AMplitude example

#### REAL and IMAGINARY

As discussed in Section 7.2.11, the wavefront propagation matrix elements are complex numbers. The REAl and IMAGINARY commands display these two components of the complex amplitude matrix. Figure 61 shows the real and imaginary components. The first graph displays the real component of each complex pixel number in the array. The second graph shows the imaginary component. The single command argument is the element number of interest.

This example uses CassWithExitPupil.in in Appendix A.1.4.

MACOS>real

Enter element where WF is to be plotted:5 Wavefront plane 1 is evaluated at this element. Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Transmission Distance= 5.5601459D+00 u-v plane diam= 8.4230375E+00 du= 3.3031520E-02

x-y plane diam= 1.6767097E-04 dx= 6.5753318E-07 Type <RETURN> for next page:

Type <RETURN> for next page: MACOS>

Real part	Imaginary part

FIGURE 61 REAl example

### Image polarity and stretch-aware labels

Image polarity (IMGMODE).  Image-type plots (INTensity, PIXillated, OPD-as-image) can be rendered in either of two polarities:

Astronomical (default, 'ASTRO' or 'NEG'): large pixel values render dark, small values render bright.  Matches the PGPlot convention many MACOS users are accustomed to.

Conventional ('CONV' or 'POS'): large values bright, small values dark.  Matches general scientific imaging.

Toggle with the IMGMODE command:

MACOS> IMGMODE
    Enter image polarity (NEG/POS, ASTRO/CONV): [NEG]: CONV

The setting persists for the session.  It does not affect the non-image plot types (CONTOUR, SLICE, WIRE, SPOTDIAG, PLOTCOL), which always render with black ink on white.

Stretch-aware colorbar labels.  When STRETCH is active (LOG10 or SQRT), the colorbar wedge label reflects the active stretch:

INT, STRETCH=LIN: label 'Intensity'

INT, STRETCH=LOG10: label 'LOG10 Intensity'

INT, STRETCH=SQRT: label 'SQRT Intensity'

PIX, STRETCH=LOG10: label 'LOG10 Pixel value'

OPD with BaseUnits: label 'OPD (mm)' etc.

The label is set by the command handler before the draw routine emits it, so the polarity / stretch and the label always match.

Bottom-of-plot annotation.  Most image plot types now print one or two annotation lines just below the colorbar:

OPD: 'OPD=<rms> <unit> RMS, <pv> <unit> P-V'

SPOT: 'RMS spot radius=<r> <unit>'

INT: 'Peak pixel=<MaxInt>'

PIX: 'Peak pixel=<maxval>'

### Plot Type Commands

#### GRAY

The GRAy command resets the type of plot to GRAy scale. This is the default plot type. It has no arguments.

#### WIRE

The WIRe command takes all the values in an array and first plots them as three dimen-sional points. Then each point is then connected to its nearest neighbor on the grid by a line. Perspective is added and the obscured lines are removed. The result is a surface-like representation of the data as shown in Figure 62. There is no scale on the graph. The WIRe command is always used in conjunction with other commands and has no argu-ments.

MACOS>wire MACOS>in

Enter element where WF is to be plotted:5 Wavefront plane 1 is evaluated at this element. Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Transmission Distance= 5.5601459D+00 u-v plane diam= 8.4230375E+00 du= 3.3031520E-02

x-y plane diam= 1.6767097E-04 dx= 6.5753318E-07 Type <RETURN> for next page:

MACOS>

#### SLICE

The SLIce command is very similar to the WIRe command in that it displays the array of data as a surface. In this instance, the graph also contains a scale so the data values can be read directly. Like the WIRe command, SLIce is used with other commands and has no arguments.

Figure 57 provides an illustration of the SLICE plot type.

Plot Type Commands

FIGURE 62 WIRe plot

#### CONTOUR

The CONtour command sets the plot type to display data using a contour map (an eleva-tion map). Data points with similar values are connected by the same line. The CONtour map shows graphically those points with similar values. The CONtour command has no arguments.

This example uses CassWithExitPupil.in in Appendix A.1.4. Figure 63 shows the results.

MACOS>con MACOS>opd

Enter element where OPD is to be evaluated:5 Average OPD is -1.894794E-12

Average total path is 1.5622291803998E+01 Average delta path is -3.7770819600190E-01 RMS OPD error is 1.091947E-12

Type <RETURN> for next page:

MACOS>

#### COLUMN and ROW

The COLumn and ROW commands set the plot type to display a slice across the data sur-face. The slice is along a column or row of the data matrix, respectively. The user is prompted for the column or row number when the data is to be plotted out (for instance, when an INTensity command is issued). The COLumn and ROW commands have no arguments.

This example uses CassWithExitPupil.in in Appendix A.1.4. Figure 64 shows the results.

FIGURE 63 CONtour example

FIGURE 64 COLumn example

#### TEXT

The TEXt command writes the data points in the array to a file for further analysis. The file is in ASCII format and can be printed or read into a text editor. There are up to mdttl rows of numbers, each with up to mdttl numbers across: this can be quite a large file! The data points are the same as numbers used to create the graphical display. They are single precision. The TEXt command creates files as output and has no argu-ments. The filenames specify the contents and are of the format: [filename].[image type][surface number].txt where [image type] can be in (intensity), am (amplitude), ph (phase), re (real), im (imaginary), spot, and OPD.

This example uses CassWithExitPupil.in in Appendix A.1.4. The TEXt file is stored in CassWithExitPupil.int5.txt

Plot Type Commands

MACOS>text MACOS>intensity

Enter element where WF is to be plotted:5 Wavefront plane 1 is evaluated at this element. Wavefront Propagation Data Summary:

Wavelength= 1.0000000D-06; Transmission Distance= 5.5601459D+00 u-v plane diam= 8.4230375E+00 du= 3.3031520E-02

x-y plane diam= 1.6767097E-04 dx= 6.5753318E-07 Writing Wavefront Intensity, Elt=5

FORMATTED File=CassWithExitPupil.int5.txt MACOS>

#### BINARY

The BINary command writes numerical files which can be used by other signal/image processing software. It takes the array data and writes it out to a file in BINary format. The BINary command produces files for output and has no arguments. The filenames specify the contents and are of the format: [filename].[image type][surface number] where [image type] can be in (intensity), am (amplitude), ph (phase), re (real), im (imaginary), spot, and OPD.

#### MATLAB

The MATlab command writes the data arrays to MATlab format. The MATlab com-mand produces files for output and has no arguments. The filenames specify the con-tents and are of the format: [filename].[image type][surface number].dat where [image type] can be in (intensity), am (amplitude), ph (phase), re (real), im (imaginary), spot, and OPD.

#### FITS

The FITs command writes the data arrays to fits format files. The FITs command produces files for output and has no arguments. The filenames specify the contents and are of the format: [filename].[image type][surface number].fit, where [image type] can be in (intensity), am (amplitude), ph (phase), re (real), im (imaginary), and OPD.

Symbols

.filt 37

.filt-files 37

A

adaptive optics 71

ADD 27, 127, 131, 160

Akima 165

Aliasing 20

aliasing 19

AMplitude 27, 132, 159 amplitude

complex 19

AnaCoef 88

anamorphic coefficients 88

Anamorphic Surfaces 70

Aperture 37, 87

aperture 73

definition 88

shape 38

size 37

type 87, 88

Apertures 43

ApType 88

ApVec 88

ASCII 32

AsphCoef 65, 88

asphere 65

asphere coefficients 88

aspheric coefficient 65

ATMOS 72

B

BEAM 35

BEAm 27, 119, 159

Beam coordinates 15

beam coordinates 15, 39 see coordinates

beam 102

beam diameter 87

beam intensity 87

BeamType 87

BINARY 30

BINary 27, 32, 137, 162

Breckenridge 165

BUILD 92

BUIld 15

BUild 26, 142, 143, 162

BYE 25

C

captions. See light source CENTER 36, 102

CENTEr 26

CENter 102, 156

CENTRoid 26

ChfRayDir 36

ChfRayPos 36

chief ray 36

direction 87

direction vector 36

location 87

position vector 36

ChRayDir 87

ChRayPos 87

CMAtrix 32

C-matrix 142, 143

CmDM 32

CoatIndxElt 88 coating

layer index 88

layer thickness 88 number of layers 88

coatings 88

CoatThkElt 88

collimated beam 35

COLumn 27, 135, 162

comments

in input files 84 complex number 131

COMpose 27, 126, 160

conic

constant 65, 88

radius 88

CONtour 27, 135, 162

COORD 102

COOrd 26

coordinates beam 102

hexagaonal 57

local transformation matrix 89 number output 90

output transformation matrix 90 CosPower 87

CumRayL 32, 92

D

DAD 161

DADD 27

Deformable Mirrors 71 differential

chief ray 105

ray-trace 143 diffraction

exit pupil 115

grating 51

multi-plane 111

scalar 119

vector 119

diffraction grating 49

DMBUild 26

DMBuild 144, 162

DRAW 92

dummy command variables 150 Dummy Elements 53

Dumont 165

E

eccentricity 64, 88

eElt 64, 88

electric field vector 87 element

location 42

orientation 43

element coordinates 15

number 88

element type 42 elements

number of 87

EltName 88

EltType 88

END 26

Ex0 87

Example 95

EXEcute 26, 33, 156

ExInc 37, 87, 88

exit pupil 105

finding 105

EXPort 15, 26, 32, 148, 155

extinction coefficient 37, 87, 88

Ey0 87

F

far-field propagator 114 fast Fourier transform 19 fElt 64, 88

FEX 91, 105, 113, 159

FFP 26, 103

FFT 19

field angle 103, 104 field point

finding 103 file

path descriptors 84

file output 30

filter files 37

FITS 27, 30

FITs 137, 162

flat surface 64

Flower segmentation 61

Flux 37, 87, 121

focal length element 88

geometric 64

focal plane 55

FocalPln 42

focused beam 35

Fourier transformations 19 Fresnel

propagation distance 89

G

GAIn 131, 158

gap 87

Gaussian beam 119 geometric

focal length 64

Glass Tables 32

global coordinate system 13 Goodman 165

Graphics 29, 30, 31

Grating 42

grating 51

GRAy 134, 162

GRay 27

GridType 38, 87

H

h1HOE 88

h2HOE 88

Help 26, 155

hexagonal coordinates 57

HOE 42, 51

diffraction order 88

recording wavelength 89 holgraphic optical element 51 Hughes 165

Huygen’s Principle 17

I

iAp 87

iAsCoef 86

iAxis 86

iElt 86

iLayer 87

iLin 86

illumination profile 35 index

coating layers 88 index of refraction 87

real part 88

IndRef 37, 87, 88

influence functions 71

INTENSITY 113

INTensity 92, 94

INtensity 27, 122, 159

interpolated surface 72

iObs 86

iPert 86

iRay 93

iZern 87

J

JOUrnal 26, 33, 156

K

KcElt 65, 88

KrElt 65, 88

L

Lawrence 165

lens 42, 44

types in array 88 width in array 88

LensArray 42

LensArrayWidth 88

lenslet arrays 38

light source 36

flux 37

location 36

orientation 36

polychromatic 37

wavelength 37

linear model 139, 141

validity 139 linear ray state 140

LINtensity 26, 139, 147, 163

lMon 68, 88

LNEGok 27

LOAD 155

LOAd 84

local coordinates 15

LOG 160

LOg 27

LOPD 26

LOPd 139, 147, 163

LPErturb 26, 139, 162

LPIxilate 26, 147, 163

LPRead 26, 144, 163

LPReset 163

LREset 26, 145

LSPot 26, 139, 147, 163

M

MACOS data directory 32, 46

macos.glass 32, 46, 47

MACOS_DATA 32

macosData 32

Malacara 66

MAP 26, 93, 156

MATLAB 30

MATlab 27, 137, 162

Matlab 30

Matlab file 32

Maxwell’s equations 18

mdttl 113

memory allocation 34

MFILE 32

mirror 41, 43

MODIFY 155, 156

MODify 26, 85

MonCoef 88

monolithic 38

monomial coefficients 88

Monomial Surfaces 70

mPix 113

multi-plane diffraction 111

MULtispec 37

N

nCoatElt 88 near-field

propagator 114

nECoord 88

Negative obscurations 43

nElt 41, 87

NEW 26, 39

nGridPts 39, 87

nLayer 88

NObs 88

NOIse 27

NOLNeg 27

NomOPDMat 150

NONe 161

non-sequential surface reflective 62

refractive 64

NOPOL 26

NOPolarization 157

NOREGrid 27

nOutCord 90

nSeg 87

NSReflector 42

NSRefractor 42

NULl 161

numerical aperture 37

O

OASIS manual 165

OBS 26

Obscratn 37, 87

OBScuration 99, 158

obscuration 87

definition 88

type 88

obscuration size 37

Obscurations 43 obscurations

number 88

ObsSrf 42

ObsType 88

ObsVec 88

OLD 26, 84, 155

OPD 26, 94, 97, 157

optical

path difference 97

path length 140

sensitivity matrix 142

optical element 41 optical path length 140 OrderHOE 88

orientation 13

ORS 27, 91, 113, 118, 158

output coordinates 15

OutRaySpot 150

P

padding 20

paraxial wave equation 116 PARtials 26, 145, 162

PERturb 26, 107, 140, 157

Perturb 43, 49, 51 perturbation

file 108

PFP 26, 27, 104

PGPLOT 29, 31

physical optics 17

PinHole 88

PixArray 150, 160

PIXILATE 113

PIxilate 160

PIXillated 27

PLOcate 27, 124

Plot Type 134

plot type 27, 30

pMon 68, 88

point-spread function 20

POLarization 109, 157

POLarized 26

PREad 26, 108, 157

principal axis 89 propagation

type 89 propagator

far-field 114

near-field 114

PropType 89, 116

PSF 20

PsiElt 14

psiElt 43, 89

Q

Quit 25, 26

R

radius of curvature 65 RAY 95, 156

Ray 93 ray

direction 92

direction perturbation 140

geometric length 92

grid 38

optical length 92

optical path difference 140 position 92

refractive index 92

transverse aberration 140

undefined 94 ray grid

number points 87 ray state

exact 140

linear 140

RayDir 32, 92

RayIndex 92

RayL 92

RayPos 32, 92 rays

undefined 100

RAYtrace 26

Ray-tracing 16

REAl 133, 160

REal 27

Redding 165 reference

surface 53

wavelength 52

reference surfaces 19

Reflector 42

refractive index 36

Refractor 42

RefSrf 42

REGrid 27, 116, 159

RESet 26, 86, 155

return surface 56, 115 and linear models 143

ReturnSrf 42

RMSWFE 150

rotation point 15, 43, 89

ROW 27, 135, 162

RptElt 15, 89

RuleWidth 89

rxBeam 87

ryBeam 87

S

Sampling 19

sampling density 19

SAOImage 31

SAVe 26, 85, 155

SCAlar 27, 119, 158

scalar diffraction 119 SCOMP

ray-trace information 92, 93

SEEd 27

SegCoord 87

Segment 42 segment

location 87

width 87

segmented surface 38, 57 segments

number 87

Seigman 116

SHOw 26, 155

Siegman 165

single-plane diffraction 111

SINT 26, 109, 158

SLIce 27, 134, 162

source

distance 87

sphere 64

SPOt 26, 94, 99, 158

SRS 27, 91, 113, 118, 159

STAtus 26, 86, 155

STOP 26, 36, 101

STRetch 27

SUMmarize 26, 85, 155 surface

conic 65

flat 64

focal plane 55

holgraphic optical element 51 interpolated 72

reference 53

reflective non-sequential 62

refractive non-sequential 64

return 56

segmented 38, 57

spherical 64

Zernike 66

Surface coordinates 15

surface coordinates 15

surface type 42 surfaces

dummy 19

reference 19

Sziklas 116, 165

T

TElt 89

TEXT 30

TEXt 27, 32, 136, 162

TOut 90

U

UDSrfCoef 89

UDSrfType 89

User-Defined Surfaces 71

V

VECtor 27, 119, 158

vector diffraction 119

vertex 65, 89

vertex point 13

VptElt 13, 42, 89

W

WaveHOE 89

wavelen 37, 87

wavelength 37, 87

WFElt 113

width 87

WINdow 126

WIRe 27, 134, 161

X

xGrid 39, 87, 102

xMon 68, 89

xObs 89

Y

yGrid 39, 87, 102

yMon 68, 89

Yu 165

Z

Zelt 87

zElt 89

ZernCoef 90

Zernike coefficients 90

Zernike surface 66

zGrid 39, 87

zMon 68, 90

zSource 36, 87

## SECTION 8: Differential Ray-Tracing and Linear Optical Models

and Linear Optical Models

The MACOS differential ray-trace functions are used to compute optical sensitivity matrices. These provide linear models of the effects of optical element perturbations on optical performance. The linear models can be exercised internally to MACOS using functions such as LPErturb, LOPd and LINtensity which parallel commands intro-duced earlier. Linear optical models can also be EXPported for use external to MACOS.

BUild DMBuild PArtials LPErturb LPRead LREset LSPot LOPd

LPIxillate LINtensity EXPort

### Background

linear models of optical systems provide a linear matrix transformation between ele-ment perturbations and ray perturbations. These optical models can be used in place of the full-blown ray-trace calculations for systems that undergo only small changes. They can also be used in ways that the nonlinear models cannot, such as in linear statistical studies.

Linear models are generally valid where

- The small-angle approximation is valid for changes in ray direction

- The ray transverse aberration is “small” relative to the curvature of the optical ele-ments

The limits are not absolute, but vary from system to system. The user can check the range of validity of a linear model for a particular system. This is done by using the MACOS LPErtrurb, LOPd, LSPot and LINtensity commands to exercise the linear model. Results are compared with exact results produced for the same cases using the MACOS PERturb, OPD, SPOt and INtensity commands. If, over the expected range of perturbation values, the linear models produce results that are within the accuracy requirements of the particular analysis, then they can be safely used for that analysis. Otherwise a nonlinear SMACOS model should be used.

#### Linear Ray States

Rays are defined at any particular point in a beam train by their location, direction and length. In MACOS, the current exact ray state is defined as the position, direction and path length of a ray immediately after the current element (see Figure 65). The position of the iRayth ray is a 3-vector giving the point of incidence with the current element. MACOS stores it in the array RayPos as RayPos(1:3,iRay). The direction of the iRayth ray is a unit-magnitude 3-vector stored in RayDir(1:3,iRay). Note that if the current element is a mirror or lens, the direction is of the reflected or refracted ray. The optical path length of the iRayth ray from the source to the current element is a scalar and is stored in CumRayL(iRay).

Expanded view at element 4

iRay=122,iElt=1

CumRayL

RayDir

RayPos

Ray-trace

Ray state

FIGURE 65 Ray state

The effect of a perturbation in the optical system, such as a rotation or translation of an optical element, is to change the ray state at every downstream element. Such perturba-tions can be applied using the PERturb command, and the ray retraced to find the new ray state.

If a perturbation is sufficiently small, it will effect the rays linearly. That is, the new ray state at every downstream element is very nearly a linear function of the perturbation. This is true for some range of perturbation magnitudes that varies with each system, but is in general many times the wavelength of light.

MACOS linear modeling features use analytic differential ray-trace equations to com-pute the partial derivatives of the rays with respect to element perturbations. The differ-ential ray-trace equations are derived in References 1, 8, and 9. They provide a set of analytical formulas giving the ray partial derivative matrices in a form that is easily computed.

The linear ray state differs slightly from the exact ray state. The linear ray state is a sin-gle 7-vector, composed of a 3-vector of ray direction perturbation, a 3-vector of ray transverse aberration, and a scalar for ray optical path difference. It is written x

dr

x =  d

dL

(8.1)

where dr is the ray direction perturbation. The ray direction perturbation is a 3-vector whose magnitude is small compared to 1 and which is perpendicular to the ray direc-tion, RayDir. The total ray direction is equal to the vector sum of RayDir and dr (to the accuracy of the small angle approximation). Figure 66 shows the linear ray state.

### Background

r

RayDir

dr	Translation, i

r

d

s

Nominal surface

Rotation, i

RayPo

Source

CumRayL CumRayL

+ dL

ri-1

ri,nom

d

dri

Perturbed surface

ri

FIGURE 66 Linear ray state

The transverse aberration  is the lateral displacement of the ray. It is a vector perpen-dicular to the ray direction, from the nominal ray position to a point on the perturbed ray. It is not necessarily located on the surface of the element and is not, in general, the derivative of the ray position, RayPos. Exceptions are the focal-plane element type and any surface that is perpendicular to the nominal ray direction.

The path length perturbation, dL, is the difference between the optical path length of the perturbed and nominal rays. It is not the OPD, as that is the difference of the ray and the chief ray. However, the OPD is easily computed from dL as discussed below.

#### Linear Models

The linear ray state at a particular element is a function of perturbations of the upstream elements and of perturbations in the input ray. These perturbations are with respect to a “nominal state” xn nom , which may or may not be zero. For each ray at an element n, the linear ray state has the functional form:

xn = xn nom + xn u1 +  + xn un + xn x0

(8.1)

u1

un

x0

Here the u vectors are 6-vectors of element perturbations, composed of 3-vectors  for element rotational perturbation and  for element translation perturbation:





u =

(8.2)

The individual optical sensitivity matrices can be printed using the PARtials com-mand. The sensitivity of the nth ray to perturbations of the input (source) ray is a 7 by 7 matrix of the form:

xn

x0

=

rn

rn

r0

 n

r0

Ln

r0

 0

 n

 0

Ln

 0

(8.3)

rn

L0

 n

L0

Ln

L0

The sensitivity of the nth ray to perturbations of the ith element is a 7 by 6 matrix of the form:

rn rn

i i

 n   n

i i

Ln Ln

i i

xn =

ui

(8.4)

The complete expression for the linear ray state for a single ray in terms of element and input-ray perturbations is written in matrix form as:

x = xn nom +

xn xn  xn

(8.5)

x0 u1	un

x0 u1



un

The same form is extended to give the linear ray state for multiple rays at the same ele-ment

| xray1  xraym | = | xray1  xraym nom | + |  | xn xn  xn x0 u1 un ray1  xn xn  xn |  | x0 u1  un | (8.6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | x0 u1 un raym |  |  |  |

Both of these equations have the form x = xnom + Cu , where the C-matrix is the optical sensitivity matrix, giving the change in ray state due to perturbations of the optical ele-ments. The C-matrix provides a linear model of the optical system. The MACOS BUild command assembles these sensitivities from the individual ray partial derivatives.

It can be extremely useful to incorporate local coordinates directly into the optical sensi-tivity C-matrices. If the perturbations for one element are to be applied in coordinates other than the global coordinate system–perhaps in an actuator coordinate system–then that local coordinate transformation matrix can be included directly within the C-matrix. The transform from element iElt coordinates to global coordinates is written

GTiElt .

Commands for Building Models

Similarly, local coordinates can be used at the output end of the linear model. This allows for the creation of models for ray direction only, or transverse aberration or path length only. Focal-plane axes are easily incorporated. We write the transform for ray state from global coordinates to output coordinates as outTG . As an example, the 1x7 output transform matrix that captures only the path length ray state in global coordinates is

pathlengthTG  =

0 0 0 0 0 0 1

(8.7)

As another example, the 2x7 output transform which takes the transverse aberration in ray state and casts it into detector coordinates x and y is

detectorTG  =

0 0 0 xˆ 0

0 0 0 yˆ 0

(8.8)

Both actuator and detector coordinates are specified by the user as part of the .in-file prescription.

With the input and output coordinate matrices included, the C-matrix takes the form:

outTG xn GT0 xn GT1  xn GTn

xray1

	=

x0

u1



un

ray1

(8.9)

xraym

outTG xn G 0 xn G 1

xn G n

T		T

x0	u1

 	T

un

x0 u1



un

raynRay

### Commands for Building Models

#### BUILD

The BUild command traces all of the rays in a beam from the source to the specified final element. It then computes the full C-matrix linear model, giving the sensitivity of each ray at the last element to perturbations of each element in the beam train. It has one argument, the number iElt of the last element in the propagation.

BUild performs the MACOS differential ray-trace calculations, and must be performed before exercising commands that manipulate linear models, such as LPE, LOP, LSP, LIN, or before EXPorting C-matrix information.

Note that the BUild command ignores the presence of reference surfaces and obscuring surfaces unless they are the last surface. The BUild command currently does not work if there are return surfaces in the beam train.

MACOS>build

As an example, consider the Cassegrain.in (see Appendix A.1.2). This prescription has output coordinates set up to compute a wavefront model as discussed previously. Once the prescription is loaded using the OLD command, the C-matrix is easily built:

Enter terminal element number:4

Tracing 150 rays and BUILDing linear model... Compute time was 0.4141 sec

MACOS>

#### DMBUILD

The DMBuild command is not used in this version.

### Commands for Changing Models

#### LPERTURB

The LPErturb command allows the user to enter an element perturbation for use with the linear model analysis commands. LPErturb takes different arguments depending on whether element input coordinates are to be used. The first argument iElt is the number of the element to be perturbed. Note that element 0 (the source) cannot be per-turbed using LPErturb. The second argument is a yes-or-no response indicating whether input coordinates are to be used. If NO is selected, the next two prompts are for rotational and translational 3-vector perturbations. If YES is selected, a vector of the same length as the number of input coordinates is input in the input coordinate system.

After each LPErturb command, the current perturbation vector (in global coordinates) is printed out. The LPErturb command adds the perturbation data to the existing per-turbation vector. Multiple LPErturb commands sum each perturbation with the previ-ous perturbation vector. The perturbation vector can be reset to zero using the LREset command. Using Cassegrain.in:

MACOS>lperturb

Enter element to be perturbed:3 Use element coordinates? (NO):

Enter rotational perturbation vector (x,y,z):0,1e-7,0 Enter translational perturbation vector (x,y,z):0,0,0

Element	1 Linear Perturbations:

Rotational=	0.000000D+00 0.000000D+00 0.000000D+00 Translational=	0.000000D+00 0.000000D+00 0.000000D+00

Element	2 Linear Perturbations:

Rotational=	0.000000D+00 0.000000D+00 0.000000D+00 Translational=	0.000000D+00 0.000000D+00 0.000000D+00

Element	3 Linear Perturbations:

Rotational=	0.000000D+00 1.000000D-07 0.000000D+00 Translational=	0.000000D+00 0.000000D+00 0.000000D+00

Element	4 Linear Perturbations:

Rotational=	0.000000D+00 0.000000D+00 0.000000D+00 Translational=	0.000000D+00 0.000000D+00 0.000000D+00

Element	5 Linear Perturbations:

Rotational=	0.000000D+00 0.000000D+00 0.000000D+00 Translational=	0.000000D+00 0.000000D+00 0.000000D+00 MACOS>

Commands for Exercisizing Models

#### LPREAD

The LPRead command reads element perturbations from a specified datafile. MACOS assumes that the name of the perturbation file is infile.pert. If this file does not exist, MACOS prompts for the perturbation file name.

The perturbation file must be in the following all numeric format. The perturbations must be in global coordinates. The order of the perturbation vectors in the .pert-file must be identical to the order of the elements (the first perturbation vector corresponds to the first element, the sixth perturbation vector corresponds with the sixth element). Each element must have a perturbation vector. If an element is unperturbed, then a string of zeroes should be entered as its perturbation vector. The perturbation vectors have 6 components: a 3-vector rotational perturbations and a 3-vector translational perturba-tions. The 6 components must be separated by spaces.

MACOS>lpr FileCassegrain.pert read. MACOS>

#### LRESET

The LREset command zeros any previously entered linear perturbations. It should be used, when necessary, to reset the linear perturbation vector after LPErturb commands.

### Commands for Exercisizing Models

#### PARTIALS

The PARtials command prints individual optical sensitivity matrices for specified rays. The user is asked whether to print in input/output or global coordinates. For Cas-segrain.in, the partials of the chief ray in global coordinates are:

MACOS>partials

Enter ray number for partials (0=done, 1=chief ray):1 Print partials in actuator/sensor coordinates? (YES): no

Partial of ray	1 at Element	4 (Ref_surf) to ray at Element	0 (Inpu-tRay)

0.3036E+01	0.0000E+00	0.0000E+00 -0.4458E-01	0.0000E+00	0.0000E+00

0.0000E+00

0.0000E+00	0.3036E+01	0.0000E+00	0.0000E+00 -0.4458E-01	0.0000E+00

0.0000E+00

0.0000E+00	0.0000E+00	0.1000E+01	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00

0.5549E+01	0.0000E+00	0.0000E+00	0.2479E+00	0.0000E+00	0.0000E+00

0.0000E+00

0.0000E+00	0.5549E+01	0.0000E+00	0.0000E+00	0.2479E+00	0.0000E+00

0.0000E+00

0.0000E+00	0.0000E+00	0.1006E+02	0.0000E+00	0.0000E+00	0.1000E+01

0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.1000E+01

Partial of ray	1 at Element	4 (Ref_surf) to Element	2 (Primary ) per-turbations

0.0000E+00 -0.6606E+01	0.0000E+00	0.6117E+00	0.0000E+00	0.0000E+00

0.6606E+01	0.0000E+00	0.0000E+00	0.0000E+00	0.6117E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

| 0.0000E+00 | -0.8122E+01 | 0.0000E+00 | 0.7521E+00 | 0.0000E+00 | 0.0000E+00 |
| --- | --- | --- | --- | --- | --- |
| 0.8122E+01 | 0.0000E+00 | 0.0000E+00 | 0.0000E+00 | 0.7521E+00 | 0.0000E+00 |
| 0.0000E+00 | 0.0000E+00 | 0.0000E+00 | 0.0000E+00 | 0.0000E+00 | 0.0000E+00 |
| 0.0000E+00 | 0.0000E+00 | 0.0000E+00 | 0.0000E+00 | 0.0000E+00 | 0.2000E+01 |

Partial of ray	1 at Element	4 (Ref_surf) to Element	3 (Secondar) per-turbations

0.0000E+00	0.1241E+01	0.0000E+00 -0.5671E+00	0.0000E+00	0.0000E+00

-0.1241E+01	0.0000E+00	0.0000E+00	0.0000E+00 -0.5671E+00	0.0000E+00 0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00 -0.2000E+01

Partial of ray	1 at Element	4 (Ref_surf) to Element	4 (Ref_surf) per-turbations

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

Enter ray number for partials (0=done, 1=chief ray):

The specific partials take the form in Eqns. 8.3 and 8.4. The first matrix shows that source chief ray direction perturbations are magnified 3.036 times at the reference sur-face. The effect on ray direction at the reference surface of primary mirror tilts and decenters are magnified by 6.606 and 0.5671 times, respectively.

The path length partials show a factor-of-two sensitivity for this ray. Looking at the OPD partials only for a marginal ray, these numbers change:

Enter ray number for partials (0=done, 1=chief ray):2 Print partials in actuator/sensor coordinates? (YES): y

Partial of ray	2 at Element	4 (Ref_surf) to ray at Element	0 (Inpu-tRay)

0.2996E+01 -0.2639E-17	0.1976E-16 -0.4450E-01	0.3920E-19	0.0000E+00

0.0000E+00

-0.2639E-17	0.2984E+01	0.8899E-01	0.3920E-19 -0.4432E-01	0.0000E+00 0.0000E+00

0.5643E+01 -0.4971E-17	0.1977E-15	0.2500E+00 -0.2202E-18	0.1976E-

16	0.0000E+00

-0.4971E-17	0.5620E+01	0.8902E+00 -0.2202E-18	0.2490E+00	0.8899E-01	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.1000E+01

Partial of ray	2 at Element	4 (Ref_surf) to Element	2 (Primary ) per-turbations

| -0.4462E-16 | -0.6401E+01 | 0.0000E+00 | 0.5827E+00 | -0.8407E-17 | 0.4262E-16 |
| --- | --- | --- | --- | --- | --- |
| 0.6200E+01 | 0.4462E-16 | 0.0000E+00 | -0.8407E-17 | 0.5448E+00 | 0.1920E+00 |
| -0.1069E-15 | -0.8239E+01 | 0.0000E+00 | 0.7500E+00 | -0.1533E-16 | 0.7920E-16 |
| 0.7757E+01 | 0.1069E-15 | 0.0000E+00 | -0.1533E-16 | 0.6810E+00 | 0.3567E+00 |
| -0.3934E+01 | 0.8735E-15 | 0.0000E+00 | -0.7951E-16 | -0.3581E+00 | 0.1934E+01 |

Partial of ray	2 at Element	4 (Ref_surf) to Element	3 (Secondar) per-turbations

-0.3440E-17	0.1228E+01	0.5551E-16 -0.5382E+00	0.8367E-17 -0.4262E-16

-0.1244E+01	0.3440E-17	0.0000E+00	0.8367E-17 -0.5005E+00 -0.1920E+00 0.3146E-16 -0.6986E-32	0.0000E+00	0.3062E-32	0.1379E-16 -0.9888E-16

0.1417E+00 -0.3146E-16	0.0000E+00	0.1379E-16	0.6210E-01 -0.4453E+00

0.6140E+00 -0.1363E-15	0.0000E+00	0.5975E-16	0.2691E+00 -0.1930E+01

Partial of ray	2 at Element	4 (Ref_surf) to Element	4 (Ref_surf) per-turbations

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

Commands for Exercisizing Models

0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00	0.0000E+00

Enter ray number for partials (0=done, 1=chief ray):

#### LOPD

The LOPd command generates an OPD map of the beam at the final surface of the linear model. It parallels the OPD command discussed in Section 5.2.5. Using Cassegrain.in:

MACOS>lopd

Tracing perturbed system Average OPD is 1.822578E-12

RMS OPD including piston is 3.023756E-08 RMS WFE excluding piston is 3.023755E-08 Type <RETURN> for next page:

#### LSPOT

The LSPot command generates a spot diagram of the beam at the final surface of the linear model. It parallels the SPOt command discussed in Section 5.2.6. Using Casseg-rain.in:

MACOS>lspot

Tracing perturbed system Average OPD is -9.502741E-07

RMS OPD including piston is 1.081200E-01 RMS WFE excluding piston is 1.081200E-01 Type <RETURN> for next page:

#### LINTENSITY

The LINtensity command computes beam wavefront phase at the final surface of the linear model and then performs a single far-field propagation to the next element in the prescription. If the last element in the linear model is the last element in the prescription, the user is prompted for transmission distance and other information. LINtensity per-forms calculations that parallel the propagation commands discussed in Sections 6.3 and 7.2.1, respectively. Using Cassegrain.in:

MACOS>lintensity Tracing perturbed system

Average OPD is 1.822578E-12

RMS OPD including piston is 3.023756E-08 RMS WFE excluding piston is 3.023755E-08 FFT/Point Spread Function Data Summary:

Wavelength= 9.9999999748E-07; Transmission Distance= 5.5601458549E+00 u-v plane diam= 1.8044868469E+01 du= 7.0764191449E-02

x-y plane diam= 7.8265948105E-05 dx= 3.0692527275E-07

Type <RETURN> for next page:

#### LPIXILATE

The LPIxilate command computes beam wavefront phase at the final surface of the linear model and then performs a single far-field propagation to the next element in the prescription. If the last element in the linear model is the last element in the prescription, the user is prompted for transmission distance and other information. Finaly, the image

MACOS>lpix

is pixilated. The LPIxilate command parallels the PIxilate command discussed Sec-tion 6.3. Using Cassegrain.in:

Tracing perturbed system Average OPD is 1.822375E-12

RMS OPD including piston is 2.104591E-12 RMS WFE excluding piston is 1.052736E-12 FFT/Point Spread Function Data Summary:

Wavelength= 9.9999999748E-07; Transmission Distance= 5.5601458549E+00 u-v plane diam= 1.8044868469E+01 du= 7.0764191449E-02

x-y plane diam= 7.8265948105E-05 dx= 3.0692527275E-07

Enter number of pixels per side:10 Enter size of pixel:1e-7

Type <RETURN> for next page:

### Commands for Exporting Models

The EXPort command provides a way to export the C-Matrix of the linear model and is discussed in Section 3.8.5.

## SECTION 9: Subroutine MACOS and User Extensions

Nonlinear Optical Models

Subroutine MACOS (SMACOS) provides the full functionality of MACOS in a Fortran subroutine. Regular MACOS commands are available to any other program via a For-tran CALL statement. This provides a nonlinear optics modeling capability for dynamic simulations or optimization programs. This section describes SMACOS in 4 subsec-tions:

- Background

- Passing data between SMACOS and user applications

- Example

- Command reference

### Background

Certain tasks require a full nonlinear optical modeling capability in a form that can be directly called by another program. Examples include

- Dynamic simulation of integrated optics/structures/controls systems where struc-tural motions determine optical outputs such as focal-plane images and drive con-troller actions

- Monte Carlo studies of optical systems where uncertain parameters are varied ran-domly, collected, and analyzed statistically

- Optical parameter estimation where optical model parameters are determined by driving modelled optical outputs, such as images, to match actual measured outputs

- Design optimization where sets of parameters are varied to extremalize an optical performance metric

SMACOS consists of source code files, object code files, and include files that are com-piled and linked with the user’s main program.

To use SMACOS, several steps must be followed:

- Develop .in-file optical prescriptions for each system to be modeled

- In the user’s main program, use Fortran include statements to define SMACOS vari-ables (see Section 9.2)

- Set up the sequence of commands in the main program using SMACOS subroutine calls

- Compile, link, and execute the code

The process of using SMACOS begins with the creation of standard MACOS .in-file optical prescriptions. This can be done interactively using the standard MACOS appli-cation as described in Section 4 or an existing .in-file can be modified using a text editor.

The prescription should be checked to verify that the light PROpagates through the sys-tems and the expected images are produced.

SMACOS normally includes the standard MACOS graphics. In some cases this may not be desirable: these routines take up appreciable memory in the computer. An alternative library of routines is available for applications that do not require graphical output.

### Call Line Variables

The user’s main program must define the SMACOS call line dummy variables. SMA-COS is called:

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

&	OPDMat,RaySpot,RMSWFE,PixArray)

The variables are explained in below.

#### Dummy Command Variables

SMACOS calls using the following dummy variables to pass the arguments needed to carry out SMACOS commands:

      - The SMACOS dummy character variable command is set to the MACOS command name: command=’OLD’

      - Any character arguments are stored in the SMACOS character argument array:

CARG(1)=’Cassegrain’

      - Any logical arguments are stored in the SMACOS logical argument array:

LARG(1)=.TRUE.

      - Any double-precision arguments are stored in the SMACOS double precision argu-ment array: DARG(1)=3.1415926d0

      - Any single-precision arguments are stored in the SMACOS double precision argu-ment array: RARG(1)=3.1416e0

      - Any integer arguments are stored in the SMACOS integer argument array:

IARG(1)=12

These dummy variables are dimensioned:

CHARACTER*132 command CHARACTER*32 CARG(9) REAL*8 DARG(9)) INTEGER IARG(9) LOGICAL LARG

REAL*4 RARG(9)

#### Dummy Output Variables

SMACOS uses the following dummy variables to pass output:

      - OutRaySpot: spot diagram data generated by SPOt and LSPot

      - RMSWFE: RMS wavefront error generated by OPD and LOPd

      - OPDMat: wavefront generated by OPD and LOPd

      - PixArray: image data generated by ADD, BLUR, PIX, LPIX and other commands These dummy variables are dimensioned:

REAL*8 RaySpot(mRay, 2), OPDMat(mpts, mpts), RMSWFE REAL*4 PixArray(mPix, mPix)

Call Line Variables

#### Common Blocks and Include Files

An alternate means of addressing SMACOS data structures is via the various SMACOS common blocks. Inclusion of the SMACOS commons allows the user to directly modify data through Fortran assignment statements in the calling program. It also allows access to all of the data products generated by SMACOS analysis functions, not just those in the call line.

Access to the SMACOS commons from a user-written Fortran program can be provided by including the provided smacosvars.cmn file. This file is shown here:

C*********************************************************************** C  Begin file smacosvars.cmn

C  +	+

C | Copyright (C) 1995-9, California Institute of Technology. | C | U.S. Government Sponsorship under NASA Contract NAS7-918 | C  | is acknowledged.	|

C	+	+

C***********************************************************************

C	This file defines call-line variables and common blocks that give C	user programs direct access to MACOS variables. The form of the C	SMACOS call line is:

C	CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

C	&	OPDMat,RaySpot,RMSWFE,PixArray)

C	The param.cmn file defines array sizes for SMACOS call-line and

C	internal MACOS variables. The param.cmn file should be copied from C	param128.cmn, param256.cmn, etc. -- whichever size of SMACOS you C	are using:

INCLUDE 'param.cmn'

C	These are the SMACOS call-line variables:

CHARACTER*132 command CHARACTER*32 CARG(9)

REAL*8 DARG(9), RaySpot(mRay, 2), OPDMat(mpts, mpts) INTEGER IARG(9)

LOGICAL LARG

REAL*4 RARG(9), RMSWFE, PixArray(mPix, mPix)

C	These are other SMACOS common blocks, that you may use or comment C	out as you wish. The ifEcho flag, if set to .TRUE., provides for C	full printout of SMACOS operations. If set to .FALSE., reduced

C	printout is provided.

LOGICAL ifEcho COMMON /SCIO/ifEcho

INTEGER npts,NoiseSeed(2),iSpot

REAL*8 xcent,ycent,xLocal(3),yLocal(3),zLocal(3), & CntrSpot(3),RefSpot(2)

COMMON /UserCommon4/npts,NoiseSeed,iSpot

COMMON /UserCommon8/xcent,ycent,xLocal,yLocal,zLocal, & CntrSpot,RefSpot

C	These includes provide access to internal MACOS variables. They can

C	be commented out if you do not plan to use them:

INCLUDE 'elt.cmn' INCLUDE 'src.cmn'

C	These includes provide access to internal MACOS scratch arrays. They C	should be commented out if you do not plan to use them:

LOGICAL L1(md2)

INTEGER DrawEltVec(mDrawElt,mDrawRay) REAL*4 R1(mdttl,mdttl),RV1(md2),RV2(md2) REAL*8 D1(mdttl,mdttl),D2(mdttl,mdttl) REAL*8 DV1(md2),DV2(md2)

COMMON /SCRATCH/ D1,D2

EQUIVALENCE (D1(1,1),RV1(1)),(D1(1,1),DV1(1)),(D2(1,1),DV2(1)), & (D1(1,1),R1(1,1)),(D2(1,1),RV2(1))

c END of smacosvars.cmn C***********************************************************************

Most of the useful data is buried in the elt.cmn and src.cmn include files invoked above. The elt.cmn file is shown below:

C*********************************************************************** C	Begin file elt.cmn

C	+	+

C	| Copyright (C) 1995-9, California Institute of Technology. | C	| U.S. Government Sponsorship under NASA Contract NAS7-918	|

C	| is acknowledged.	|

C	+	+

C*********************************************************************** C	Variable definitions

LOGICAL LRayToElt(0:mElt,bRay),LRayOK(mRay),LEltOK(mElt),

& LRayPass(mRay),LEltObs(mElt),ifComputeDZ(mIntSrf),LGlass(mElt)

CHARACTER*16 EltName(0:mElt),EltTypeName(20),SrfTypeName(20), & ApTypeName(0:2),ObsTypeName(-5:5),PropTypeName(11),

& GridTypeName(5),ZernTypeNameL(3),GlassElt(mElt),BaseUnits, & WaveUnits,GlassName(mGlass)

INTEGER nElt,nRay,EltType(0:mElt),nECoord(0:mElt),nOutCord, & nSeg,SegCoord(3,mElt),EltToSegMap(mElt),SegToEltMap(mEm1),

& RayToSegMap(mRay,mSegMirs),RayID(mdttl,mdttl),RayWFMap(2,mRay), & PropType(0:mElt),ApType(mElt),nObs(mElt),ObsType(mObs,mElt),

& iEltToiWF(mElt),SrfType(mElt),OpType(mElt),iEltToIntSrf(mElt), & IWORK(LIWORK,mIntSrf),nDP(mIntSrf),LensArrayType(mElt),

& nCoatElt(mElt),UDSrfType(0:mElt),EltID(0:mElt),ZernaType(mElt), & ZernTypeL(mElt),nRayToSeg(mRay),iEltToGridSrf(mElt),

& nGridMat(mElt),spcOption

REAL*4 dxidxim1(7,7,mElt),dxidui(7,6,mElt), & MWFFT(mdttl,mdttl),XFP(mdttl),YFP(mdttl)

REAL*8 Wavelen,VptElt(3,mElt),RptElt(3,mElt),fElt(mElt),Flux, & eElt(mElt),psiElt(3,mElt),RayPos(3,mRay),

& RayDir(3,mRay),RayL(mRay),dxElt(0:mElt),

& IndRef(0:mElt),CumRayL(mRay),TElt(6,6,mElt),Tout(7,7),

& ApVec(6,mElt),ObsVec(6,mObs,mElt),xObs(3,mElt),zElt(0:mElt), & KcElt(mElt),KrElt(mElt),AsphCoef(4,mElt),MonCoef(120,mElt), & ZernCoef(45,mElt),pMon(3,mElt),xMon(3,mElt),yMon(3,mElt),

& zMon(3,mElt),lMon(mElt),zSource,XYZSRF(mDP,3,mIntSrf),

Call Line Variables

& DZSRF(2,mDP,mIntSrf),WavelenUM,WavelenConv,

& LensArrayWidth(mElt),h1HOE(3,mElt),h2HOE(3,mElt),

& OrderHOE(mElt),WaveHOE(mElt),PinHole(mElt),Extinc(0:mElt), & CoatIndxElt(20,mElt),CoatThkElt(20,mElt),AnaCoef(4,mElt),

& RuleWidth(mElt),UDSrfCoef(1500,mElt),yObs(3,mElt),zObs(3,mElt), & DWORK(mDP,mIntSrf),UDSrfParam(10,mElt),

& GridMat(mGridMat,mGridMat,mGridSrf),GridSrfdx(mGridSrf), & GlassCoef(6,mElt),GlassTable(6,mGlass),WaveBU,CBM,CWM,CWB

COMPLEX*16 WFElt(mdttl,mdttl,mWF),RayE(3,mRay), & WFbuff(mdttl,mdttl),JmatElt(2,2,mElt)

C	COMMON block definitions

COMMON /EltLog/ LRayToElt,LRayOK,LEltOK,LRayPass,LEltObs, & ifComputeDZ,LGlass

COMMON /EltChar/ EltName,EltTypeName,SrfTypeName, & ApTypeName,ObsTypeName,PropTypeName,GridTypeName,

& GlassElt,ZernTypeNameL,BaseUnits,WaveUnits,GlassName

COMMON /EltInt/ nElt,nRay,EltType,nECoord,nOutCord,nSeg, & SegCoord,EltToSegMap,SegToEltMap,RayToSegMap,

& RayID,RayWFMap,PropType,ApType,ObsType,nObs,

& iEltToiWF,SrfType,OpType,iEltToIntSrf,IWORK,nDP,LensArrayType, & nCoatElt,UDSrfType,EltID,ZernTypeL,nRayToSeg,iEltToGridSrf,

& nGridMat,spcOption

COMMON /EltReal/ dxidxim1,dxidui,MWFFT,XFP,YFP

COMMON /EltDble/ Wavelen,VptElt,RptElt,fElt,eElt,psiElt,

& RayPos,RayDir,RayL,IndRef,CumRayL,TElt,Tout,Flux,ApVec,ObsVec, & xObs,dxElt,zElt,KcElt,KrElt,AsphCoef,MonCoef,ZernCoef,pMon,

& xMon,yMon,zMon,lMon,zSource,XYZSRF,DZSRF,LensArrayWidth, & h1HOE,h2HOE,OrderHOE,WaveHOE,RuleWidth,

& PinHole,Extinc,CoatIndxElt,CoatThkElt,AnaCoef,

& UDSrfCoef,yObs,zObs,DWORK,UDSrfParam,GridMat,GridSrfdx, & WavelenUM,WavelenConv,GlassCoef,GlassTable,WaveBU,CBM,CWM

COMMON /EltCmplx/ WFElt,RayE,JmatElt

SAVE /EltCmplx/ SAVE /EltDble/ SAVE /EltReal/ SAVE /EltInt/ SAVE /EltLog/ SAVE /EltChar/

C End Element Definition Include File - 'elt.cmn' C***********************************************************************

Some of the important variables in this common file have been identified elsewhere in this manual. See Section 4.11 for a summary of variables used in the basic optical pre-scription. Variables containing the data generated by ray-trace, diffraction, and linear-ized ray-trace commands include:

- CumRayL(iRay): Cumulative ray optical pathlength at the current element

- RayDir(iRay): Ray direction at the current element

- RayPos(iRay): Ray position at the current element

- WFElt(i,j,iWF): Complex amplitude at the current element

- dxElt(iElt): Ray spacing at the current element

- Cmatrix: Matrix of ray partial derivatives at the current element

### Specification of Model Size in SMACOS

A new subroutine is added to MACOS 3.2b, as part of the SMACOS library, that allows the user to change model size at runtime. The subroutine call syntax is

CALL SMACOS_Set_Model_Size(model_size)

where the argument model_size is a either a constant or a variale for the specified model size. The call to SMACOS_Set_Model_Size will release the dynamic storage used for the existing model size, and allocate storage for the new model size. It also does some necessary setup to make sure a proper SMACOS initialization is performed when the model size is changed. The model parameter file macos_param.txt must be accessible from the user’s execution directory as discussed in Section 3.3.

### Example

MACOS>old

The user code accesses MACOS functions using the SMACOS call line in a fashion analogous to the MACOS command line. For instance, the MACOS command sequence to load Cassegrain.in and compute the intensity at the final element is:

Enter file name: Cassegrain

Input file Cassegrain.in being loaded. Optical Train Summary:

Tracing rays past	5 elements.

Aperture type =	1 (Annular	) with 15 grid points.

Elt	1: Obscuration	with Kc= 0.0000D+00, Kr=-1.0000D+20, nECoords

= -6

Elt	2: Conic Mirror	with Kc=-1.0000D+00, Kr=-1.0800D+01, nECoords

= -6

Elt	3: Conic Mirror	with Kc=-2.6706D+00, Kr=-3.5268D+00, nECoords

= -6

Elt	4: Reference Surface	with Kc= 0.0000D+00, Kr=-5.5601D+00, nECoords

= -6

Elt	5: Focal Plane	with Kc= 0.0000D+00, Kr=-1.0000D+18, nECoords

= -6

Number of output coordinates is	5 MACOS>intensity

Enter last element for propagation:5

Tracing	4396 rays and propagating	65536 grid points... FF Prop between Elt 4 and Elt 5 to WF 1:

z1= 5.5601D+00 dx1= 7.0764D-02 min= 7.0704D-02 max= 7.0825D-02 dev= 3.5308D-05 lin= 99.95%

z2= 0.0000D+00 dx2= 3.0693D-07

Compute time was 0.3236 sec MACOS>

The same functions can be performed using SMACOS calls as follows:

C Load Cassegrain.in prescription file command=’OLD’ CARG(1)=’Cassegrain’

CALL SMACOS(command,CARG,DARG,IARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

Commands

C Propagate

command=’INTENSITY’ IARG(1)=5

CALL SMACOS(command,CARG,DARG,IARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

The SMACOS call line passes results of some calculations in dummy output arrays (see Section 9.2.2).

### Commands

Every MACOS command is available as an SMACOS command, with the exception of NEW. The MOD command has a different form in SMACOS than in MACOS, as it is expected that the SMACOS user will prefer to directly access and change any data val-ues, rather than incur the overhead of a full SMACOS call. The SMACOS MOD com-mand simply resets the current ray-trace and WF element counters to 0, so that subsequent computations will start from the source. It has no arguments.

One command is available for SMACOS that is not used in MACOS: the NONe com-mand, which turns off the MACOS plot functions.

This section summarizes most of the SMACOS commands and their arguments.

#### File and Data Manipulation

The SMACOS commands for loading .in-files, displaying element data and export-ing data are:

      - LOAD or OLD: load existing optical system dataset. This command has one argument, the name of the prescription file (minus the .in suffix). The command is set up in the main program as follows:

command='OLD'

CARG(1)=filename

and is executed using the call statement:

CALL SMACOS(command,CARG,DARG,IARG,RARG,…)

The call is the same for all commands and so is not repeated for each.

      - Help: display a summary of the SMACOS commands. This command has no argu-ments. The command is set up in the main program as follows:

command='HELP'

      - SUMmarize: display a summary of the optical system data on the user’s terminal. This command has no arguments. It is set up:

command='SUMMARIZE'

      - STAtus: display a summary of the status of the current calculations. This command has no arguments. It is set up:

command='STATUS'

      - RESet: reset various internal flags and counters to their initial values. This has the effect of forcing the next calculations to be performed as if the current file was just loaded. This command has no arguments. It is set up:

command='RESET'

      - SHOw: display the data defining a specified element. This command has 1 integer argument: the number of the element to be displayed. The source is considered to be element 0. It is set up:

command='SHOW' IARG(1)=iElt

      - SAVe: write out the current optical data as a new .in-file. This command has 1 char-acter argument: the name of the new file. It is set up:

command='SAVE' CARG(1)=new_filename

CARG(2)=’YES’ (to overwrite if file exists)

      - EXPort: this function writes specified data to disk using one of three formats: MAT-LAB “.m-file”, fits, or Fortran binary formats. It is set up differently depending on the particular variable to be exported, its form, etc. The user is advised to prototype the particular command sequence using MACOS, and then set up the appropriate sequence of arguments.

command='EXPORT' IARG(1)=… CARG(1)=…

      - EXEcute: runs a journal file. This command has 1 character argument, namely the name of the journal file to be executed. The command is set up in the main program as follows:

command='EXECUTE' CARG(1)=’journal_file_name’

      - JOUrnal: writes a journal file. This command has 1 character argument, namely the name of the journal file to be written. The command is set up as follows:

command='JOURNAL' CARG(1)=’journal_file_name’ CARG(2)=’YES’ (to overwrite if file exists)

      - MODify: resets the surface counters for ray-trace and diffraction computations, so that changes to elements will be picked up. This command has no arguments. It is set up as follows:

command='MODIFY'

Note that the SMACOS MODify command (unlike the MACOS MODify command) does not change any data elements. It is up to the user to do this directly, by assignment statements in the calling pro-gram.

#### Ray-Trace Analysis

      - MAP: this command displays and prints a cross-sectional map of the source light beam, as described in Section 5.2.2. It has no arguments. The command is set up as follows:

command='MAP'

      - RAY: trace a specified ray through the entire system. This command has one argu-ment, the ray number. The command is set up as follows:

command='RAY' IARG(1)=iRay

      - FFP: find field point corresponding to a specified chief ray offset at a particular opti-cal element. This command has 4 arguments: the element number of the surface at which the chief ray intercept is specified, the offset of the chief ray from the vertex of the element, and a prompt response to accept the new source parameters. The command is set up as follows:

command='FFP' IARG(1)=iElt

DARG(1)=chief ray offset x in base units DARG(2)=chief ray offset y in base units CARG(1)=’YES’ (to accept changes)

      - PFP: find field point corresponding to a specified chief ray offset in pixel coordinates

at a detector. This command has 4 arguments: the element number of the surface at

Commands

which the chief ray intercept is specified, the size of the detector pixels, the offset of the chief ray from the vertex, and a prompt response to accept the new source parameters. The command is set up as follows:

command='PFP' IARG(1)=iElt

DARG(1)=pixel size in base units DARG(2)=chief ray offset x in pixel units DARG(3)=chief ray offset y in pixel units CARG(1)=’YES’ (to accept changes)

      - CENter: center the beam in the aperture stop. This command has three arguments: the element number of the aperture stop and its offset from the vertex. The command is set up as follows:

command='CENTER' IARG(1)=iElt

DARG(1)=stop offset x DARG(2)=stop offset y CARG(1)=’YES’ (to accept changes)

      - POLarization: turn on polarization ray-tracing. This command has 4 arguments specifying the polarization state of the source rays. The command is set up as fol-lows:

command='POL' DARG(1)=TEM X (real part)

DARG(2)=TEM X (imaginary part) DARG(3)=TEM Y (real part) DARG(4)=TEM Y (imaginary part)

      - NOPolarization: turn off polarization ray-tracing. This command has no argu-ments. The command is set up as follows:

command='NOPOL'

      - PERturb: rotate and/or translate a specified optical element. This command has sev-eral arguments. An integer argument is used to specify the number of the element to be perturbed (element 0 is the source). Up to 9 double precision arguments are used to specify the perturbations. A character argument is used to specify global or ele-ment coordinates. The command is set up as follows:

command='PERTURB' IARG(1)=iElt

If iElt=0 (the source is to be perturbed) then the perturbation is entered in global coordinates

DARG(1)=rotational perturbation x DARG(2)=rotational perturbation y DARG(3)=rotational perturbation z DARG(4)=translational perturbation x DARG(5)=translational y DARG(6)=translational z

and the system aperture stop location is entered:

DARG(7)=stop x DARG(8)=stop y DARG(9)=stop z

If iElt is not zero (an optical element is to be perturbed) then the coordinate system for the perturbation is specified using a character argument:

CARG(1)=eltcoord or globalcoord

and if element coordinates were selected, the perturbations are :

DARG(1)=perturbation in element coordinate 1 DARG(2)=perturbation in element coordinate 2 DARG(3)=perturbation in element coordinate 3 DARG(4)=perturbation in element coordinate 4 DARG(5)=perturbation in element coordinate 5

DARG(6)=perturbation in element coordinate 6

or if global coordinates were selected:

DARG(1)=rotational perturbation x DARG(2)=rotational perturbation y DARG(3)=rotational perturbation z DARG(4)=translational perturbation x DARG(5)=translational perturbation y DARG(6)=translational perturbation z

      - PREad: reads perturbations from an input file. This command has 1 argument, the name of the files. The command is set up as follows:

command='PREAD' CARG(1)=filename

      - OPD: trace the full set of rays from the source (or from the current element if an OPD, SPOt, or PROpagate command has been performed previously) up to a specified element, and then calculate the OPD distribution across that element. The result is stored in the array NomOPDMat and passed in the SMACOS call line. Depending on the selected plot option, the OPD map may be plotted. This command has one argu-ment, the element number. The command is set up as follows:

command='OPD' IARG(1)=iElt

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - SPOt: trace the full set of rays from the source (or from the current element if an OPD, SPOt, or PROpagate command has been performed previously) up to a speci-fied element, and then calculate the spot diagram at that element. The result is stored in the array OutRaySpot and passed in the SMACOS call line. Depending on the selected plot option, the spot diagram may be plotted. The way obscured rays are handled depends on the setting of the OBS option. This command has one argument, the element number. The command is set up as follows:

command='SPOT' IARG(1)=iElt

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

The SPOt command expects 5 output coordinates.

      - OBScuration: specify the way obscured rays are handled for SPOt diagrams. This command has one argument, the obscuration option number. If this is set to 0, all rays are plotted; if set to 1, only unobscured rays are plotted; if set to 2, only obscured rays are plotted. The command is set up as follows:

command='OBS' CARG(1)=Obscuration option

      - SINT: activates the surface interpolation algorithms. This command has no argu-ments. The command is set up as follows:

command='SINT'

#### Diffraction Analysis

      - INTensity: trace the full set of rays and propagate the diffraction wavefront from the source (or from the current element if a diffraction command has been performed previously) up to a specified element. The resulting complex amplitude is stored in the array WFElt, which is one of the variables defined in the include files. The mod-ulus squared is plotted; it is stored in the MWFFT array. This command has one argu-ment, the element number. The command is set up as follows:

command='INTENSITY' IARG(1)=iElt

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

Commands

      - GAIn: compute and plot the ratio of the light diffracted by the optical system to a point source. This command has one argument, the element number at which to comput the gain. The command is set up as follows:

command='GAIN' IARG(1)=iElt

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - SCAlar: calculate using scalar diffraction algorithm, the default. This command has no arguments. The command is set up as follows:

command='SCALAR'

      - VECtor: calculate using scalar diffraction algorithm, the default. This command has no arguments. The command is set up as follows:

command='VECTOR'

      - ORS: optimize a reference surface for sphere-to-sphere diffraction. This command finds the best-fit sphere to the wavefront at a specified reference surface element, which is then reset to match the best-fit sphere. This command has one argument, the reference surface element number. The command is set up as follows:

command='ORS' IARG(1)=iElt

      - SRS: slave a reference surface to a previously optimized reference surface for sphere-to-sphere near-field diffraction or for sphere-to-plane far-field diffraction. This com-mand has two arguments: the element number of the surface to be slaved; and the element number of the surface to which it is to be slaved. It uses the parameters defining the second element to reset the first so that the diffraction is correctly defined. The command is set up as follows:

command='SRS' IARG(1)=iSlv1 IARG(2)=iSlv2

      - FEX: find the exit pupil of the system. This command traces two chief rays through the aperture stop of the system to the element preceeding the specified element and finds their crossing point in image space. The specified surface is then reset to be a sphere located at the crossing point and centered at the intersection of the source chief ray with the preceeding element. Both the specified element and the one pre-ceeding it must be return surfaces. The command has two arguments: the number of the element to be set to the exit pupil and the location of the system aperture stop in global coordinates. The command is set up as follows:

command='FEXIT' IARG(1)=iElt

CARG(1)=’YES’ (to accept changes)

      - REGrid: toggle the “regrid” option for sphere-to-sphere propagations. With regrid-ding set on, the ray grid is recalculated after each near-field propagation to assure correct alignment of the rays with the diffraction wavefront grid points. This com-mand is most useful in highly aberrated systems. In some circumstances it must be used with the OPD interpolation command (not currently implemented). The com-mand has no arguments. It is set up as follows:

command='REGRID'

      - BEAm: allows the user to select between a uniform beam and a Gaussian beam. This command has three arguments: the beam type (uniform or Gaussian), the x beam diameter (for a gaussian beam) and y beam diameter (for a Gaussian beam). The command is set up as follows:

command='BEAM' IARG(1)=BeamType

DARG(1)=x beam diamter DARG(2)=y beam diamter

#### Wavefront and Image Plots

      - INtensity: compute and plot the intensity of the wavefront at the specified element. Plotting is done only if plot type is not NONe and the graphics options are included in the compile. The type of plot is as set by previous calls to SMACOS. This command has one argument, the element number. The command is set up as follows:

command='IN' IARG(1)=iElt

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - AMplitude: compute and plot the amplitude and phase of the wavefront at the spec-ified element. This command has one argument, the element number. The command is set up as follows:

command='AM' IARG(1)=iElt

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - REAl: compute and plot the real and imaginary parts of the wavefront at the specified element. This command has one argument, the element number. The command is set up as follows:

command='REAL' IARG(1)=iElt

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - LOG: compute and plot the logarithm (base 10) of the intensity of the wavefront at the specified element. This command has one argument, the element number. The com-mand is set up as follows:

command='LOG' IARG(1)=iElt

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - PIxilate: compute and plot the pixilated wavefront intensity at the specified ele-ment. This command has three arguments: the element number, the number of pixels across one side of the array (assumed square), and the size of a pixel. The minimum size of the pixel is half the wavefront grid spacing. This is not automatically checked, so be careful! The PIxilated image is stored in the PixArray variable and passed through the SMACOS call line. The PLOcate command sets the option to locate the center of the pixel array to the vertex point of the element. The command is set up as follows:

command='PIXILATE' IARG(1)=iElt

IARG(2)=number of pixels per side RARG(1)=size of pixel

      - COMpose: initiate a composed image to which subsequent intensity plots can be ADDed. This command has three arguments: the element number, the number of pix-els across one side of the array (assumed square), and the size of a pixel. The mini-mum size of the pixel is half the wavefront grid spacing. This is not automatically checked, so be careful! The command is set up as follows:

command='COMPOSE' IARG(1)=iElt

IARG(2)=number of pixels per side RARG(1)=size of pixel in base units

      - WINdow: define a window on the detector that stays fixed wrt the detector. Subse-quent images that fall on the window, as determined by the ray trace, are placed appropriately, provided the FEX command is used for each image. Used with the PIX,

Commands

COMPOSE, ADD, etc. commands. This command has 5 arguments. It is set up as fol-lows:

command='WINDOW'

CARG(1)=’TOUT’ (detector coordinates option) DARG(1)=size of pixel in base units

DARG(2)=x element vertex location in pixel coords DARG(3)=y element vertex location in pixel coords DARG(4)=x window center location in pixel coords DARG(5)=y window center location in pixel coords

      - ADD: add an intensity plot to a composed image. This command has two arguments: the element number and a character argument to specify whether to plot the com-posed image. The composed image is stored in the PixArray variable and passed through the SMACOS call line. The command is set up as follows:

command='ADD' IARG(1)=iElt

CARG(1)='PLOT' or 'NOPLOT'

CARG(2)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - DADD: display the currently composed image. This command has no arguments. The command is set up as follows:

command='DADD'

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - NOISE: add detection noise to the currently composed image. Noise here means ran-dom fluctuations in the values of the pixels of an image. This command has several arguments, which vary with context. The command is set up for Gaussian noise as follows:

command='NOISE' CARG(1)='GAUSSIAN'

DARG(1)=standard deviation of noise in DN CARG(2)='PLOT' or 'NOPLOT'

CARG(3)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

For photon noise, a Poisson process where the standard deviation is the expected value:

command='NOISE' CARG(1)='PHOTON' CARG(2)='PLOT' or 'NOPLOT'

CARG(3)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

For Poisson noise on a uniform background level:

command='NOISE' CARG(1)='POISSON'

RARG(1)=background level in DN

DARG(1)=detector quantum efficiency (in the range 0 to 1) CARG(2)='PLOT' or 'NOPLOT'

CARG(3)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - BLUR: blur the currently composed image using a 9-pixel kernel. This command is intended to simulate detector charge leakage. It has one argument, the blur kernel peak, which can take values between 1 and 0. The command is set up as follows:

command='BLUR' RARG(1)=blur kernel peak CARG(1)='PLOT' or 'NOPLOT'

CARG(2)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - GBLUR: propagate and blur an image, then add to the currently composed image. This command is more general than the BLUR command, using a Gaussian kernel. It is often used to simulate long-exposure jitter blurring. The command is set up as fol-lows:

command='GBLUR'

DARG(1)=Gaussian blur kernel width in base units CARG(1)='PLOT' or 'NOPLOT'

CARG(2)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

#### Plot Types

      - NONe: turns off plotting entirely. The command is set up as follows:

command='NONE'

      - NULl: turns off plotting entirely. The command is set up as follows:

command='NULL'

      - WIRe: sets surface plot type to “wireframe”. The command is set up as follows:

command='WIRE'

      - SLIce: sets surface plot type to “slice”. The command is set up as follows:

command='SLICE'

      - GRAy: sets surface plot type to “gray scale”. The command is set up as follows:

command='GRAY'

      - CONtour: sets surface plot type to “contour plot”. The command is set up as follows:

command='CONTOUR'

      - ROW: sets surface plot type to “row plot”. The command is set up as follows:

command='ROW'

      - COLumn: sets surface plot type to “column plot”. The command is set up as follows:

command='ROW'

      - TEXt: sets surface plot type to “text”. This provides for the plot data to be written to a text file instead of being plotted. The command is set up as follows:

command='TEXT'

      - BINary: sets surface plot type to “binary”. This provides for the plot data to be writ-ten to a binary file instead of being plotted. The command is set up as follows:

command='BINARY'

      - MATlab: sets surface plot type to “Matlab”. This provides for the plot data to be writ-ten to a Matlab .m-file instead of being plotted. The command is set up as follows:

command='MATLAB'

      - FITs: sets surface plot type to “fits”. This provides for the plot data to be written to a fits file instead of being plotted. The command is set up as follows:

command='FITS'

#### Linear Models

      - BUild: trace the full set of rays and the ray partial derivative matrices from the source up to a specified element, and then compute the composite C-matrix. The result is stored in the array Cmatrix and passed in the SMACOS call line. This com-mand has one argument, the element number. The command is set up as follows:

command='BUILD' IARG(1)=iElt

      - DMBuild: performs as the DMBuild, but also includes interpolated surface partials. The result is stored in the array C-matrix and passed in the SMACOS call line. This command has one argument, the element number. The command is set up as follows:

command='DMBUILD' IARG(1)=iElt

      - PArtials: displays and prints the full set of ray partial derivatives for a specified ray. The command is set up as follows:

Commands

command='PARTIALS' IARG(1)=iRay

      - LPErturb: rotate and/or translate a specified optical element. This command has several arguments. An integer argument is used to specify the number of the element to be perturbed (LPErturb does not work with element 0). Up to 9 double precision arguments are used to specify the perturbations. A character argument is used to specify global or element coordinates. The command is set up as follows:

command='LPERTURB'

IARG(1)=iElt

and the coordinate system for the perturbation is specified using a character argu-ment:

CARG(1)=eltcoord or globalcoord

If element coordinates are choosen:

DARG(1)=perturbation in element coordinate 1 DARG(2)=perturbation in element coordinate 2 DARG(3)=perturbation in element coordinate 3 DARG(4)=perturbation in element coordinate 4 DARG(5)=perturbation in element coordinate 5 DARG(6)=perturbation in element coordinate 6

else if global coords are choosen:

DARG(1)=rotational perturbation x DARG(2)=rotational perturbation y DARG(3)=rotational perturbation z DARG(4)=translational perturbation x DARG(5)=translational perturbation y DARG(6)=translational perturbation z

      - LPRead: rotate and/or translate a specified optical element using data stored in a sep-arate file. This command has no arguments. It assumes that properly formatted data (see Section 8.3.2) is stored in a file named filnam.pert where filnam is the name of the .in-file. The command is set up as follows:

command='LPREAD'

      - LPReset: reset the linear perturbation vector to zero. This command has no argu-ments. The command is set up as follows:

command='LRESET'

      - LSPot: use the linear model generated in the BUild and LPErturb commands to compute the spot diagram at the last element specified in the BUild command. This element should be a focal plane (EltType=3). The output of this command is stored in OutRaySpot and (depending on the active plot option) may be plotted on the screen. This command has no arguments. The command is set up as follows:

command='LSPOT'

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - LOPd: use the linear model generated in the BUILD and LPERTURB commands to com-pute the OPD map at the last element specified in the BUILD command. This element should be a reference surface (EltType=4). The output of this command is stored in PertOPDMat and (depending on the active plot option) may be plotted on the screen. This command has no arguments. The command is set up as follows:

command='LOPD'

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - LINtensity: use the linear model generated in the BUild and LPErturb commands to compute and plot the intensity of the wavefront at the last element specified in the BUild command. This element should be a reference surface (EltType=4) followed

by a focal plane set up for a far-field diffraction. This command has no arguments. The command is set up as follows:

command='LINTENSITY'

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

      - LPIxilate: use the linear model generated in the BUild and LPErturb commands to compute and plot the pixilated wavefront intensity at the last element specified in the BUild command. This element should be a reference surface (EltType=3) fol-lowed by a focal plane set up for a far-field diffraction. This command has two argu-ments: the number of pixels across one side of the array (assumed square) and the size of a pixel. The minimum size of the pixel is half the wavefront grid spacing. This is not automatically checked, so be careful! The PLOcate command does not apply to the LPIxilate command. The command is set up as follows:

command='LPIXILATE' IARG(1)=number of pixels per side RARG(1)=size of pixel

CARG(1)=’YES’ (to overwrite if a file-writing plot type is selected and a file of the same name exists)

### GMI: MATLAB MEX interface (FreeForm updates)

GMI (MACOS_resources/GMI/) is a thin MATLAB MEX wrapper around SMACOS, packaging perturbations and pulling back OPD / PIX / SPOT / complex-EF arrays into MATLAB.  V4.00 changes:

NRHS = 14 (was 13).  Old call_GMI.m callers passing 13 arguments will fail with 'GMI requires 14 input arguments'.  The new 14th argument is pmonzern, a MonZernCoef perturbation vector parallel to pzern for FreeForm surfaces.

param.monzernSrf -- column vector of FreeForm element indices to perturb (analogous to param.zernSrf).  Encoded in pflg with a 9999 sentinel when absent.

param.pmonzern -- per-mode coefficient vector, length length(monzernSrf) * nmonzern, where nmonzern defaults to param.mzern.  Read inside call_GMI.m and forwarded as the 14th MEX argument; user code does not call GMI() directly.

GMI.inc compile-time sizes: numseg = 7, mzern = 15 (up from 6 / 12) to support 7-segment FreeForm test cases with 15 modes per segment.  When a runtime param.mzern exceeds the compile-time mzern, the MEX validation rejects with 'pzern must be scalar, empty or a mpzern x 1 matrix'.  Bump mzern and force a rebuild (rm GMI.o GMIG.o; source makegmi.sh -- make does not detect .inc changes).

Nominal save/restore expanded to FreeForm frames.  ObtainNominalSettings / SetToNominalSettings now snapshot and restore pFF/xFF/yFF/zFF and (when nGridMat > 0) pData/xData/yData/zData alongside the existing pMon / psiElt / vptElt / TElt entries.  This fixes a v3.x bug where per-call prb perturbations on FreeForm elements leaked the FreeForm coordinate frames forward, so the 'nominal-equivalent' OPD drifted across iterations of a sensitivity loop.

See MACOS_resources/GMI/README.md and MACOS_resources/GMI/CLAUDE.md for full architecture details.

## REFERENCES

- D. Redding and W. Breckenridge, “Modeling Optics for Dynamics and Control Analysis,” Journal of Guidance, Control and Dynamics, September, 1991.

- J. W. Goodman, Introduction to Fourier Optics, McGraw-Hill, New York, 1968.

- Sziklas and Siegman, “Mode Calculations in Unstable Resonators with Flowing Sat-urable Gain2: Fast Fourier Transform Method,” Applied Optics, v14, p. 1874-1889, 1975.

- Oppenheim and Schaefer, Digital Signal Processing.

- Brigham,

- G. Lawrence, OASIS Manual, Version 5.0, August, 1985 with August 1986 updates. Published by the Rocketdyne Division of Rockwell International Corporation, Albu-querque Optical Support Office.

- G. Lawrence, Optical Modeling preprint

- D. Redding and W. Breckenridge, “Coordinate-Free Raytrace Equations for Aspheric and Deformed Optical Surfaces,” Memo DCR-91-38, July 13, 1991.

- D. Redding, “Mathematical Description of Generalized Optical Element Surfaces,” Memo DCR-91-26, May 27, 1991.

- P. C. Hughes, Spacecraft Attitude Dynamics,1986.

- H. Akima, “A method of bivariate interpolation and smooth surface fitting for irrr-regularly distributed data points,” ACM Trans. on Mathematical Software, V4, No 2, June 1978.

- H. Akima, “ALGORITH 526:Bivariate interpolation and smooth surface fitting for irrrregularly distributed data points,” ACM Trans. on Mathematical Software, V4, No 2, June 1978.

- D. C. Redding, D, P. Dumont, and J. Yu, “Hubble Space Telescope Prescription Retrieval,” Optical Society of America, Space Optics Meeting, Paper MF4. Will-iamsburg VA. 11/18-19/91.

- D. C. Redding and W. Breckenridge, “Optical Modeling for Dynamics and Control Analysis,” Journal of Guidance, Control, and Dynamics, Vol. 14, No. 5. Sept.-Oct. 1991.

## APPENDIX A: Examples and Tutorials

Demonstrations

This appendix contains prescriptions and macros that illustrate various aspects of MACOS usage. Electronic versions of these files have been provided with the MACOS distribution tapes. The interested user can execute these examples to gain experience using MACOS.

### Cassegrain Telescope Examples

#### Cassegrain.jou

% This macro generates example figures

% for Section 2 of the MACOS User Manual

old Cassegrain modify ngridpts=51 q

spot 1 Tout	% Fig 3a

draw;;;;	% Fig 3b

spot 5 Tout	% Fig 4a

perturb 2 global 0,1e-4,0 0,0,0

spot 5 Tout	% Fig 4b

#### Cassegrain.in

% Cassegrain telescope example

ChfRayDir= 0d0 0d0 1d0 ChfRayPos= 0d0 0d0 -5d0 zSource= 1.d+21

IndRef= 1d0

Extinc= 0d0 Wavelen= 1.d-06

Flux=	1d0 GridType=	Circular Aperture=	4d0 Obscratn=	0d0 nGridpts=	15

xGrid= -1d0 0d0 0d0 yGrid= 0d0 -1d0 0d0 nElt=  5

iElt=  1

EltName= SecMirObs Element= Obscuring Surface= Conic

KrElt= -1.d+20

Modeling and Analysis for Controlled Optical Systems	167

KcElt=	0d0

psiElt=	0d0 0d0 -1d0 VptElt=	0d0 0d0 -4.2d+00 RptElt=	0d0 0d0 -4.2d+00 IndRef=	1d0

Extinc=	1d22

nObs=	3

ObsType=	Circle

ObsVec=	5.d-01 0d0 0d0 ObsType=	Rectangle

ObsVec= -0.003125 0.003125 -3 3

ObsType=	Rectangle

ObsVec= -3 3 -0.003125 0.003125

xObs=		1d0 0d0 0d0 ApType=		Circular ApVec=	2d0 0d0 0d0

zElt=		1.d+20 PropType=		Geometric nECoord=	-6

iElt=	2

EltName= Primary Element= Reflector Surface= Conic

KrElt= -1.08d+01

KcElt= -1d0

psiElt= 0d0 0d0 -1d0 VptElt= 0d0 0d0 0d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0

Extinc=	1d22

nObs=	0

ApType=	Circular

ApVec=	2.1d0 0d0 0d0 zElt=	5.4d+00

PropType=	Geometric nECoord=	-6

iElt=  3

EltName= Secondary Element= Reflector Surface= Conic

KrElt= -3.526787501D+00 KcElt= -2.670556022D+00

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 -4.061145902D+00

RptElt=	0d0 0d0 -5.4d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	1.338854098D+00

PropType=	Geometric nECoord=	-6

iElt=	4

EltName= Ref_surf Element= Reference Surface= Conic

KrElt= -5.560145902D+00

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -4.060145902D+00 RptElt=	0d0 0d0 -4.060145902D+00

| IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 1d0 1d22 0 None 5.560145902D+00 FarField -6 |  |
| --- | --- | --- |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 5 Detector FocalPlane Flat -1d22 0d0 0d0 0d0 1d0 0d0 0d0 1.5d+00 0d0 0d0 1.5d+00 1d0 1d22 0 None 0d0 Geometric -6 |  |
| nOutCord= Tout= | 5 1d0 0d0 0d0 0d0 | 0d0 0d0 0d0 |
|  | 0d0 1d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 | 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 1d0 |

#### CassWithExitPupil.jou

% This macro generates more example figures

% for Section 2 of the MACOS User Manual

old CassWithExitPupil modify ngridpts=128 q

perturb 3 global 0,0,0 0,0,1e-4

stop obj 0,0,0

fex 5 yes

opd 5 Tout	% Fig 6a

perturb 2 global 0,1e-4,0 0,0,0

opd 5 Tout	% Fig 6b

old CassWithExitPupil modify ngridpts=64 q stop obj 0,0,0

fex 5 yes

intensity 6	% Fig 10a

perturb 2 global 0,1e-4,0 0,0,0

fex 5 yes

intensity 6	% Fig 10b

#### CassWithExitPupil.in

% Cassegrain telescope example

% This version has an exit pupil

ChfRayDir= 0d0 0d0 1d0 ChfRayPos= 0d0 0d0 -5d0 zSource= 1.d+21

IndRef= 1d0

Extinc= 0d0 Wavelen= 1.d-06

Flux=	1d0 GridType=	Circular Aperture=	4d0 Obscratn=	0d0 nGridpts=	15

xGrid= -1d0 0d0 0d0 yGrid= 0d0 -1d0 0d0 nElt=  6

iElt=  1

EltName= SecMirObs Element= Obscuring Surface= Conic

KrElt= -1.d+20 KcElt=	0d0

psiElt=	0d0 0d0 -1d0 VptElt=	0d0 0d0 -4.2d+00 RptElt=	0d0 0d0 -4.2d+00 IndRef=	1d0

Extinc=	1d22

nObs=	3

ObsType=	Circle

ObsVec=	5.d-01 0d0 0d0 ObsType=	Rectangle

ObsVec= -0.003125 0.003125 -3 3

ObsType=	Rectangle

ObsVec= -3 3 -0.003125 0.003125

xObs=		1d0 0d0 0d0 ApType=		Circular ApVec=	2d0 0d0 0d0

zElt=		1.d+20 PropType=		Geometric nECoord=	-6

iElt=	2

EltName= Primary Element= Reflector Surface= Conic

KrElt= -1.08d+01

KcElt= -1d0

psiElt= 0d0 0d0 -1d0 VptElt= 0d0 0d0 0d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0

Extinc=	1d22

nObs=	0

ApType=	Circular

ApVec=	2.1d0 0d0 0d0 zElt=	5.4d+00

PropType=	Geometric nECoord=	-6

iElt=  3

EltName= Secondary Element= Reflector Surface= Conic

KrElt= -3.526787501D+00 KcElt= -2.670556022D+00

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 -4.061145902D+00

RptElt=	0d0 0d0 -5.4d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	1.338854098D+00

PropType=	Geometric nECoord=	-6

iElt=	4

EltName= Return1 Element= Return Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 1.5d+00 RptElt=	0d0 0d0 1.5d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		0d0 PropType=		Geometric nECoord=	-6

iElt=	5

EltName= ExitPupil Element= Return Surface= Conic

KrElt= -5.560145902D+00

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -4.060145902D+00 RptElt=	0d0 0d0 -4.060145902D+00

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	5.560145902D+00

PropType=	FarField nECoord=	-6

iElt=	6

EltName= Detector Element= FocalPlane Surface= Flat

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 1.5d+00 RptElt=	0d0 0d0 1.5d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		0d0 PropType=		Geometric nECoord=	-6

nOutCord=  5

Tout= 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0

### Diffractive Elements Examples

#### GratingExample.jou

% This macro generates example figures

% for Section 4 of the MACOS User Manual old gratingExample

mod OrderHOE(1)=0 q draw;;;;

mod OrderHOE(1)=1 q

draw;;;;	% Fig ??

mod OrderHOE(1)=2 q draw;;;;

mod OrderHOE(1)=-1 q draw;;;;

mod OrderHOE(1)=-2 q

draw;;;;	% Fig ??

#### GratingExample.in

ChfRayDir= 0d0 0d0 1d0 ChfRayPos= 0d0 0d0 -6d0 zSource= 1d22

IndRef= 1d0

Extinc= 0d0 Wavelen= 6.d-07

Flux=	1d0 GridType=	Circular Aperture=	1d0 Obscratn=	0d0 nGridpts=	65

xGrid= 1d0 0d0 0d0 yGrid= 0d0 1d0 0d0 nElt=  2

iElt=  1

EltName= Grating Element= Grating Surface= Flat

KrElt= -1d22 KcElt= 0d0

psiElt= 0d0 0d0 -1d0 VptElt= 0d0 0d0 0d0 RptElt= 0d0 0d0 0d0

| IndRef= 1d0 Extinc= 1d22 h1HOE= 1d0 0d0 0d0 OrderHOE= 1d0 RuleWidth= 18.97366596d-7 nObs= 0 ApType= None zElt= 1d22 PropType= Geometric nECoord= -6 |  |  |
| --- | --- | --- |
| iElt= 2 EltName= FP Element= FocalPlane Surface= Flat KrElt= 1d22 KcElt= 0d0 psiElt= 0d0 0d0 1d0 VptElt= 0d0 0d0 -3d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0 Extinc= 0d0 nObs= 0 ApType= None zElt= 1d22 PropType= Geometric nECoord= -6 |  |  |
| nOutCord= 5 Tout= 1d0 0d0 0d0 0d0 | 0d0 0d0 | 0d0 |
| 0d0 1d0 0d0 0d0 | 0d0 0d0 | 0d0 |
| 0d0 0d0 0d0 1d0 | 0d0 0d0 | 0d0 |
| 0d0 0d0 0d0 0d0 | 1d0 0d0 | 0d0 |
| 0d0 0d0 0d0 0d0 | 0d0 0d0 | 1d0 |
| A.2.3 HOEExample.jou |  |  |

% This macro generates example figures

% for Section 4 of the MACOS User Manual

loa HOEExample mod ngridpts=15 q map

ray 76

stop elt 1 0,0 fex;;;

int;;

draw;;;;	% Fig ??

mod Element(1)=reflector q ray 76

fex;;; int;;

| draw;;;; | % Fig ?? |
| --- | --- |
| A.2.4 HOEExample.in |  |
| ChfRayDir= 0d0 0d0 1d0 ChfRayPos= 0d0 0d0 -4d0 zSource= 1d22 IndRef= 1d0 Extinc= 0d0 Wavelen= 6.d-07 Flux= 1d0 GridType= Circular Aperture= 5.d-01 Obscratn= 0d0 nGridpts= 64 xGrid= 1d0 0d0 0d0 yGrid= 0d0 1d0 0d0 nElt= 2 |  |
| iElt= 1 EltName= HOEonMirror Element= HOE Surface= Conic KrElt= -6d0 KcElt= -1d0 psiElt= 0d0 0d0 -1d0 VptElt= 0d0 0d0 0d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0 Extinc= 1d22 h1HOE= 0d0 0d0 -1d22 h2HOE= 1d0 0d0 -3d0 OrderHOE= 1d0 WaveHOE= 6.d-07 nObs= 0 ApType= None zElt= 1d22 PropType= Geometric nECoord= -6 |  |
| iElt= 2 EltName= FP Element= FocalPlane Surface= Flat KrElt= -1d22 KcElt= 0d0 psiElt= 0d0 0d0 1d0 VptElt= 0d0 0d0 -3d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0 Extinc= 0d0 nObs= 0 ApType= None zElt= 1d22 PropType= Geometric nECoord= -6 |  |
| nOutCord= 5 Tout= 1d0 0d0 0d0 0d0 | 0d0 0d0 0d0 |
| 0d0 1d0 0d0 0d0 | 0d0 0d0 0d0 |
| 0d0 0d0 0d0 1d0 | 0d0 0d0 0d0 |
| 0d0 0d0 0d0 0d0 | 1d0 0d0 0d0 |
| 0d0 0d0 0d0 0d0 | 0d0 0d0 1d0 |

### Non-Sequential Surface Examples

#### CornerCube.jou

old CornerCube draw;;;xz

mod ChfRayPos(1)=0.762 ChfRayPos(2)=0.762 Aperture=0.125 nGridpts=11

q draw;;;yz

#### CornerCube.in

ChfRayDir= 0d0 0d0 -1d0 ChfRayPos= 1.d-05 1.d-05 1.d+01 zSource=	1.d22

IndRef=	1d0

Extinc=	0d0 Wavelen=	1.059d-04

Flux=	1d0 GridType=	Circular Aperture=	2.54d+00 Obscratn=	0d0 nGridpts=	65

xGrid= 0d0 1d0 0d0 yGrid= -1d0 0d0 0d0 nElt=  5

iElt=	1

EltName= Ref Srf Element= Reference Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0 VptElt=	0d0 0d0 3.5d+00 RptElt=	0d0 0d0 0d0 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	0d0 PropType=	NFP1surf nECoord=	0

iElt=	2

EltName= CC1 Element= NSReflector Surface= Conic

KrElt=	1d22

KcElt=	0d0

psiElt=	8.164994677D-01 0d0 5.773461867D-01

VptElt=	0d0 0d0 0d0 RptElt=	0d0 0d0 0d0

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	3

EltName= CC2 Element= NSReflector Surface= Conic

KrElt=	1d22

KcElt=	0d0

psiElt= -4.082497338D-01 7.071092812D-01 5.773461867D-01

VptElt= 0d0 0d0 0d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	4

EltName= CC3 Element= NSReflector Surface= Conic

KrElt=	1d22

KcElt=	0d0

psiElt= -4.082497338D-01 -7.071092812D-01 5.773461867D-01

VptElt= 0d0 0d0 0d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	5

EltName= FP Element= FocalPlane Surface= Flat

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0 VptElt=	0d0 0d0 3.5d+00 RptElt=	0d0 0d0 0d0 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	1.d+03 PropType=	Geometric nECoord=	0

nOutCord=  5

Tout= 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0

0d0 0d0 0d0 0d0 0d0 0d0 1d0

#### Luneberg.jou

% Luneberg.jou: a MACOS example macro. This macro

% demonstrates use of non-sequential refracting

% elements, polarization, and the gain command.

old Luneberg draw 0 16 XZ

beam cos 8.7500000000D-01 1.5

mod nGridPts=128 q int 16

mod nGridPts=64 q gain 17

col

gain 17 129

pol 1,0 0,0

mod nGridPts=128 q int 16

mod nGridPts=64 q gain 17

col

gain 17 129

#### Luneberg.in

% Luneberg.in: a MACOS example .in-file. This

% macro demonstrates use of non-sequential

% refractors, polarization, and the gain command.

ChfRayDir= 0d0 0d0 1d0

ChfRayPos= 0d0 0d0 -2.121332605D-01

zSource= -1.d-02 IndRef=	1d0

Extinc=	0d0 Wavelen=	2.15d-02

Flux=	1d0 GridType=	Circular Aperture=	1.75d+00 Obscratn=	0d0 nGridpts=	33

xGrid= 1d0 0d0 0d0 yGrid= 0d0 1d0 0d0 nElt= 17

iElt=	1

EltName= Shell1 Element= NSRefractor Surface= Conic

KrElt= -2.0353d-01

KcElt=	0d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 -2.0353d-01

RptElt=	0d0 0d0 0d0 IndRef=	1.056692495D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	2

EltName= Shell2 Element= NSRefractor Surface= Conic

KrElt= -1.887578424D-01

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -1.887578424D-01

RptElt=	0d0 0d0 0d0 IndRef=	1.084805420D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	3

EltName= Shell3 Element= NSRefractor Surface= Conic

KrElt= -1.872644104D-01

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -1.872644104D-01

RptElt=	0d0 0d0 0d0 IndRef=	1.147803142D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	4

EltName= Shell4 Element= NSRefractor Surface= Conic

KrElt= -1.771655689D-01

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -1.771655689D-01

RptElt=	0d0 0d0 0d0 IndRef=	1.182543332D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	5

EltName= Shell5 Element= NSRefractor Surface= Conic

KrElt= -1.529604025D-01

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -1.529604025D-01

RptElt=	0d0 0d0 0d0 IndRef=	1.235415610D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	6

EltName= Shell6 Element= NSRefractor Surface= Conic

KrElt= -1.387929631D-01

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -1.387929631D-01

RptElt=	0d0 0d0 0d0 IndRef=	1.265593240D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	7

EltName= Shell7 Element= NSRefractor Surface= Conic

KrElt= -1.297999186D-01

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -1.297999186D-01

RptElt=	0d0 0d0 0d0 IndRef=	1.283513736D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	8

EltName= Shell8 Element= NSRefractor Surface= Conic

KrElt= -1.087287483D-01

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -1.087287483D-01

RptElt=	0d0 0d0 0d0 IndRef=	1.312911218D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	9

EltName= Shell9 Element= NSRefractor Surface= Conic

KrElt= -9.780754704D-02

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -9.780754704D-02

RptElt=	0d0 0d0 0d0 IndRef=	1.340947910D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	10

EltName= Shell10 Element= NSRefractor Surface= Conic

KrElt= -8.777974770D-02

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -8.777974770D-02

RptElt=	0d0 0d0 0d0 IndRef=	1.338212289D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	11

EltName= Shell11 Element= NSRefractor Surface= Conic

KrElt= -7.943573990D-02

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -7.943573990D-02

RptElt=	0d0 0d0 0d0 IndRef=	1.371777076D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	12

EltName= Shell12 Element= NSRefractor Surface= Conic

KrElt= -6.183854602D-02

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -6.183854602D-02

RptElt=	0d0 0d0 0d0 IndRef=	1.386006296D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	13 EltName= Shell13 Element= NSRefractor Surface= Conic

KrElt= -4.663931773D-02

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -4.663931773D-02

RptElt=	0d0 0d0 0d0 IndRef=	1.402911090D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	14

EltName= Shell14 Element= NSRefractor Surface= Conic

KrElt= -2.956340242D-02

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -2.956340242D-02

RptElt=	0d0 0d0 0d0 IndRef=	1.412d+00 Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	15

EltName= Aperture Element= Return Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 2.221359167D-01

RptElt=	0d0 0d0 0d0 IndRef=	1.0488d+00

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		1.d+01 PropType=		Geometric nECoord=	-6

iElt=	16

EltName= ExitPupil Element= Return

| Surface= KrElt= KcElt= psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | Conic -1d22 0d0 0d0 0d0 -1d0 0d0 0d0 0d0 0d0 0d0 0d0 1.0488d+00 1d22 0 None 1.d+01 FarField -6 |  |
| --- | --- | --- |
| iElt= | 17 |  |
| EltName= Element= Surface= KrElt= KcElt= psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | FP FocalPlane Flat -1d22 0d0 0d0 0d0 -1d0 0d0 0d0 1d3 0d0 0d0 0d0 1d0 1d22 0 None 0d0 Geometric -6 |  |
| nOutCord= Tout= | 5 1d0 0d0 0d0 | 0d0 0d0 0d0 0d0 |
|  | 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 | 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 1d0 |

### Adaptive Optics Example

#### AOExample.jou

% MACOS Adaptive Optics Example Macro

%	First show system response without atmosphere:

old AOexample;	% Load prescription

draw 0 26 xz	% Draw full system

draw 6 26 xy	% Draw AO optics only

spo 1 b y	% Pupil spot diagram

opd 16 y	% OPD in exit pupil shows deformable

%	mirror test pattern

spo 26 Tout y	% Hartman sensor spots on detector

%	Next show system response with atmosphere but no

%	deformable mirror:

modify Surface(7)=Flat	% Flatten deformable mirror q

atmos 1 20	% Add atmospheric phase disturbance

2.854331e-5 0 atm 12.5	%	at entrance pupil

opd 16 y	% OPD in exit pupil

spo 26 Tout y	% Hartman sensor spots on detector

#### AOExample.in

% MACOS Adaptive Optics Example Prescription

| ChfRayDir= | 0d0 0d0 1d0 |
| --- | --- |
| ChfRayPos= | 0d0 0d0 -2d+02 |
| zSource= | 1d22 |
| IndRef= | 1d0 |
| Extinc= | 0d0 |
| Wavelen= | 2.854330709D-05 |
| Flux= | 1d0 |
| GridType= | Circular |
| Aperture= | 1.991873225632d02 |
| Obscratn= | 40.5d0 |
| nGridpts= | 128 |
| xGrid= | 0d0 -1d0 0d0 |
| yGrid= | -1d0 0d0 0d0 |
| nElt= | 26 |
| iElt= | 1 |
| EltName= | Pupil |
| Element= | Reference |
| Surface= | Conic |
| KrElt= | -1d22 |
| KcElt= | 0d0 |
| psiElt= | 0d0 0d0 -1d0 |
| VptElt= | 0d0 0d0 -6d0 |
| RptElt= | 0d0 0d0 0d0 |
| IndRef= | 1d0 |
| Extinc= | 0d0 |
| nObs= | 0 |
| ApType= | None |
| zElt= | 1d22 |
| PropType= | Geometric |
| nECoord= | -6 |
| iElt= | 2 |
| EltName= | PM |
| Element= | Reflector |
| Surface= | Conic |
| KrElt= | -1.3357d+03 |
| KcElt= | -1d0 |
| psiElt= | 0d0 0d0 -1d0 |
| VptElt= | 0d0 0d0 0d0 |
| RptElt= | 0d0 0d0 0d0 |
| IndRef= | 1d0 |
| Extinc= | 0d0 |
| nObs= | 0 |
| ApType= | None |
| zElt= | 6.6785d+02 |
| PropType= | Geometric |
| nECoord= | -6 |
| iElt= | 3 |
| EltName= | SM |

Element= Reflector Surface= Conic

KrElt= -3.184999999D+02 KcElt= -2.352756999D+00

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 -5.423064572D+02 RptElt=	0d0 0d0 -5.423064572D+02

IndRef=	1d0

Extinc=	0d0

nObs=	0

ApType=	None

zElt=	1.256970591D+02

PropType=	Geometric nECoord=	-6

iElt=	4

EltName= FM1 Element= Reflector Surface= Flat

KrElt= -1d22 KcElt=	0d0

psiElt=	7.071067812D-01 0d0 7.071067812D-01

VptElt= 0d0 0d0 7.5375d+01 RptElt= 0d0 0d0 7.5375d+01 IndRef= 1d0

Extinc= 0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	5

EltName= OAP1 Element= Reflector Surface= Conic

KrElt= -1.364d+02

KcElt= -1d0

psiElt=	9.668919598D-01 -2.551860851D-01 0d0 VptElt= -4.140665047D+01 1.740405528D+01 7.5375d+01 RptElt= -4.140665047D+01 1.740405528D+01 7.5375d+01

IndRef=	1d0

Extinc=	0d0

nObs=	0

ApType=	None zElt=	6.82d+01

PropType=	Geometric nECoord=	-6

iElt=	6

EltName= FSM Element= Reflector Surface= Flat

KrElt= -1d22 KcElt=	0d0

psiElt=	9.999929341D-01 3.759223637D-03 0d0 VptElt=	1.318856255D+01 -1.530759269D+01 7.5375d+01 RptElt=	1.318856255D+01 -1.530759269D+01 7.5375d+01

IndRef=	1d0

Extinc=	0d0

nObs=	0

ApType=	None

zElt=	1d22 PropType=	Geometric

nECoord=	-6

iElt=	7

EltName= DM Element= Reflector Surface= UserDefined

KrElt= -1d22

| KcElt= 0d0 UDSrfType= 1 UDSrfFile= AOmirror.test pMon= -2.249426003D+00 | -1.951098758D+01 | % 1 or 5 7.5375d+01 |
| --- | --- | --- |
| xMon= -4.697040833D-01 | 8.828239202D-01 | 0d0 |
| yMon= 0d0 0d0 1d0 zMon= 8.828239202D-01 | 4.697040833D-01 | 0d0 |
| lMon= 0d0 psiElt= 8.828239202D-01 | 4.697040833D-01 | 0d0 |
| VptElt= -2.250697813D+00 | -1.950691308D+01 | 7.5375d+01 |
| RptElt= -2.250697813D+00 IndRef= 1d0 Extinc= 0d0 nObs= 0 ApType= None zElt= 1d22 PropType= Geometric nECoord= -6 | -1.950691308D+01 | 7.5375d+01 |
| iElt= 8 EltName= FM2 Element= Reflector Surface= Flat KrElt= -1d22 KcElt= 0d0 psiElt= 9.037029675D-02 | 9.959082335D-01 | 0d0 |
| VptElt= 2.331918268D+01 | 2.576404533D+00 | 7.5375d+01 |
| RptElt= 2.331918268D+01 IndRef= 1d0 Extinc= 0d0 nObs= 0 ApType= None zElt= 1d22 PropType= Geometric nECoord= -6 | 2.576404533D+00 | 7.5375d+01 |
| iElt= 9 EltName= OAP2 Element= Reflector Surface= Conic KrElt= -1.364d+02 KcElt= -1d0 psiElt= -6.268090394D-01 | 7.791729129D-01 | 0d0 |
| VptElt= 2.538458250D+01 | -2.822361844D+01 | 7.5375d+01 |
| RptElt= 2.538458250D+01 IndRef= 1d0 Extinc= 0d0 nObs= 0 ApType= None zElt= 6.82d+01 PropType= Geometric nECoord= -6 | -2.822361844D+01 | 7.5375d+01 |

| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 10 SSM1 Reflector Flat -1d22 0d0 -9.937679192D-01 | 1.114689322D-01 | 0d0 |
| --- | --- | --- | --- |
| VptElt= | 5.421565944D+00 | 8.117154092D+00 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 5.421565944D+00 1d0 0d0 0 None 1d22 Geometric -6 | 8.117154092D+00 | 7.5375d+01 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 11 SSM2 Reflector Flat -1d22 0d0 -9.970527522D-01 | 7.671902813D-02 | 0d0 |
| VptElt= | 1.733428143D+01 | 1.332169152D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 1.733428143D+01 1d0 0d0 0 None 1d22 Geometric -6 | 1.332169152D+01 | 7.5375d+01 |
| iElt= | 12 |  |  |
| EltName= Element= Surface= KrElt= KcElt= psiElt= | FS Reflector Flat -1d22 0d0 -9.593139745D-01 | 2.823414568D-01 | 0d0 |
| VptElt= | 4.387255193D+00 | 2.153812421D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 4.387255193D+00 1d0 0d0 0 None 1d22 Geometric -6 | 2.153812421D+01 | 7.5375d+01 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 13 WFSRL1A Refractor Conic -1.550872636D+00 0d0 9.999756307D-01 | -6.981260298D-03 | 0d0 |
| VptElt= | 8.496015971D+00 | 2.150943918D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= | 8.496015971D+00 1.512425970D+00 0d0 0 | 2.150943918D+01 | 7.5375d+01 |

| ApType= zElt= PropType= nECoord= | None 5.288796394D+00 Geometric -6 |  |  |
| --- | --- | --- | --- |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 14 WFSRL1B Refractor Conic -4.419109771D+00 0d0 -9.999756307D-01 | 6.981260298D-03 | 0d0 |
| VptElt= | 8.996003786D+00 | 2.150594855D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 8.996003786D+00 1.789618121D+00 0d0 0 None 3.411018511D+00 Geometric -6 | 2.150594855D+01 | 7.5375d+01 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 15 WFSRL1C Refractor Conic -2.771554824D+01 0d0 9.999756307D-01 | -6.981260298D-03 | 0d0 |
| VptElt= | 9.495991602D+00 | 2.150245792D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 9.495991602D+00 1d0 0d0 0 None 2.198859945D+01 Geometric -6 | 2.150245792D+01 | 7.5375d+01 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 16 LensArrayReference Reference Conic -1d22 0d0 9.999756303D-01 | -6.981321726D-03 | 3.414481680D-15 |
| VptElt= | 1.272900024D+01 | 2.147992068D+01 | 7.537500000D+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 1.272900024D+01 1d0 0d0 0 None 1d22 Geometric -6 | 2.147992068D+01 | 7.537500000D+01 |
| iElt= EltName= Element= | 17 LensArray1A LensArray |  |  |

Surface= Conic

KrElt= -0.47244094488189d+00

KcElt=	0d0

psiElt=	9.999756303D-01 -6.981321725D-03 0d0

VptElt=	12.7291897290d0 21.4720113661d0 75.3671259843d0 RptElt=	1.272924470D+01 2.147988519D+01 7.5375d+01

LensArrayType= 2

LensArrayWidth= 0.01574803149606

pMon= 1.272924470D+01 2.147988519D+01 7.5375d+01

xMon= 0d0 0d0 -1d0

yMon= -6.981321725D-03 -9.999756303D-01 0d0 zMon= -9.999756303D-01 6.981321725D-03 0d0

IndRef=	1.512425970D+00

Extinc=	0d0

nObs=	0

ApType=	None zElt=	5.17d+00

PropType=	Geometric nECoord=	-6

iElt=	18

EltName= LensArray1B Element= Refractor Surface= Flat

KrElt= -1d22 KcElt=	0d0

psiElt= -9.999756307D-01 6.981260298D-03 0d0 VptElt=	1.297923860D+01 2.147813988D+01 7.5375d+01 RptElt=	1.297923860D+01 2.147813988D+01 7.5375d+01

IndRef=	1d0

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	19

EltName= FieldLensA Element= Refractor Surface= Conic

KrElt= -2.122047244D+00

KcElt=	0d0

psiElt=	9.999756307D-01 -6.981260298D-03 0d0 VptElt=	1.358583579D+01 2.147390496D+01 7.5375d+01 RptElt=	1.358583579D+01 2.147390496D+01 7.5375d+01 IndRef=	1.677871170D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=	1.359427521D+00

PropType=	Geometric nECoord=	-6

iElt=	20

EltName= FieldLensB Element= Refractor Surface= Conic

KrElt= -3.224409449D+00

KcElt=	0d0

psiElt=	9.999756307D-01 -6.981260298D-03 0d0 VptElt=	1.374331196D+01 2.147280555D+01 7.5375d+01 RptElt=	1.374331196D+01 2.147280555D+01 7.5375d+01

IndRef=	1d0

| Extinc= | 0d0 |  |  |
| --- | --- | --- | --- |
| nObs= ApType= zElt= PropType= nECoord= | 0 None 1.359427521D+00 Geometric -6 |  |  |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 21 WFSLAR1A Refractor Conic -8.055118110D-01 0d0 9.999756307D-01 | -6.981260298D-03 | 0d0 |
| VptElt= | 1.880962705D+01 | 2.143743542D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 1.880962705D+01 1.611736488D+00 0d0 0 None 1.359427521D+00 Geometric -6 | 2.143743542D+01 | 7.5375d+01 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 22 WFSLAR1B Refractor Conic -5.090551181D-01 0d0 -9.999756307D-01 | 6.981260298D-03 | 0d0 |
| VptElt= | 1.896631614D+01 | 2.143634151D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 1.896631614D+01 1.677871170D+00 0d0 0 None 6.038543863D-01 Geometric -6 | 2.143634151D+01 | 7.5375d+01 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 23 WFSLAR1C Refractor Conic -2.980708661D+00 0d0 -9.999756307D-01 | 6.981260298D-03 | 0d0 |
| VptElt= | 1.901946445D+01 | 2.143597046D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 1.901946445D+01 1d0 0d0 0 None 2.027987141D+00 Geometric -6 | 2.143597046D+01 | 7.5375d+01 |
| iElt= | 24 |  |  |

| EltName= Element= Surface= | CCDWinA Refractor Conic |  |  |
| --- | --- | --- | --- |
| KrElt= KcElt= psiElt= | -1d22 0d0 -9.999756307D-01 | 6.981260298D-03 | 0d0 |
| VptElt= | 2.027861775D+01 | 2.142717977D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 2.027861775D+01 1.512425970D+00 0d0 0 None 1d22 Geometric -6 | 2.142717977D+01 | 7.5375d+01 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= | 25 CCDWinB Refractor Conic -1d22 0d0 -9.999756307D-01 | 6.981260298D-03 | 0d0 |
| VptElt= | 2.040361470D+01 | 2.142630711D+01 | 7.5375d+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 2.040361470D+01 1d0 0d0 0 None 1d22 Geometric -6 | 2.142630711D+01 | 7.5375d+01 |
| iElt= | 26 |  |  |
| EltName= Element= Surface= KrElt= KcElt= psiElt= | CCD FocalPlane Flat -1.000000000D+22 0.000000000D+00 -9.999764894D-01 | 6.857153362D-03 | 0d0 |
| VptElt= | 2.049655742D+01 | 2.142565820D+01 | 7.537500000D+01 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 2.049655742D+01 1.000000000D+00 0.000000000D+00 0 None 1.000000000D+22 Geometric -6 | 2.142565820D+01 | 7.537500000D+01 |
| nOutCord= Tout= | 5 -6.981260298D-03 | -9.999756307D-01 | 0d0 0d0 0d0 0d0 0d0 |

0d0 0d0 -1d0 0d0 0d0 0d0 0d0 0d0

0d0 0d0 0d0 -6.981260298D-03 -9.999756307D-01 0d0 0d0

0d0 0d0 0d0 0d0 0d0 -1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0

#### AOmirror.test

5.5000000e+01	4e-05

7.6000000e+01 -4e-05

7.7000000e+01 -4e-05

9.5000000e+01 -4e-05

9.6000000e+01 -4e-05

| 1.0200000e+02 | 3e-05 |
| --- | --- |
| 1.0300000e+02 | 3e-05 |
| 1.0400000e+02 | 3e-05 |
| 1.0500000e+02 | 3e-05 |
| 1.0600000e+02 | 3e-05 |
| 1.0700000e+02 | 3e-05 |
| 1.2300000e+02 | 4e-05 |
| 1.2400000e+02 | 4e-05 |
| 1.2500000e+02 | 4e-05 |
| 1.2600000e+02 | 4e-05 |
| 1.2700000e+02 | 4e-05 |
| 1.2800000e+02 | 4e-05 |
| 2.0700000e+02 | 5e-06 |
| 2.2800000e+02 | 5e-06 |
| 2.0800000e+02 | 1e-05 |
| 2.2900000e+02 | 1e-05 |
| 2.4900000e+02 | 1e-05 |
| 2.6800000e+02 | 1e-05 |
| 2.0900000e+02 | 1.5e-05 |
| 2.3000000e+02 | 1.5e-05 |
| 2.5000000e+02 | 1.5e-05 |
| 2.6900000e+02 | 1.5e-05 |
| 2.8700000e+02 | 1.5e-05 |
| 2.1000000e+02 | 2e-05 |
| 2.3100000e+02 | 2e-05 |
| 2.5100000e+02 | 2e-05 |
| 2.7000000e+02 | 2e-05 |
| 2.8800000e+02 | 2e-05 |
| 3.0400000e+02 | 2e-05 |
| 2.1100000e+02 | 2.5e-05 |
| 2.3200000e+02 | 2.5e-05 |
| 2.5200000e+02 | 2.5e-05 |
| 2.7100000e+02 | 2.5e-05 |
| 2.8900000e+02 | 2.5e-05 |
| 3.0500000e+02 | 2.5e-05 |
| 3.1900000e+02 | 2.5e-05 |
| 2.1200000e+02 | 3e-05 |
| 2.3300000e+02 | 3e-05 |
| 2.5300000e+02 | 3e-05 |
| 2.7200000e+02 | 3e-05 |
| 2.9000000e+02 | 3e-05 |
| 3.0600000e+02 | 3e-05 |
| 3.2000000e+02 | 3e-05 |
| 3.3200000e+02 | 3e-05 |
| 2.1300000e+02 | 3.5e-05 |
| 2.3400000e+02 | 3.5e-05 |
| 2.5400000e+02 | 3.5e-05 |
| 2.7300000e+02 | 3.5e-05 |
| 2.9100000e+02 | 3.5e-05 |
| 3.0700000e+02 | 3.5e-05 |
| 3.2100000e+02 | 3.5e-05 |
| 3.3300000e+02 | 3.5e-05 |
| 2.1400000e+02 | 4e-05 |
| 2.3500000e+02 | 4e-05 |
| 2.5500000e+02 | 4e-05 |
| 2.7400000e+02 | 4e-05 |
| 2.9200000e+02 | 4e-05 |
| 3.0800000e+02 | 4e-05 |
| 3.2200000e+02 | 4e-05 |

| 3.3400000e+02 | 4e-05 |
| --- | --- |
| 3.4300000e+02 | 4e-05 |
| 2.4200000e+02 | 4e-05 |
| 2.4300000e+02 | 4e-05 |
| 2.4400000e+02 | 4e-05 |
| 2.4500000e+02 | 4e-05 |
| 2.2000000e+02 | 4e-05 |
| 2.2100000e+02 | 4e-05 |
| 2.2200000e+02 | 4e-05 |
| 2.2300000e+02 | 4e-05 |
| 2.2400000e+02 | 4e-05 |
| 1.9900000e+02 | 4e-05 |
| 2.0000000e+02 | 4e-05 |
| 2.0100000e+02 | 4e-05 |
| 2.0200000e+02 | 4e-05 |
| 2.2000000e+02 | 4e-05 |
| 1.7800000e+02 | 4e-05 |
| 1.7900000e+02 | 4e-05 |
| 1.8000000e+02 | 4e-05 |
| 1.8100000e+02 | 4e-05 |
| 1.8200000e+02 | 4e-05 |
| 1.7500000e+02 | 4e-05 |

### Segmented Telescope Example

#### SegDemo.jou

% SegmentDemo.jou -- a MACOS demonstration macro that illustrates the gener-ation

%	of a linear model of a segmented mirror system. The linear model is used to

%	generate an image and OPD numbers. These are compared with the same num-bers

%	computed using the full nonlinear model. They agree very closely.

% Run this macro with macos256

old SegmentDemo;	% Load prescription file

%

draw 0 10 xz;	% Sketch optics layout

%

map 0	% Displays the ray mapping in the entrance pupil; also

% shows the segment mapping

%

intensity 10	% Generate simulated image at the focal plane -- no

% aberrations

%

log 10	% Display simulated unaberrated image with log10 stretch

%

%

perturb 3	% Perturb the 3rd element using the full nonlinear model NO		% Use global coordinates

0,1e-4,0	% Rotational perturbation: rotate 100 urad about y 0,0,0		% No translational perturbation

%

opd 9	% Display wavefront error using linear model

%

log 10	% Display image (log10 stretch) using full nonlinear

% model

%

old SegmentDemo;	% Reload prescription file

%

build 9	% Compute linear optical model

%

lperturb 3	% Perturb the 3rd element using the linear model NO		% Use global coordinates

0,1e-4,0	% Rotational perturbation: rotate 100 urad about y 0,0,0		% No translational perturbation

%

lopd	% Display wavefront error using linear model

%

llog	% Display image (log10 stretch) using linear model

#### SegDemo.in

% SegmentDemo.in -- prescription file for SegmentDemo.jou, a MACOS demon-stration

%	macro.

| ChfRayDir= | 0d0 0d0 -1d0 |  |
| --- | --- | --- |
| ChfRayPos= | 0d0 0d0 1.5d+00 |  |
| zSource= | 1.d+19 |  |
| IndRef= | 1d0 |  |
| Extinc= | 0d0 |  |
| Wavelen= | 1.d-04 |  |
| Flux= | 1d0 |  |
| GridType= | Pie |  |
| Aperture= | 3.65d+00 |  |
| Obscratn= | 3.d-01 |  |
| nGridpts= | 25 |  |
| xGrid= | 1d0 0d0 0d0 |  |
| yGrid= | 0d0 1d0 0d0 |  |
| nElt= | 10 |  |
| nSeg= | 7 |  |
| width= | 1.2d+00 |  |
| gap= | 0d0 |  |
| SegXgrid= | 1d0 0d0 0d0 |  |
| SegCoord= | 0 0 | 0 |
|  | 1 -1 | -2 |
|  | 2 1 | -1 |
|  | 1 2 | 1 |
|  | -1 1 | 2 |
|  | -2 -1 | 1 |
|  | -1 -2 | -1 |

iElt=	1

EltName= S1 Element= Segment Surface= Conic

KrElt= -2.92d+00

KcElt= -1d0

psiElt= 0d0 0d0 1d0 VptElt= 0d0 0d0 0d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0

Extinc=	1d22

nObs=	0

ApType=	None zElt=	1.46d+00

PropType=	Geometric

nECoord=	-6

iElt=	2

EltName= S2 Element= Segment Surface= Conic

KrElt= -2.92d+00

KcElt= -1d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 0d0

RptElt=	6.124997330D-01 -1.060881274D+00 2.569563356D-01

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None zElt=	1.46d+00

PropType=	Geometric nECoord=		6

TElt=	4.610697370D-01 8.660255296D-01 -1.934282287D-01 0d0 0d0 0d0

-7.985966745D-01 4.999997821D-01 3.350277145D-01 0d0 0d0 0d0 3.868566261D-01 -1.885624059D-18 9.221398760D-01 0d0 0d0 0d0

0d0 0d0 0d0 4.610697370D-01 8.660255296D-01 -1.934282287D-01

0d0 0d0 0d0 -7.985966745D-01 4.999997821D-01 3.350277145D-01

0d0 0d0 0d0 3.868566261D-01 -1.885624059D-18 9.221398760D-01

iElt=	3

EltName= S3 Element= Segment Surface= Conic

KrElt= -2.92d+00

KcElt= -1d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 0d0

RptElt=	1.225d+00 -3.699397078D-07 2.569563356D-01

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None zElt=	1.46d+00

PropType=	Geometric nECoord=		6

TElt=	9.221398760D-01 3.019915982D-07 -3.868566261D-01 0d0 0d0 0d0

-2.784784949D-07 1d0 1.168274508D-07 0d0 0d0 0d0 3.868566261D-01 3.238991466D-24 9.221398760D-01 0d0 0d0 0d0

0d0 0d0 0d0 9.221398760D-01 3.019915982D-07 -3.868566261D-01

0d0 0d0 0d0 -2.784784949D-07 1d0 1.168274508D-07

0d0 0d0 0d0 3.868566261D-01 3.238991466D-24 9.221398760D-01

iElt=	4

EltName= S4 Element= Segment Surface= Conic

KrElt= -2.92d+00

KcElt= -1d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 0d0

RptElt=	6.125003738D-01 1.060880904D+00 2.569563356D-01

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None zElt=	1.46d+00

PropType=	Geometric nECoord=		6

| TElt= | 4.610702194D-01 -8.660252276D-01 -1.934284311D-01 | 0d0 | 0d0 | 0d0 |
| --- | --- | --- | --- | --- |
|  | 7.985963960D-01 5.000003051D-01 -3.350275976D-01 | 0d0 | 0d0 | 0d0 |
|  | 3.868566261D-01 2.270467800D-17 9.221398760D-01 | 0d0 | 0d0 | 0d0 |

0d0 0d0 0d0 4.610702194D-01 -8.660252276D-01 -1.934284311D-01

0d0 0d0 0d0 7.985963960D-01 5.000003051D-01 -3.350275976D-01

0d0 0d0 0d0 3.868566261D-01 2.270467800D-17 9.221398760D-01

iElt=	5

EltName= S5 Element= Segment Surface= Conic

KrElt= -2.92d+00

KcElt= -1d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 0d0

RptElt= -6.124995728D-01 1.060881366D+00 2.569563356D-01

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None zElt=	1.46d+00

PropType=	Geometric nECoord=		6

TElt= -4.610696164D-01 -8.660256051D-01 1.934281781D-01 0d0 0d0 0d0 7.985967441D-01 -4.999996513D-01 -3.350277437D-01 0d0 0d0 0d0

3.868566261D-01 -2.095137844D-19 9.221398760D-01 0d0 0d0 0d0

0d0 0d0 0d0 -4.610696164D-01 -8.660256051D-01 1.934281781D-01

0d0 0d0 0d0 7.985967441D-01 -4.999996513D-01 -3.350277437D-01

0d0 0d0 0d0 3.868566261D-01 -2.095137844D-19 9.221398760D-01

iElt=	6

EltName= S6 Element= Segment Surface= Conic

KrElt= -2.92d+00

KcElt= -1d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 0d0

RptElt= -1.225d+00 5.549095617D-07 2.569563356D-01

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None zElt=	1.46d+00

PropType=	Geometric nECoord=		6

TElt= -9.221398760D-01 -4.529873973D-07 3.868566261D-01 0d0 0d0 0d0 4.177177424D-07 -1d0 -1.752411762D-07 0d0 0d0 0d0

3.868566261D-01 -1.071601397D-23 9.221398760D-01 0d0 0d0 0d0

0d0 0d0 0d0 -9.221398760D-01 -4.529873973D-07 3.868566261D-01

0d0 0d0 0d0 4.177177424D-07 -1d0 -1.752411762D-07

0d0 0d0 0d0 3.868566261D-01 -1.071601397D-23 9.221398760D-01

iElt=	7

EltName= S7 Element= Segment Surface= Conic

KrElt= -2.92d+00

KcElt= -1d0

| psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= TElt= | 0d0 0d0 1d0 0d0 0d0 0d0 -6.125005340D-01 -1.060880811D+00 1d0 1d22 0 None 1.46d+00 Geometric 6 -4.610703399D-01 8.660251521D-01 | 2.569563356D-01 1.934284817D-01 0d0 0d0 0d0 |  |
| --- | --- | --- | --- |
|  | -7.985963264D-01 -5.000004359D-01 3.868566261D-01 -1.153428518D-17 | 3.350275684D-01 0d0 0d0 0d0 9.221398760D-01 0d0 0d0 0d0 |  |
| 0d0 0d0 0d0 -4.610703399D-01 8.660251521D-01 1.934284817D-01 |  |  |  |
|  | 0d0 0d0 0d0 -7.985963264D-01 -5.000004359D-01 0d0 0d0 0d0 3.868566261D-01 -1.153428518D-17 |  | 3.350275684D-01 9.221398760D-01 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 8 SecMir Reflector Conic -2.229620353D-01 -1.174529970D+00 0d0 0d0 1d0 0d0 0d0 1.353d+00 0d0 0d0 1.353d+00 1d0 1d22 0 None 1.07d-01 Geometric -6 |  |  |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 9 RefSurf Reference Conic -2.6d+00 0d0 0d0 0d0 -1d0 0d0 0d0 1.291d+00 0d0 0d0 1.291d+00 1d0 1d22 0 None 2.6d+00 FarField -6 |  |  |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= | 10 FclPlane FocalPlane Flat 0d0 -1.d+38 0d0 0d0 -1d0 0d0 0d0 -1.309d+00 0d0 0d0 -1.309d+00 1d0 1d22 0 None |  |  |
| 196 | Modeling and Analysis for Controlled Optical Systems |  |  |

zElt=		1.d+19 PropType=		Geometric nECoord=	-6

nOutCord=  5

Tout= 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0

### Near-Field Propagation Example

#### coroExample.jou

% MACOS demonstration macro

% This system is a simple coronograph. The demo

% generates a bright on-axis (occulted) image, then

% adds a dim, nearby but slightly off-axis image.

% Run this with macos512!

| old coroExample; draw 0 16 XZ | % | sketch system |
| --- | --- | --- |
| stretch sqrt mod ngridpts=128 nObs(6)=0 | % | undo occulting mask |
| q int 1 | % | pupil function |
| int 6 | % | unocculted PSF |
| pix 6 64 ; | % | blowup of PSF |
| mod ngridpts=128 nObs(6)=1 | % | redo occulting mask |
| q pix 6 64 ; | % | blowup of occulted PSF |
| stretch linear int 10 | % | pupil |
| int 11 | % | pupil at Lyot stop |
| stretch sqrt int 16 | % | on-axis PSF at detector |
| win Tout 1.392542d-6 0,0 0,0 | % | set window for composite image |
| compose 16 64 1.392542E-06 | % | build composite image |
| add yes | % | blowup of on-axis PSF |
| stop elt 11 0,0 | % | set system stop at Lyot stop |
| ffp 6 2d-5,2d-5 yes | % | move new image off occulting mask |
| ors 5 yes | % | reset return surface |
| ors 7 yes | % | reset return surface |
| fex 15 yes | % | reset return surface |
| mod flux=0.01 | % | 100x dimmer object |

q stretch log10

int 6	% new dim image at occulting mask stretch sqrt

add yes	% add dim image to composite image

#### coroExample.in

ChfRayDir= 0d0 0d0 1d0 ChfRayPos= 0d0 0d0 -2.d+01 zSource=	1.d+30

IndRef=	1d0

Extinc=	0d0 Wavelen=	1.d-06

Flux=	1d0 GridType=	Circular Aperture=	4d0 Obscratn=	0d0 nGridpts=	512

xGrid= 1d0 0d0 0d0 yGrid= 0d0 1d0 0d0 nElt= 16

iElt=	1

EltName= SMObs_1 Element= Obscuring Surface= Conic

KrElt= -1.d+30 KcElt=	0d0

psiElt=	0d0 0d0 -1d0 VptElt=	0d0 0d0 -5.4d+00 RptElt=	0d0 0d0 -5.4d+00 IndRef=	1d0

Extinc=	1d22

nObs=	3

ObsType=	Circle

ObsVec=	5d-01 0d0 0d0 ObsType=	Rectangle

ObsVec= -5d-02 5d-02 -2d0 2d0 ObsType=	Rectangle

ObsVec= -2d0 2d0 -5d-02 5d-02 xObs=	1d0 0d0 0d0

ApType=		Circular ApVec=		2d0 0d0 0d0 zElt=	1.d+30

PropType=	Geometric nECoord=	-6

iElt=	2

EltName= PM_1 Element= Reflector Surface= Conic

KrElt= -1.08d+01

KcElt= -1d0

psiElt= 0d0 0d0 -1d0 VptElt= 0d0 0d0 0d0 RptElt= 0d0 0d0 0d0 IndRef= 1d0

Extinc=	1d22

nObs=	0

ApType=	None zElt=	5.4d+00

PropType=	Geometric nECoord=	-6

iElt=	3

EltName= SM_1 Element= Reflector

Surface= Conic

KrElt= -3.526787501D+00 KcElt= -2.670556022D+00

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 -4.061145902D+00

RptElt=	0d0 0d0 -5.4d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	1.338854098D+00

PropType=	Geometric nECoord=	-6

iElt=  4

EltName= ret1_1 Element= Return Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 1.5d+00 RptElt=	0d0 0d0 1.5d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		0d0 PropType=		Geometric nECoord=	-6

iElt=  5

EltName= ret2_1 Element= Return Surface= Conic

KrElt= -6.329334641D+00

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 -4.829334641D+00 RptElt=	0d0 0d0 -4.829334641D+00

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	6.329334641D+00

PropType=	NF1 nECoord=	-6

iElt=	6

EltName= CoroMask Element= Obscuring Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 1.5d+00 RptElt=	0d0 0d0 1.5d+00 IndRef=	1d0

Extinc=	1d22

nObs=	1

ObsType=	Circle

ObsVec=	1.5d-05 0d0 0d0 xObs=	1d0 0d0 0d0

ApType=		Circular ApVec=		2d0 0d0 0d0 zElt=	1.d+30

PropType= NF2 nECoord= -6

iElt=  7

EltName= ret2_2 Element= Return Surface= Conic

KrElt= -6.329334641D+00

KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 7.829334641D+00 RptElt=	0d0 0d0 7.829334641D+00

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt= -6.329334641D+00

PropType=	Geometric nECoord=	-6

iElt=  8

EltName= ret1_2 Element= Return Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0 VptElt=	0d0 0d0 1.5d+00 RptElt=	0d0 0d0 1.5d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		0d0 PropType=		Geometric nECoord=	-6

iElt=	9

EltName= SM_2 Element= Reflector Surface= Conic

KrElt= -3.526787501D+00 KcElt= -2.670556022D+00

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 7.061145902D+00

RptElt=	0d0 0d0 8.4d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	1.338854098D+00

PropType=	Geometric nECoord=	-6

iElt=	10

EltName= PM_2 Element= Reflector

Surface= Conic KrElt= -1.08d+01

KcElt= -1d0

psiElt= 0d0 0d0 1d0 VptElt= 0d0 0d0 3d0 RptElt= 0d0 0d0 3d0 IndRef= 1d0

Extinc=	1d22

nObs=	0

ApType= None zElt=  5.4d+00

PropType=	Geometric nECoord=	-6

iElt=	11

EltName= LyotStop Element= Obscuring Surface= Conic

KrElt= -1.d+30 KcElt=	0d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 8.4d+00 RptElt=	0d0 0d0 8.4d+00 IndRef=	1d0

Extinc=	1d22

nObs=	3

ObsType=	Circle

ObsVec=	5.d-01 0d0 0d0 ObsType=	Rectangle

ObsVec= -2.5d-01 2.5d-01 -2d0 2d0

ObsType=	Rectangle

ObsVec= -2d0 2d0 -2.5d-01 2.5d-01

xObs=		1d0 0d0 0d0 ApType=		Circular ApVec=	2d0 0d0 0d0

zElt=		1.d+30 PropType=		Geometric nECoord=	-6

iElt=	12

EltName= PM_3 Element= Reflector Surface= Conic

KrElt= -1.08d+01

KcElt= -1d0

psiElt=	0d0 0d0 -1d0 VptElt=	0d0 0d0 1.38d+01 RptElt=	0d0 0d0 1.38d+01 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType= None zElt=  5.4d+00

PropType=	Geometric nECoord=	-6

iElt=	13

EltName= SM_3 Element= Reflector

Surface= Conic

KrElt= -3.526787501D+00 KcElt= -2.670556022D+00

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 9.738854098D+00

RptElt=	0d0 0d0 8.4d+00 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	1.338854098D+00

PropType=	Geometric nECoord=	-6

iElt=  14

EltName= ret1_3 Element= Return Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 1.53d+01 RptElt=	0d0 0d0 1.53d+01 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		0d0 PropType=		Geometric nECoord=	-6

iElt=  15

EltName= ret2_3 Element= Return Surface= Conic

KrElt= -6.329334641D+00

KcElt=	0d0

psiElt=	0d0 0d0 1d0

VptElt=	0d0 0d0 8.970665359D+00 RptElt=	0d0 0d0 8.970665359D+00

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=	6.329334641D+00

PropType=	FarField nECoord=	-6

iElt=	16

EltName= foc_pln Element= FocalPlane Surface= Flat

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 1d0 VptElt=	0d0 0d0 1.53d+01 RptElt=	0d0 0d0 1.53d+01 IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		0d0 PropType=		Geometric nECoord=	-6

nOutCord=  5

Tout= 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0

### Image Simulation Example

#### ImageDemo.jou

    - ImageDemo.in

ChfRayDir= 0d0 0d0 -1d0 ChfRayPos= 0d0 0d0 6.d+02 zSource=	1d22

IndRef=	1d0

Extinc=	0d0 Wavelen=	7.d-04

Flux=	1.093342085D+00

GridType=	Circular Aperture=	3.8d+02 Obscratn=	0d0 nGridpts=	33

xGrid= -1d0 2.033814221D-09 0d0 yGrid= -2.033814221D-09 -1d0 0d0

nElt=	13

iElt=	1

EltName= SEC_OBS Element= Obscuring Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 4.002557400D+02 RptElt=	0d0 0d0 4.002557400D+02

IndRef=	1d0

Extinc=	1d22

nObs=	2

ObsType=	Circle

ObsVec=	4.67036d+01 0d0 0d0

ObsType=	Rectangle

ObsVec=	0d0 1.75d+02 2.39d+01 2.69d+01 xObs=	9.896513868D-01 1.434926220D-01 0d0

ApType=	Circular

ApVec=	1.75d+02 0d0 0d0 zElt=	1d22

PropType=	Annulus nECoord=	-6

iElt=	2

EltName= SPIDER_2 Element= Obscuring Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 4.002557399D+02 RptElt=	0d0 0d0 4.002557400D+02

IndRef=	1d0

Extinc=	1d22

nObs=	1

ObsType=	Rectangle

ObsVec=	0d0 1.75d+02 2.39d+01 2.69d+01 xObs= -6.190939493D-01 7.853169309D-01 0d0

ApType=	Circular

ApVec=	1.75d+02 0d0 0d0 zElt=	1d22

PropType=	Annulus nECoord=	-6

iElt=	3

EltName= SPIDER_3 Element= Obscuring Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 4.002557398D+02 RptElt=	0d0 0d0 4.002557400D+02

IndRef=	1d0

Extinc=	1d22

nObs=	1

ObsType=	Rectangle

ObsVec=	0d0 1.75d+02 2.39d+01 2.69d+01 xObs= -3.705574375D-01 -9.288095529D-01 0d0

ApType=	Circular

ApVec=	1.75d+02 0d0 0d0 zElt=	1d22

PropType=	Annulus nECoord=	-6

| iElt= EltName= Element= Surface= | 4 PRI_MIR Reflector Zernike |  |
| --- | --- | --- |
| KrElt= | -9.337886403D+02 |  |
| KcElt= | -1.009724001D+00 |  |
| psiElt= | 0d0 0d0 1d0 |  |
| VptElt= | 0d0 0d0 0d0 |  |
| RptElt= | 0d0 0d0 0d0 |  |
| IndRef= | 1d0 |  |
| Extinc= | 1d22 |  |
| ZernCoef= | 0d0 0d0 0d0 -1.415805330D-04 -3.404653098D-05 | 5.412244128D-06 |

1.464334396D-05 -5.046596565D-05 1.587508743D-05 -3.536721061D-

05 -1.951889059D-06 -2.971194787D-05

1.561541879D-05 -8.027617601D-06 8.191919142D-06 0d0 0d0 0d0

0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0

pMon=	0d0 0d0 0d0 xMon=	1d0 0d0 0d0 yMon=	0d0 1d0 0d0 zMon=	0d0 0d0 1d0 lMon=	1.75d+02 nObs=		0

ApType=	None

zElt=	1d22

| PropType= nECoord= | Geometric -6 |  |
| --- | --- | --- |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= VptElt= | 5 SEC_MIR Reflector Conic -1.565935400D+02 -1.932283999D+00 0d0 0d0 1d0 -7.887151838D-04 -6.647047308D-05 | 4.002602400D+02 |
| RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | -7.887151838D-04 -6.647047308D-05 1d0 1d22 0 None 1d22 Geometric -6 | 4.002602400D+02 |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 6 COR_S1 Refractor Conic -4.241546000D+01 0d0 0d0 0d0 1d0 0d0 0d0 -3.988054000D+01 0d0 0d0 -4.260596000D+01 1.61783d+00 0d0 0 None 1d22 Geometric -6 |  |
| iElt= EltName= Element= Surface= KrElt= KcElt= psiElt= VptElt= RptElt= IndRef= Extinc= nObs= ApType= zElt= PropType= nECoord= | 7 COR_S2 Refractor Conic -3.15595d+01 0d0 0d0 0d0 -1d0 0d0 0d0 -4.138168000D+01 0d0 0d0 -4.260596000D+01 1.657863000D+00 0d0 0 None 1d22 Geometric -6 |  |
| iElt= EltName= Element= Surface= KrElt= | 8 COR_S3 Refractor Conic -5.533237600D+02 |  |

KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 -4.533138000D+01 RptElt=	0d0 0d0 -4.260596000D+01

IndRef=	1d0

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	9

EltName= FLT_S1 Element= Refractor Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 -4.763008000D+01

RptElt=	0d0 0d0 -4.9d+01 IndRef=	1.535018000D+00

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	10

EltName= FLT_S2 Element= Refractor Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 -5.163058000D+01

RptElt=	0d0 0d0 -4.9d+01 IndRef=	1d0

Extinc=	0d0

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	11

EltName= IMG_RET Element= Return Surface= Conic

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	0d0 0d0 -5.175758000D+01 RptElt=	0d0 0d0 -5.175758000D+01

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

iElt=	12

EltName= EXIT_RET Element= Return Surface= Conic

KrElt= -6.874662920D+01

KcElt=	0d0

psiElt= 1.095041158D-04 -5.786697499D-06 -9.999999940D-01 VptElt= -7.904014798D-05 -6.661258889D-06 1.698904900D+01 RptElt= -7.904014798D-05 -6.661258889D-06 1.698904900D+01

IndRef= 1d0

Extinc= 1d22

nObs=	0

ApType=	None

zElt=	6.874662920D+01

PropType=	FarField nECoord=	-6

iElt=	13

EltName= IMG_PL Element= FocalPlane Surface= Flat

KrElt= -1d22 KcElt=	0d0

psiElt=	0d0 0d0 -1d0

VptElt=	7.421487214D-01 -4.232318881D+00 -5.175758000D+01 RptElt=	0d0 0d0 -5.175758000D+01

IndRef=	1d0

Extinc=	1d22

nObs=	0

ApType=	None

zElt=		1d22 PropType=		Geometric nECoord=	-6

nOutCord=  5

Tout= 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 0d0 1d0

### S-MACOS Example Files

#### smacosvars.cmn

C*********************************************************************** C	Begin file smacosvars.cmn

C	This file defines call-line variables and common blocks that give C	user programs direct access to MACOS variables. The form of the C	SMACOS call line is:

| C |  | CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG, |
| --- | --- | --- |
| C | & | OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, |
| C | & | PixArray,Cmatrix) |

C	The param.cmn file defines array sizes for SMACOS call-line and

C	internal MACOS variables. The param.cmn file should be copied from C	param128.cmn, param256.cmn, etc. -- whichever size of SMACOS you C	are using:

INCLUDE 'param.cmn'

C	These are the SMACOS call-line variables:

CHARACTER*132 command CHARACTER*32 CARG(9) REAL*8 DARG(9) INTEGER IARG(9) LOGICAL LARG

REAL*4 RARG(9), OutRaySpot(mRay, 4), NomRayDir(2, mRay), & RMSWFE, NomOPDMat(mpts, mpts), PertOPDMat(mpts, mpts), & PixArray(mPix, mPix), Cmatrix(7, mCm, bRay)

C	These are other SMACOS common blocks, that you may use or comment C	out as you wish. The ifEcho flag, if set to .TRUE., provides for C	full printout of SMACOS operations. If set to .FALSE., reduced

C	printout is provided.

LOGICAL ifEcho COMMON /SCIO/ifEcho

INTEGER npts,NoiseSeed(2) REAL*4 CntrSpot(3)

COMMON /UserCommon/npts,NoiseSeed,CntrSpot

C	These includes provide access to internal MACOS variables. They can C	be commented out if you do not plan to use them:

INCLUDE 'elt.cmn' INCLUDE 'src.cmn'

C	These includes provide access to internal MACOS scratch arrays. They C	should be commented out if you do not plan to use them:

LOGICAL L1(md2)

INTEGER DrawEltVec(mDrawElt,mDrawRay) REAL*4 R1(mdttl,mdttl),RV1(md2),RV2(md2) REAL*8 D1(mdttl,mdttl),D2(mdttl,mdttl) REAL*8 DV1(md2),DV2(md2)

COMMON /SCRATCH/ D1,D2

EQUIVALENCE (D1(1,1),RV1(1)),(D1(1,1),DV1(1)),(D2(1,1),DV2(1)), & (D1(1,1),R1(1,1)),(D2(1,1),RV2(1))

c END of smacosvars.cmn C***********************************************************************

#### smacosExample.f

C*********************************************************************** C	Begin file smacosExample.f

IMPLICIT NONE

C SMACOS variable declarations INCLUDE 'smacosvars.cmn'

C These are local variables:

INTEGER REAL*4 REAL*8

C	Load prescription file

command='OLD' CARG(1)='cassExitPupil'

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

C	Set plot type

command='GRAY'

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

C	Set system stop

command='STOP' CARG(1)='ELT' IARG(1)=3 DARG(1)=0d0 DARG(2)=0d0

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

C	Set exit pupil

command='FEX' IARG(1)=5 CARG(1)='YES'

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

C	Plot OPD at exit pupil after resetting ray grid size

nGridPts=128 command='RESET'

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

command='OPD' IARG(1)=5

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

C	Plot detector intensity after resetting ray grid size

nGridPts=96 command='RESET'

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

command='INTENSITY' IARG(1)=6

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

C	Plot detector using a log10 stretch

command='STRETCH' CARG(1)='LOG10'

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

command='INTENSITY' IARG(1)=6

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

C	Quit!

command='QUIT'

CALL SMACOS(command,CARG,DARG,IARG,LARG,RARG,

& OutRaySpot,NomRayDir,RMSWFE,NomOPDMat,PertOPDMat, & PixArray,Cmatrix)

STOP END

C	End file smacosExample.f C***********************************************************************

## APPENDIX C  Complete Command Reference

The following is the complete command list as printed by the HELP command, regenerated from macos_f90/macos_help.inc (single source of truth).  Casing convention: <UPPER prefix><lower tail> -- the uppercase part is the minimum-match abbreviation tested by the dispatcher.  Tags: [Rx] needs a loaded prescription; [BLD] needs a built linear model; [DIFF] needs a propagated wavefront.

MACOS command summary

Casing: UPPER part is the minimum-match abbreviation.

Tags: [Rx]=needs OLD/NEW, [BLD]=needs BUILD,

[DIFF]=needs PROpagate

#### SESSION & FILES

QUit, END   - exit MACOS
   HELP        - this command list
   REset       - reset options & defaults to startup
   STatus      - what is loaded / built / propagated
   SUMmarize   - pretty-print the optical system
   ELTS        - list elements (type, surface)
   EXEcute     - run a .jou journal file
   JOUrnal     - generate a .jou journal file
   MREset      - reset MACOS model size (128..8192)
   ! <cmd>     - run a shell command
   PWD, CD     - working directory shell helpers
   RX, LS, LL  - list .in files / current directory
   VI, EMAcs   - open file in editor

#### PRESCRIPTION I/O

NEW         - start a new optical system
   OLD <file>  - load a .in prescription
   FID <id>    - load by file id (from RX listing)
   SAVe        - write current system to .in
   EXPort      - write select results to file
   SHOw <elt>  - print all data for one element [Rx]
   MODify <elt>- interactively edit one element  [Rx]

#### SOURCE & WAVELENGTH

CHIefray    - set chief-ray position/direction
   WLENS       - set wavelength list
   SWL         - sweep / step wavelength
   MULtispec   - multi-color image stacking
   NFIlt, RFIlt, SFIlt - narrow / regular / spectral
                         filter
   ATMosphere  - turbulence model on/off
   SETC, SAOpt - stop-aperture options

#### RAY TRACING

RAY <i>     - trace ray i; print state at each
                 surface                       [Rx]
   SEGraytrace <i> - trace a ray through segment i s
                 center (uses RptElt)          [Rx]
   PRAy, RRAy, TPR - alternate ray-state printers
   MAP         - print ray-to-segment map
   COOrd <elt> - beam coordinates at element    [Rx]
   ACOor       - anchor coordinates
   EFL         - system effective focal length /
                 f-ratio / plate scale          [Rx]
   BWK         - chief-ray walk vs nominal      [Rx]
   FEXit       - find exit pupil
   FSR         - free spectral range
   CENter      - center beam in system stop
   STOp        - define system stop element
   CENTRoid    - use centroid as ref point for FEX
   FFP <x,y>   - find field point that places chief
                 ray at element coordinate
   PFP <x,y>   - find field point that places chief
                 ray at pixel coordinate
   FDP         - find detector plane orientation
   SPCenter    - spot-diagram centering option

#### BEHAVIOR TOGGLES

LNEg/NOLNeg - allow/disallow negative pathlengths
   POLarized   - polarization tracing on
   NOPolarization - polarization off
   OBS         - spot-diagram obscuration option
   REGrid/NOREGrid - resample wavefront at element
                     during propagation
   ORS         - optimize reference surface
   SRS         - slave one ref surface to another
   NONe        - turn off plotting (no PGPLOT calls)

#### SURFACE DATA

SINt        - set up interpolated-surface data
   UDSinit     - set up user-defined-surface data

#### PERTURBATION

PERturb <elt>   - apply 6-DOF perturbation     [Rx]
   GPERturb <grp>  - perturb an element group     [Rx]
   PREad <file>    - apply perturbations from file

#### LINEAR MODEL

BUild       - build linear model from chief-ray
                 ray partials                     [Rx]
   DMBuild     - build with deformable mirror     [Rx]
   PARtials <i>- print partial derivatives, ray i [BLD]
   LPErturb    - linear-model perturb              [BLD]
   LPRead      - linear perturb from file          [BLD]
   LREset      - zero linear perturbations         [BLD]
   LSPot, LOPd - linear-model spot / OPD           [BLD]
   LPIxilate   - linear-model pixelated image      [BLD]

#### DIFFRACTION

PROpagate   - propagate diffraction wavefront [Rx]
   BEAm        - set beam type (Gauss, plane, ...)
   VECtor, SCAlar - vector vs scalar diffraction
                    (with POLarized)
   COMpose     - start a multi-object/color image
   ADD, DAdd   - add to / display composed image
   CADD        - add current intensity to composed
   NOIse, SEEd - image noise + RNG seed
   STRetch     - LIN / LOG / SQRT display stretch

#### OUTPUTS (RAY-TRACE & DIFFRACTION)

OPD         - wavefront error map             [Rx]
   SPOt        - spot diagram                    [Rx]
   ZABerr      - Zernike OPD aberration          [Rx]
   ZCOef       - Zernike coefficient extraction  [Rx]
   LOS         - line-of-sight output
   METcalc     - metrology beam output
   INTensity   - wavefront intensity            [DIFF]
   PIXel       - pixelated intensity output     [DIFF]
   AMPlitude   - amplitude output               [DIFF]
   PHAse, PH   - phase output                   [DIFF]
   REAl        - real & imaginary wavefront     [DIFF]
   LOG         - log10 intensity                [DIFF]
   MTF, CMTF   - modulation transfer function   [DIFF]
   IMG         - load saved image data

#### PLOT STYLE

GRAy        - gray-scale surface plot
   WIRe        - wireframe plot
   SLIce       - slice surface plot
   COLumn      - column line plot
   CONtour     - contour surface plot
   IMGmode     - toggle image polarity
                 (NEG=astro, POS=conventional)
   CIR, GIR    - color / gray image rendering
   PGP         - PGPLOT panel layout (1,2,3,4,9)

#### FILE OUTPUT

TEXt        - ASCII print output
   BINary      - binary image file
   FITs, WFIts - FITS / wide-FITS image file
   MAT         - MATLAB .mat file
   GETMatlabmatrix - fetch named matrix into MATLAB
                     (smacos only)

#### WINDOW & POST-PROCESSING

PLOcate     - set plot pixel-array location
   NOPLOC      - clear plot location
   PWIn        - plot window definition
   SZCo        - sizing / coordinate options
   GBS         - Gaussian beam-size post-processing
   BLUr, GBLur - blur / Gaussian blur
   GAIn        - apply gain map to intensity
   ODRaw, PGD  - outline drawing / PGPLOT diagnostics
   ROW         - extract / plot row from image

#### SYSTEM OPTIMIZATION

AVAR <elt>  - add a variable element
   MVAR <elt>  - modify a variable element
   DVAR <elt>  - delete a variable element
   VARS        - list all variable elements
   AFOV <fov>  - add a field of view
   DFOV <fov>  - delete a field of view
   FOVS        - list all fields of view
   CALib       - run system optimization
   SFOV        - select field of view

#### MISC / DEBUG

SRAy, SRT, SRTrace - older single-ray variants
                        (use RAY)
   DOPD, DOPL, ZRM     - debug / diagnostic prints
   JWST_v3d, Vis3d     - 3D rendering demo